| main | CN | 1 |  | te | for example, bathymetry, currents, water levels, regulated areas, services and weather. | It is supposed to delete weather,because ECDIS doesn‘t support weather overlay till now. | DONE |
| --- | --- | --- | --- | --- | --- | --- | --- |
| main | CN | 1 |  | ed | IEC publications standards | It is supposed to replace IEC publications by IEC standards to maintain consistency in the context. | ONG: CHECK |
| main | CN | 3.1 |  | te | According to the Outcome by NCSR and S-100WG9 comments by ChinaMSA,S-98 would not include ENP related contents. | It is supposed to delete the abbreviations of ENP. | ONG: I’m not sure this is the case. Discuss. |
| main | CN | 4.1 | 1 | ed | The Electronic Navigational Data Service and System Database concepts include multiple products for use in navigation systems. They were introduced in IMO MSC 530(106) MSC.530(106)/Rev.1 and update the previous concept of SENC defined in IMO MSC.232(82). | It is supposed to replace MSC 530(106) by MSC.530(106) or MSC.530(106)/Rev.1 to maintain consistency in the context. | DONE |
| main | CN | 4.2 | 1 | ed | IMO MSC 530(106)/Rev.1MSC.530(106)/Rev.1 also defines Electronic Chart Display and Information System (ECDIS): | It is supposed to replace MSC 530(106)Rev.1 by MSC.530(106)/Rev.1 to maintain consistency in the context. | DONE |
| main | CN | 4.3 | 3 a) | ed | For example, an INS component used for the tasks of route monitoring and route planning must meet the requirements for an ECDIS, which are described in IMO MSC 530(106)MSC.530(106).Rev1. | It is supposed to replace MSC 530(106) by MSC.530(106).Rev1. | DONE |
| main | CN | 6.1.4 | 1 | ed | The chart content of display base and standard display is listed in MSC.530(106) MSC.530(106)/Rev.1appendix 2. | It is supposed to replace MSC 530(106) by MSC.530(106).Rev1. | DONE |
| main | CN | 7.2.2 |  | ed | dusk conditions available on the system (MSC 530(106)/AMSC.530(106)/Rev.1 | It is supposed to replace MSC 530(106) by MSC.530(106).Rev1. | DONE |
| main | CN | 7.3 | 3 | ed | as described in IMO A.1021(26) and MSC.302(87) . | It is supposed to add IMO A.1021(26) Code on Alerts and Indicators,2009. |  |
| main | CN | 8.2.1 | 1 | ed | The IMO Performance Standards divides | Editorial correction | DONE |
| main | CN | 8.3 | 1 | ed | The IMO Performance Standards | Editorial correction | DONE |
| main | CN | 8.3 | 2,3,4 | ed | MSC.530(106)/Rev.1 4.4,...These updates must should be automatically applied to the system database. MSC.530(106)/Rev.1 4.5, ,...They must should be distinguishable on the display MSC.530(106)/Rev.1 4.7, | According to MSC.530(106)/Rev.1,it is supposed to replace must by should to maintain consistency. | ONG: What do we do with direct quotes? |
| main | CN | 11.4.2 |  | ed | The display of each supported data product shallshould be differentiated. | It is supposed to replace shall by should to maintain consistency. | ONG: Check if quote. |
| main | CN | 11.8 | 3 | ed | The IMO Performance Standards for | Editorial correction | DONE |
| main | CN | 11.9.2 | 1 | ed | IMO Performance Standards | Editorial correction | DONE |
| main | CN | 11.9.6 | 1 | ed | The IMO Performance Standards for | Editorial correction | DONE |
| main | CN | 11.9.7 | 1 | ed | The IMO Performance Standards for | Editorial correction | DONE |
| main | CN | 11.9.9 | 1 | ed | The IMO Performance Standards for | Editorial correction | DONE |
| main | CN | 11.9.10 | 1 | ed | The IMO Performance Standards for | Editorial correction | DONE |
| main | CN | 11.9.1 |  | ed | Case 1: for display scales larger than 1/80,0001:80,000 Case 1: for display scales at 1/80,0001:80,000 or smaller EXAMPLE 1: For display scales larger than 1/80,0001:80,000 (for example, a scale of 1/50,0001:50,000) EXAMPLE 2: For display scales of 1/80,0001:80,000 or smaller (for example, 1/250,0001:25,000) | It is supposed to replace 1/80,000 ,1/50,000,1/25,000 by1:80,000,1:50,000,1:25,000 several times to maintain consistency. | DONE |
| main | CN | 11.9.6 | 2 | ed | The ECDIS must implement support for the Alerts and Indications Catalogues which may be provided within each product’s Portrayal Catalogues. | According to section 7.3 ,it is supposed to replace Alert by Alerts to maintain consistency. | Check. DONE. |
| main | CN | 11.9.7 | 3 | ed | The ECDIS must implement support for the Alerts and Indications Catalogues. | Editorial correction | S-100 refers to it as the “Alert Catalogue” S-100 9-13.4. DONE |
| main | CN | 11.9.9 | 3 | ed | The ECDIS must implement support for the Alerts and Indications Catalogues. | Editorial correction | As above. DONE |
| main | CN | 11.9.10 | 1 | ed | clause 11.3.68 Route Planning states; | According to MSC.530(106)/Rev.1,it is supposed to replace 11.3.6 by 11.3.8. | ONG: Check. |
| main | CN | 15 | 1 | ed | IMO Resolution MSC.530(106)/Rev.1 | Maintain consistency. | DONE |
| main | CN | 18.1 | 1 |  | 270 x 270 mm (IMO MSC 530(106)MSC.530(106)/Rev.1 10.2). | It is supposed to replace MSC 530(106) by MSC.530(106).Rev1. | DONE |
| main | CN | 19.3.1 |  | te |  | It is supposed to add details related to gaps | There is no requirement for ECDIS for ENC gaps. S-52 says “S-57 Use of the object catalogue states: “There must be no gaps in data between adjoining cells of the same Navigational Purpose. Despite the best efforts of the IHO, there are still some areas of the world where gaps in ENC coverage exist. “ so title has been changed. DONE |
| Appendix C | CN | C-1 |  | te | 1.An ENP (Electronic Nautical Publication) ENDS Update Status Report which reports the status of ENPs ENDS(Two Times) 2 A report detailing the status of all datasets within the System Database of the ECDIS for the appropriate type (ENC or ENPENDS). 3.Electronic Nautical Publications (ENPENDS) Update Status Report(Two Times) 4.ENC or ENPENDS(Two Times) 5.Example of ENPENDS update status report | It is supposed to replace ENP by ENDS ten times: | ONG: Check |
| Appendix C | CN | C-1 | Table3 | te | [YYYYMMDDTZHHMMSS] | According to ISO8601 and S-128 Editon 2.0 | ONG: Check |
| Appendix C | CN | C-1 | Completeness and Up-to-datedness | ed | The primary use case for the completeness feature is to assess the installation of S-124 Navigational Wwarnings datasets against the in-force bulletins produced by navigational warning providers. Completeness reports must be able to be restricted to individual products. They may be referenced to either S-128 datasets, or individual S-124 Navigational Warning datasets representing in-force bulletins. | It is supposed to update the phrases to maintain consistency. | DONE |
| Appendix C | CN | C-1 | Filtering of Update status report for route section | ed | The width of the filtering corridor is equal to the “user specified distance” implemented inside the ECDIS to fulfil IMO MSC.5320(106)/Rev.1 11.3.7: |  | DONE. |
| Appendix C | CN | C-1 | Data Server content tables | te | The status Cancelled had not determined in Table 3 | According to “2.Totals of cells with the relevant status in the order defined below” ,it is supposed to add the status Cancelled and its specification in table3. | ONG: Check. |
| Appendix C | CN | C-1 | Example of ENP update status report | ed | Date of Report: 01 Jun 2013 | According to ISO8601 and S-128 Editon 2.0 | ONG: Check. |
| Appendix D | CN | D-1 5 |  | ed | When a user selects a safety contour value, outside areas of S-102 coverage the source for the displayed safety contour is the existing S-101 ENC (or S-57 ENC in dual-fuel mode) as defined in 12.6.111.6.1. | It is supposed to update 12.6.1 to 11.61 to maintain consistency. | DONE. |
| Appendix D | CN | D-1.1 |  | ed | Selection of Base or Standard Display shallshould disable sun-illumination | It is supposed to replace shall by should to maintain consistency. | ONG: Check |
| Appendix D | CN | D-1.2 1 |  | ed | - Portrayal of S-102 data and application of ESC must be restricted by minimum and maximum display scale values contained in the exchange catalogue as specified in Section 12.211.2. | It is supposed to update 12.6.1 to 11.61 to maintain consistency. | DONE |
| Appendix D | CN | D-1.3 |  | ed | The areas of safe and unsafe water formed by the selected edges are used for processing of the safety contour related alarms under IMO MSC 530(106)MSC.530(106).Rev1. 11.4.3, which the OEM must implement. | It is supposed to replace MSC 530(106) by MSC.530(106).Rev1. | DONE |
| Appendix D | CN | D-2.3 a |  | ed | A distance parameter, the limit of the check area as specified by IMO MSC 530(106))MSC.530(106).Rev1 11.3.7 when route planning and MSC.530(106)/Rev.1 11.4.4 when route monitoring. | It is supposed to replace MSC 530(106) by MSC.530(106).Rev1. | DONE |
| Appendix D | CN | D-2.3 b |  | ed | A time resolution tuo used to construct the individual WLA sections. | Editorial correction | ONG: Check |
| Appendix D | CN | D-2.7 |  | ed | When WLA is based on a route then the limit of check area around the route is set by the user as specified by IMO MSC 530(106))MSC.530(106).Rev111.3.7. | It is supposed to replace MSC 530(106) by MSC.530(106).Rev1. | DONE |
| Appendix D | CN | D-2.7 1 |  | ed | For each section of the route a series of estimated time markers is defined along the route at times t0 – tn. The diagram also shows the “limit of check area” as specified by IMO MSC 530(106))MSC.530(106).Rev111.3.7 | It is supposed to replace MSC 530(106) by MSC.530(106).Rev1. | DONE |
| main | ntou | 2.1 | IEC60945 | ed | Use consistent style for edition number | Change Fourth Edition to Edition 4.0 | ONG: Ok |
| main | ntou | 2.1 | IEC62288 | ed | IEC62288 Ed.3.0 has Amendment 1 published in 2024 | The valid edition is Edition 3.1 (IEC 62288:2021+AMD1:2024) | ONG: Check (Ok). |
| main | ntou | 2.1 | S-97 | ed | Missing ‘Edition’ before 1.1.0 | Add ‘Edition’ before ‘1.1.0’ | Ok |
| main | ntou | 2.1 | the last paragraph | ed | Some of the listed normative references are with edition number and edition date, some without mentioning any. Is the statement “All references refer to the latest revision or amendment” indeed applicable to all the normative references listed in 2.1? | Clarify the differences and the statement. | ONG: Check |
| main | ntou | 4.2 | b) | ed | SOLAS V/17 is about “Electromagnetic Compatibility” For “Nautical charts and nautical publications”, it is SOLAS V/27. | Correct V/17 to V/27 | DONE |
| main | ntou | 4.2 |  | ed | ship’s ‘head’? | Correct to ship’s heading | DONE |
| main | ntou | 6.1.1 |  | ed | Section numbers referred to in the text should be corrected | From Section 13.1 to Section 12.1 From Section 12.12 to Section 11.12 From Section 12.4 to Section 11.4 From 12.2 to Section 11.2 | DONE. |
| main | ntou | 11.8 | 1st paragraph 2nd sentene | te | Whether to apply scale minimum to the display should be a user-defined setting. This is one of the New Mandatory ECDIS Requirements introduced to S-52 Presentation Library and IEC 61174 at Ed.4.0: (IHO S-52/Annex A, Part I/10.3.4.4) Description: “Ability to turn SCAMIN on/off”; Justification: SCAMIN is not universally applied by HO’s in the same way. Mariners need to be able to view all data and need a function to turn the SCAMIN attribute off. | Revise 11.8 to reflect this ECDIS requirement that align with MSC.530(106)/Rev.1 clause 6.1.3. | DONE. Converted to must for requirement. |
| main | ntou | 11.11.5 | 4th paragraph | ed | The requirement that ‘Update Review’ IMS must be implemented needs clarification. Does IMS stand for ‘Information Management System’? | IMS appears only once in the document. Use the full words instead of abbreviation. | No, independent mariner selections. Ok,. DONE |
| main | ntou | 11.12.1 |  | ed | The section number referred to in multiple places should be 11.12.2, not 12.12.2 | Correct 12.12.2 to 11.12.2 | DONE. |
| main | ntou | 11.12.4 11.12.1 |  | te | - The ‘publishedDate’ referred to in 11.12.4 is not found in S-124. There is a mandatory ‘publicationTime’ attribute in S-124 FC/datasets. - In clauses 11.12.1, the sorting and listing of NAVWARN messages all refer to issue date (and time). - For S-124 datasets, ‘issueDate’ is a mandatory attribute of S100_DatasetDiscoveryMetadata in the CATALOG.XML of Exchange Set, while ‘issueTime’ is optional. It would be better to use the ‘publicationTime’ attribute in the S-124 dataset and use the same attribute for 11.12.1 and 11.12.4. | - In 11.12.4, replace the ‘publishedDate’ with ‘publicationTime’. - Replace all the ‘issue date (and time)’ referred to in 11.12.1 with the ‘publicationTime’ attribute of the S-124 dataset. | Check with 124 vs using CATALOG.XML values. Using dataset, check with 124 PT. Need attribute(s) for type. DONE. |
| main | ntou | 14.1 | (6) | te | 14.1(6) specifies that Dates must be given in the form “Day Month Year” DD-MMM-YYYY. But “Day Month Year” and DD-MMM-YYYY are different forms. The form specified is different from those encoded in the (ISO8211, GML, HDF5) datasets. | Clarify the form to be used: which form is compliant to the cursor pick rule (e.g., 01 JUL 2026, 01-JUL-2026, 1 JUL 2026?) | ONG: Check S52 wording on this. Should be consistent. |
| main | ntou | 14.1 |  | te | From 14.1(6) for Dates, it seems the cursor pick rule is using a form different from those encoded in the (ISO8211, GML, HDF5) datasets. What about the forms in Pick Reports for Time and DateTime, where a Local Time with an offset to UTC needs to be considered as well? | Add rules to specify or clarify the formats to be used in Pick Reports for Time and DateTime, including for cases where they are encoded in Local Time with or without a given offset to UTC. | ONG: Yes, needs looking at. |
| main | ntou | 14.1 | language pack | te | A feature catalogue language pack may contain translations for many fields, including name, definition and remarks of attributes, associations, information types, feature types, and listed value name (label). Among the cursor pick rules in 14.1, only 14.1(1) specifies that, if a language pack is installed, the feature and attribute names must use the translated names contained in the language pack. Language pack is not mentioned in 14.1(2) and 14.1(11). What about the rule of pick reports for other translated fields contained in the language pack, if installed? | Clarify whether the translated ones should/must be used, by revising existing rules or adding one for the case if a language pack is installed. | ONG: Clarify. |
| main | ntou | 14.2 | file locator | te | Requirement to make use of the fileLocator when displaying the textual information in dataset support files should be specified. | In clause 14.2, add the requirement to make use of the fileLocator when displaying the textual information in dataset support files. | Not agreed by mtg. |
| main | ntou | 20.3 | 1st paragraph | te | HTML support file format is used in several product specifications listed in 6.1.1 (datasets ECDIS must be able to process and display), e.g., S-128 and S-131. | Add HTML to the list of dataset support file formats (in clause 20.3) that ECDIS must be able to display. | Not agreed by mtg. |
| main | ntou | 14 | signal sequence | te | signalSequence complex attribute, consisting of a pair of signalStatus(1:lit/sound, 2:eclipsed/silent) and signalDuration, is used in many S-101 (and S-125) features, such as LightAllAround, LightSectored, FogSignal, LightFogDetector, LightAirObstruction, RadarTransponderBeacon, with multiplicity [ 0,* (ordered)]. In S-57, SIGSEQ is encoded in a formatted string, e.g. (01.00)+02.00+(01.00)+02.00+(01.00)+08.00 In S-101, the above example will be encoded as six(6) signalSequence complex attributes. For such ordered multiple instances of complex attribute, it would be much more easily readable to the users if the ECDIS pick report could combine them, e.g., display the signal sequence as a single string like the SIGSEQ of S-57. re: https://github.com/iho-ohi/S-101-Documentation-and-FC/issues/230 | Add a requirement to clause 14 that ECDIS pick report must/should combine the ordered multiple instances of signal sequence complex attribute and display it in a standardize format (like SIGSEQ string in S-57). | ONG: Needs proposal words. Enhancement. MTG2/TSM |
| main | ntou | 14 | timeOfDayEnd and timeOfDayStart | te | timeOfDayEnd and timeOfDayStart are two of the sub-attributes of timeIntervalsByDayOfWeek, each with multiplicity [0,* (ordered)]. The number of instances of timeOfDayStart must be the same as that of timeOfDayEnd. Where there are multiple instances of them, the data encoded will be ordered multiple instances of timeOfDayEnd followed by ordered multiple instances of timeOfDayStart. The pick report should group the corresponding instances into pairs (of timeOfDayStart and timeOfDayEnd) for display. p.s. Based on feedback from S-131 project | Add a requirement to clause 14 that ECDIS pick report must/should group the corresponding instances into pairs (of timeOfDayStart and timeOfDayEnd) for display. | ONG: Enhancement. Consider. MTG2/TSM |
| Main | IT | 5.1 | 1 | ge | IMO Resolution MSC.191(79) cannot reference IMO MSC.1/Circ.1609 because MSC.191 was adopted in 2004, while MSC.1/Circ.1609 was issued much later, in 2019. | IMO Resolution MSC.191(79), as amended by MSC.466(101), provides mandatory requirements for presentation of navigation-related information on shipborne navigational displays that are applicable to ECDIS and INS. including references to IMO MSC.1/Circ.1609 | The amendment MSC.466(101) is the one which references 1609. This is ok.. DONE |
| Main | IT | 6.1.1 | 1 | ed | A semicolons and a full stop are missing to make the bullet list more consistent. | - S-101 ENC data as the base layer, alongside S-57 ENC data during the transition period; - S-102 Bathymetric Surface, only for Enhanced Safety Contour and Water Level Adjustment (Appendix D); - S-104 Water Level information for Surface Navigation, only for Water Level Adjustment (Appendix D); - S-111 Surface Currents, additional requirements apply (Section 13.1 and Appendix G); - S-122 Marine Protected Areas; - S-123 Marine Radio Services; - S-127 Marine Traffic Management; - S-131 Marine Harbour Infrastructure; - S-124 Navigational Warnings, additional requirements also apply (Section 12.12); - S-128 Catalogue of Nautical Products, only within Update Status Reports (Appendix C) and graphical indexes (Section 12.4); - S-129 Under Keel Clearance Management. | DONE |
| Main | IT | 6.1.4 | 1 | ed | Standard display is the display mode intended to be used as a minimum during route planning and route monitoring | Standard display is the display mode intended to be used as a minimum requirement during route planning and route monitoring | ONG: I’m not sure this is the right wording. Needs to be checked. |
| Main | IT | 11.8 | 3 | ed | The full stop is missing. | The IMO Performance Standard for ECDIS MSC.530(106)/Rev.1, clause 6.1.3 states that ECDIS should provide an indication if information at own ship's position is not displayed because of applying scale minimum for display. | DONE. |
| Main | IT | 11.9.10 | 2 | ed | This wording is recommended for greater clarity. | The selector used to take accuracy information into account must be called “Uncertainty”. | DONE. |
| Main | IT | 11.10.1 | 1 | ED | Some semicolons are missing to make the bullet list more consistent. | - Units for Depth; - Units for Height; - Scale of display; - Data quality indicator; - Sounding/vertical datum; - Horizontal datum; - The value of the safety depth selected by the mariner,; - The value of the safety contour selected by the mariner,; - The value of the safety contour displayed (which may be different from that selected by the mariner); - Method of Water Level Adjustment (if enabled) - Magnetic variation; - Date and number of last update(s) affecting the datasets currently in use; - Edition number and date of issue of the datasets currently in use; - Display projection. | DONE |
| Main | IT | 1 | 11.11.1 | ed | The full stop is missing. | In addition, it must provide updating information (identification and source of update (i.e. manual update), when entered etc) on cursor picking. | DONE |
| Main | IT | 1 | 11.12.1 | ed | Two colons and semicolons are missing | A dedicated interface must be provided to allow users to interact with NAVWARN messages. This interface must, provide the following minimum functionality;: The user must be able to classify individual messages in accordance with section 12.12.2; An on-demand listing of all (S-124) NAVWARN messages in the system must be provided allowing sorting according to: installed date and time, issue date and time, warning type, producer, series and message classification (as defined in 12.12.2); The on-demand listing of all NAVWARN messages in the system must include means for viewing an abbreviated view of any NAVWARNPart, warningInformation attributes present. This should consist of a minimum of the issue date, warning type and the first 64 characters of the warningInformation attributes.; An indication must be provided when a new NAVWARN message is loaded until it has been displayed or 24 hours have passed. This indication may be suppressed if the NAVWARN message does not meet filtering criteria set by the mariner or automatically applied (as defined in 12.12.2).; Means must be provided to view the most recent message and past messages. | DONE |
| Main | IT | 11.12.2. | 3 | ed | The full stop is missing. | Even though the full extent of a navigational warning may not be portrayed, it must still be available and discoverable in a list of NAVWARNs that can be recalled by user action at any time. | DONE |
| Main | IT | 12.2 | 4 | ed | The full stop is missing. | If the selected display time is outside the interval [(startDateTime – discriminationInterval), (endDateTime+discriminationInterval)] then the data must not be displayed. | DONE |
| Main | IT | 14.3.1 | 2 | ed | The full stop is missing. | In lieu of this method, the OEM may employ an alternate consistent and logical sorting method for the display of pick report results. | DONE |
| Main | IT | 14.4 | 3 | ed | The full stop is missing. | Multiple instances of Tidal Stream Panel Data features must be shown as separate tables. | DONE |
| Main | IT | 18.4.1 | 6 | ed | Comma missing | Consequently, these tolerances should not be applied to the night table; and for type-approval purposes they are restricted to the bright sun table. | DONE |
| Main | IT | 18.5 |  | ed | The full stop is missing. | The ECDIS Chart 1 and its indexing list of symbol names and meanings arranged numerically, together with the colour differentiation test diagrams, are intended for the Mariner’s use. The use of the Colour Differentiation Test Diagrams is described in 19.6.2. | DONE |
| Main | IT | Appendix D | 1 | ed | There is no 12.6.1 in the document | In these processes an end user selects: - Suppression of specific S-101 features by S-102.; - A Safety Contour value of their choice. When a user selects a safety contour value, outside areas of S-102 coverage the source for the displayed safety contour is the existing S-101 ENC (or S-57 ENC in dual-fuel mode) as defined in 12.6.1. | DONE, 11.6.1 |
| Main | IT | Appendix D | 1.1 | ed | Semicolons are missing | Sun illumination may be provided. Where provided: - The mariner must be able to select whether it is enabled or not.; - Selection of Base or Standard Display shall disable sun-illumination; - The ECDIS must use a consistent sun azimuth and elevation values for all data portrayed; - The mariner may be able to vary the sun azimuth and elevation values. | DONE |
| Main | IT | Appendix D | 1.2 | ed | Two colons and semicolons missing. | - Overlaps are strictly defined as those between grid cells containing data, as opposed to grid cells with no-data (fill) values. Datasets may contain overlapping grid cells, and different datasets may overlap, most commonly at national borders, but also in areas where there are multiple vertical datums within a single dataset. In these cases: - Overlaps within the same dataset (in areas with multiple vertical datums) the overlap must be resolved by taking the shoalest value for each grid cell (including vertical uncertainty when the Uncertainty selector is enabled) within the intersection. This is also used to resolve situations when the Mariner Selected Viewing Scale (MSVS) means that multiple grid cells are to be portrayed at the same screen position (the common point rule) – most often when the grid is being portrayed at a smaller zoom scale than the dataset resolution.; - Overlaps between different producers. The end user must be able to select which dataset (or data producer) they wish to use. | DONE |
| Main | IT | Appendix D | 2.7 | ed | Full stop missing | The same user-specified distance must be used for the check of safety contour, prohibited areas, geographic areas for which special conditions exist and navigational hazards and equivalent requirements when route monitoring. … When WLA is based on the planned schedule and own ship is not keeping to schedule a Caution must be raised to indicate the water level being experienced may be different to that being applied by the ECDIS. | DONE |
|  | DS | all |  | ge | Check all internal links for correctness and convert in real references in word |  |  |
|  |  | 3.1 |  | ed | Add abbreviations commonly used in this document | MSVS | DONE |
|  | DS | 6.1.1 |  | ed | The data product should be listed in numerical order for better readability | Order the data product in numerical order in the bullet list and both occurrences in the text block below the list | DONE |
|  | DS | 6.1.1 |  | ge | The wording suggest that the listed data products are not only applicable to ECDIS. | The ECDIS main screen must be able to process and display at least the following data products (other navigation displays such as chart radar may only use a sub-set of listed data products) | ONG: Not sure I understand why this is required? ADJ2 |
|  | DS | 6.1.1 | last | ed | Probably wrong link to 12.2. Most likely the link should go to 11.2 | Correct link to 11.2 (use real link to sub-clause) | DONE |
|  | DS | 6.1.4 | last | ed | Make the bullet item more generic | All other S-100 data products listed in 6.1.1 must be suppressed. | ONG: Why? |
|  | DS | 8.1.2 |  | ed | Wrong link to legend | (see 12.10.1) | DONE |
|  | DS | 8.1.3 & 8.1.4 |  | ge | Chapters are obsolete The only requirement in 8.1.4 is also contained in 11.10.1 | Remove chapters | ONG: Agree – ADJ2 |
|  | DS | 11.7.2 14.1 D-2.X D-3.2 |  | Te | This refers to all occurrences of formatted date information In addition to the format dd mmm yyyy Other formats shall be allowed. The format yyyy-mm-dd (based on ISO 8601) is widely used in different applications and it is best suitable for sorted tables by date (and time) like event logs or route plans See also S-100 5.2.0 Table 1-2 data types | Add a note in each occurrence of formatted date information Other date formats such as yyyy-mm-dd (based on ISO 8601) are permitted as long as the format is consistently used throughout the system, and it is either self-explaining or described in the operator manual | ONG: This required some discussion on the merits of 8601 in addition to those specified. |
|  | DS | 11.9.6 |  | Ed | Missing space between value and unit “30m” | “default value must be 30 m” | DONE – changed to metres. |
|  | DS | 11.9.10.1 |  | ed | Inconsistent wording and missing interpunctuation ‘either’ without ‘or’ Missing full stop after sub-attribute | Reword this sub-clause consistently as foillows: This provides either a horizontalPositionUncertainty sub-attribute. If no horizontalPositionUncertainty value is available then the amount of extension must be taken from the sub-attribute categoryOfZoneOfConfidence according to the following table of default values: | DONE |
|  | DS | 11.9.10.3 |  | ed | The example (starting from 2nd paragraph is not related to this sub-clause (S-102 & S-104) and hence should be in a separate sub-clause | Create new sub-clause under S-101 for the example “Example for S-101 only uncertainty” | DONE |
|  | DS | 11.12.1 | Heading line | ed | The word ‘portrayal’ is misleading in this case Find better wording for the heading of 11.12.1 | Proposed new heading line Additional requirements for the Graphical User Interface | DONE – changed to Requirements of the Graphical User Interface. |
|  | DS | 11.12.4 |  | te | This sub-clause does not contain an explicit requirement. Retention is only implicitly mentioned in 11.12.3 and 11.12.4 without stating any explicit requirements like they exist for e.g. Navtex. Align the requirements regarding automatic deletion from 11.12.3 with 11.12.4 | ECDIS must allow the user to retain at least xx NAVWARNs from automatic deletion. The user must be able to remove the retention status of NAVWARNs. If a NAVWARN is not retained by the user, it may be automatically deleted - 3 months after the cancellation or - After the defined end date or - One year after the publication date whatever happens first. | ONG: I believe retention is required elsewhere in the IEC standards… |
|  | DS | 14.1 | (3) | te | Allow padding of values in the pick report limited to two decimal digits for reasonable alignment of values e.g. in tables | Add at the end of (3) To aligning values e.g. in a table, values may be padded by up to two decimal digits for better readability | ONG: Discuss. |
|  | DS | 17.1 | 4th bullet | te | The combination of viewing groups with S-57 only applies to S-101, hence S-101 should be used here explicitly | Management of viewing groups must be combined between S-57 and S-101. Viewing groups common to S-57 and S-101 should be managed as a single group across all S-100 products. | DONE |
|  | DS | 18.6.1 | 4th paragraph | te | The Colour Differentiation diagram may also be used for the night palette See also 18.6.2 above the diagram and change accordingly | using the day, dusk or night colour tables | DONE |
|  | DS | 18.6.2 |  | ge | Do we need the comprehensive description of the color differentiation diagram in S-98 | Consider shortening the sub-clause Remove the diagram Figure 4 and the text below to the end of the sub-clause | REVIEW. |
|  | DS | 19.1 |  | te | Missing requirement for the number of supported versions of FC and PC | Add after the first paragraph The ECDIS must at least support X different version of Feature and Portrayal Catalogues for each supported S-100 data product. ECDIS may only support FC and PC based on the same edition of S-100. | ONG: Not decided yet. Is this a maximum or minimum requirement? |
|  | DS | 19.4.3 | a) | Ge | Misleading wording “ECDIS should be capable of being interfaced to a SECOM (IEC 63173-2) based telecommunication network. This capability should include at least” Optional ‘should’ requirement together with ‘at least’ could be read as mandatory. The sub-bullets i. to iii. refer only to the optional SECOM part | Clarify wording that reception of updated for S-124 and S-129 via SECOM is optional by putting the whole SECOM part in the note or remove that part completely. a) Reception of Updates. The ECDIS must be capable of receiving official updates, as S-100 Part 17 exchange set. b) … Move the note to the end of 19.4.3 or into a new sub-clause 19.4.3.1 For such updates of S-124 and S-129 datasets, the ECDIS may be capable of being interfaced to a SECOM (IEC 63173-2) based telecommunication network. This capability should include at least : i.User selection of data services from a list provided by a SECOM service registry, ii.Management of information about the data services selected in item (i), for example SECOM subscription status, and iii.Acknowledge receipt of datasets. The bi-directional transfer of S-421 based route plans using SECOM is described in IEC 61174, IEC 63173-1 and IEC 63173-2. | ONG: check – all should be optional. |
|  | DS | 19.4.3 | g) | te | Update status report is only applicable to ENCs and ENPs but not to e.g. S-129 and S-421 as S-129 are specifically create for a single ship on a single passage | … demonstrate the up to date status of all ENC and ENP datasets held .. | ONG: Agree. |
|  | DS | D-1 |  | ge | No need to duplicate the values for DEPWL in the footnote See also similar comment on D-2.4 | Remove footnote | Not sure. Are these the values for that colour token? |
|  | DS | D-1.2 | 2. | te | Use same wording as for S-104 (D-2.2) to define the limit for an overlap | … but in case of overlap of greater than one grid cell the ECDIS must … Add the same footnote as in D-2.2 | ONG: Agree |
|  | DS | D-1.3 | Fig D-1.4 | ge | Figure D-1.4 does not provide any additional information compared to the left side of figure D-1.5. Besides that, the grid cells in Figure D-1.4 are not quare shaped | Remove Figure D-1.4 and change references accordingly If it is decided to keep the figure, then correct the cells to be squares | ONG: Needs re-drafting |
|  | DS | D-1.3 | Fig D-1.4 | ge | The grid cells on the left side of Figure D-1.5 are not quare shaped It would be useful if the left and the right side of the figures would show the same data | Correct the figure to show square shaped cells | ONG: These need redrafting but diagrams can be redrawn too. |
|  | DS | D-2 |  | ed | Ambiguous wording Make clear that the settings for WLA may be changed at any time but only be applied in areas where S-101, S-102 and A-104 data is available. | Better The user must be able to select depth information adjustment by water level height. This adjustment must then be applied in areas where S-101, S-102 and S-104 data products are available. | ONG: I think this is ok. Needs to be agreed. MTG2 |
|  | DS | D2 | 3. a. | ed | Do we need to make clear that only 2-shades and not 4-shades are mandated | Add note if deemed necessary | DONE by portrayal, therefore now mandatory. But implemented by PC. No. Doesn’t need to be made clear. |
|  | DS | D-2.4 |  | ge | Clarify what to display if the boundary lines for WLA areas and ESC areas are co-located Proposal: WLA area lines have precedence over ESC area lines | Add In cases that the WLA area lines and ESC lines are co-located, only the WLA Area lines must be displayed | ONG: Agree – useful clarification. MTG2 |
|  | DS | D-2.4 |  | ge | No need to duplicate the values for DEPWL in the footnote See also similar comment on D-1 | Remove footnote | ONG: MTG2? |
|  | DS | D-2.7 | Fig D-2.13 |  | For clarity show only one WLA polygon (in red) neighbouring WLA polygons may be displayed with other line styles (e.g. dotted) for completeness | Change figure D-2.13 | ONG: Check |
|  | DS | D-2.7 |  | ge | No need to duplicate the values for DEPWL in the footnote See also similar comment on D-1 and D-2.4 | Remove footnote | ONG: MTG2. |
|  | DS | D-2.7 |  | ge | What is the date/time range to be displayed in the indication? Assumeing a route from Busan to Hamburg which takes 4 weeks. Shall the indication only reflect the section of the route falling within the display area? |  | ONG: Not sure. |
|  | DS | D-2.7 |  | ge | The result of the WLA Option 3 will be a broken Safety Contour. The WLA adjusted Safety Contour is only applied inside the check area as defined by the user. Outside the check area where S-102 and even S-104 data may be available the safety contour will only be adjusted to the S-102 data | Add a note clarifying this behaviour and mandating to point that out in the operator manual. Add something like this The WLA adjusted Safety Contour is only applied inside the check area as defined by the user (route limits). Outside the check area where S-102 and even S-104 data may be available the safety contour will only be adjusted to the S-102 data. In areas where only S-101 or S-57 data is available the Safety Contour will be based on the contour lines available in the ENC. The Safety Contour displayed outside the check area will not be adjusted to the planned schedule. The amount of safe water outside the checked area may be smaller than indicated. Deviation from the planned route in position or time may result in violation of the safety contour and consequently in danger of grounding. | ONG: I like the wording. Needs discussion though. |
|  | DS | D-2.7 |  | te | To overcome the problem described above, allow to use an extended WLA Option 3 where the check are is extended in order to display the adjusted safety contour outside the actual check area up to a certain limit (TBD) |  | ONG: Discuss. |
|  | DS | E-1.1 etc. |  | ed | Wrong formatting of sub-headings in Annex E which leads to inconsistent index | Fix format for sub heading in Annex E | ONG: Ok. |
|  | DS | E-1.2 |  | ed | Wrong reference to table 1. Must be table 11 | Correct reference to table 11 | ONG: Ok. |
| main | UK | 6.1.3 |  |  | propose remove IHO in the text as WMO standards are listed. |  | DONE |
| main | UK | 6.1.4 |  |  | - given the text here what is the point of Base Display? |  | ONG: Discuss |
| main | UK | 11.4.1 |  |  | seems odd to me that no graphical index is specified for other products. |  | ONG: Not a requirement |
| main | UK | 11.5.3 |  |  | seems to be incomplete, text needs review. |  | AGREE – needs words. Have deleted sentence for now. |
| main | UK | 11.11.1 |  |  | Propose that this text should refer to the informative FC mentioned in the Appendix at A-2. |  | It’s mentioned in c) ? Is it informative – TBD. |
| main | UK | C-1 |  |  | test needs review S-102 and S-104 referred to with question marks. |  | ONG: |
| main | UK |  |  |  | Question - a Feature Catalogue is included for mariners updates will a Portrayal Catalogue be provided also? |  | Yes, but implementation of symbols is up to OEM. |
|  | UK | Annex A 12 |  |  | propose that the term scale boundaries is ambiguous and this language should be improved assume that this means Data Coverage features. Note language in main document regarding scale steps |  | ONG:Agree – can be improved. |
|  | UK | Annex A 12 |  |  | Noting the text – “NotForNavigation may be included in CATALOG.XML but is ignored by S100 ECDIS” - consider that there should be a means to indicate this noting testing and trials would a dedicated producer code be an option to mention? | NotForNavigation may be included in CATALOG.XML but is ignored by S100 ECDIS. If it is required to indicate data for testing purposes then a specific Producer code should be used. Dataset Titles may also be used to indicate this for products which support this. | ONG:Agree. |
| Main Doc | DE | 2.1 | S-100 | ge | In Edition 2.5.5, S-98 references S-100 Edition 5.2.1. What impact does this have on product specifications that still reference S-100 ed. 5.2.0 at the time of publication of S-98 ed. 2.5.5? Is S-98 ed. 2.5.5 then applicable to these product specifications, or is a previous edition of S-98 valid? | - | Agreed to change to 5.2 DONE |
| Main Doc | DE | 3.2 | Feature, Information type, attribute | ge | The notation of these references should be consistent within the standard. What should the reader search for if the references are sometimes written with spaces and sometimes without? In one case, I search for ‘LandArea’ and don't find all instances because others are written as ‘Land Area’. In another case, I first search for “LandArea” and then for ‘Depth Area’, which is confusing. Not every reader is familiar with regular expressions to find a clear result. | The notations of the references should be used with the camel case name. This is the only unambiguous spelling. The use of spaces can lead to confusion with natural language usage. | ONG: Agree – will define in para difference and font between code and name (from FC). |
| Main Doc | DE | 4.2 | List item a) | ed | The explanations for “additional layers” are a bit confusing. “Additional information is information that is not contained in the ENC, while enhanced information are layers that contain improved, detailed or higher resolution information than the ENC.” The enhanced information is also not contained in ENC. According to the section of S-100 “16a-4.2.2 Other systems” there is obviously no clear difference in content between additional or enhanced information. | To transfer from S-100?? “Generally, ENC charts form the base layer in a VTMIS and other data sources form layers of additional information or enhanced information to aid the VTMIS operator in monitoring and decision making. Examples of additional layers included radar and AIS for target tracking, …” |  |
| Main Doc | DE | 4.2 | The diversity of information – second point |  | The first point refers to the physical chart information. The second point could accordingly refer to the “thematic chart information”. | “thematic chart information, for example traffic routeing; specified areas; cautions; etc.;” | ONG: |
| Main Doc | DE | 6.1.4 | Second paragraph List item 2 | te | In accordance with Annex D and the combination of S-101, S-102 and S-104, this is not the safety contour, but the enhanced safety contour. | Change: “A combination of S-101, S-102 and S-104 is used to develop the safety contour (if already selected by the user) in accordance with Appendix D” To: “A combination of S-101, S-102 and S-104 is used to develop the enhanced safety contour (if already selected by the user) in accordance with Appendix D” | ONG: |
| Main Doc | DE | - | - | ed | Double spaces in various places in the text. | Replacing double spaces with single spaces. | DONE. Have corrected as many as practicable. |
| Main Doc | DE | 11.8, 11.9.1 | - | ed | “1:50,000” and “1/80,000” | To harmonize | DONE |
| Main Doc | DE | 11.10.3 | - | ed | To replace the given note with the comment would be fine. | “If the (default 30 m or the selected) safety contour is not available, then the ECDIS should use the next deeper safety contour.” | ONG: Discuss. Other comments apply (also history) |
| Main Doc | DE | 11.11.6 | - | ed | Question 2. in the comment should answered with “no”. Unless there are valid examples for “non-official data”. | To delete this section | ONG: DISCUSS |
| Main Doc | DE | 19.5 | List item 2 | ed | No cross-reference required | To delete | ? |
| Main Doc | DE | 19.5 | List item 3 | ed | Cross-reference is no longer current | “See 20.4” should be updated to “See 19.4” (will certainly be done automatically when the change is accepted ?) | DONE |
| Main Doc | DE | C-1 |  | te | Because the information for all datasets bases on the S-128, only one Update Status Report is required. | “The ECDIS must, therefore, implement two Update Status Reports: …” change to “The ECDIS must therefore implement an Update Status Report based on the S-128 PS including the currently available datasets.” | ONG: CHECK |
| Main Doc | DE | D-1 |  | ed | Cross-reference is no longer current | “as defined in 12.6.1.” should be updated to “as defined in 11.6.1.” | DONE |
| Main Doc | DE | D-1.2 | Item 2 | ed | “i. Overlaps within the same dataset (in areas with multiple vertical datums) the overlap must be resolved by taking the shoalest value for each grid cell” - This case cannot occur because the validity of a vertical datum is clearly defined by the domain extent polygon and is checked via a S-158 test. | To delete all corresponding text passages | ONG: Is not possible? |
| Main Doc | DE | D-1.4 | Third paragraph | ed | “If no uncertainty values are stored within the grid data then the values of maximumUncertainty must be used, these will be equal in a dataset with no separate uncertainty grid values.” | If the maximumUncertainty value equals the fill value (defined by the Product Specification), the depths value calculation must be handled as if the “uncertainty” switch is off for the Product Specification. | ONG: CHECK |
| Annex A | DE | 5 | Item 2 | ed | “a. Overlaps are strictly defined as those between grid cells containing data, as opposed to grid cells with no-data (fill) values. Datasets may contain overlapping grid cells, and different datasets may overlap, most commonly at national borders, but also in areas where there are multiple vertical datums within a single dataset. In these cases” The sentence is incomplete. | ?? | Needs a colon to introduce the list. DONE |
| Annex A | DE | 7.2.1 | Third item | ed | Cross-reference is no longer current | “See 20.4.” should be updated to “See S-98 Main Document 19.4.” | DONE (I think). |
| Main | IT | 5.1 | 1 | ge | IMO Resolution MSC.191(79) cannot reference IMO MSC.1/Circ.1609 because MSC.191 was adopted in 2004, while MSC.1/Circ.1609 was issued much later, in 2019. | IMO Resolution MSC.191(79), as amended by MSC.466(101), provides mandatory requirements for presentation of navigation-related information on shipborne navigational displays that are applicable to ECDIS and INS. including references to IMO MSC.1/Circ.1609 | DONE |
| Main | IT | 6.1.1 | 1 | ed | A semicolons and a full stop are missing to make the bullet list more consistent. | - S-101 ENC data as the base layer, alongside S-57 ENC data during the transition period; - S-102 Bathymetric Surface, only for Enhanced Safety Contour and Water Level Adjustment (Appendix D); - S-104 Water Level information for Surface Navigation, only for Water Level Adjustment (Appendix D); - S-111 Surface Currents, additional requirements apply (Section 13.1 and Appendix G); - S-122 Marine Protected Areas; - S-123 Marine Radio Services; - S-127 Marine Traffic Management; - S-131 Marine Harbour Infrastructure; - S-124 Navigational Warnings, additional requirements also apply (Section 12.12); - S-128 Catalogue of Nautical Products, only within Update Status Reports (Appendix C) and graphical indexes (Section 12.4); - S-129 Under Keel Clearance Management. | DONE |
| Main | IT | 6.1.4 | 1 | ed | Standard display is the display mode intended to be used as a minimum during route planning and route monitoring | Standard display is the display mode intended to be used as a minimum requirement during route planning and route monitoring | ONG: I’m not sure the wording is correct? Needs review. |
| Main | IT | 11.8 | 3 | ed | The full stop is missing. | The IMO Performance Standard for ECDIS MSC.530(106)/Rev.1, clause 6.1.3 states that ECDIS should provide an indication if information at own ship's position is not displayed because of applying scale minimum for display. | DONE |
| Main | IT | 11.9.10 | 2 | ed | This wording is recommended for greater clarity. | The selector used to take accuracy information into account must be called “Uncertainty”. | DONE |
| Main | IT | 11.10.1 | 1 | ED | Some semicolons are missing to make the bullet list more consistent. | - Units for Depth; - Units for Height; - Scale of display; - Data quality indicator; - Sounding/vertical datum; - Horizontal datum; - The value of the safety depth selected by the mariner,; - The value of the safety contour selected by the mariner,; - The value of the safety contour displayed (which may be different from that selected by the mariner); - Method of Water Level Adjustment (if enabled) - Magnetic variation; - Date and number of last update(s) affecting the datasets currently in use; - Edition number and date of issue of the datasets currently in use; - Display projection. | DONE |
| Main | IT | 1 | 11.11.1 | ed | The full stop is missing. | In addition, it must provide updating information (identification and source of update (i.e. manual update), when entered etc) on cursor picking. | DONE |
| Main | IT | 1 | 11.12.1 | ed | Two colons and semicolons are missing | A dedicated interface must be provided to allow users to interact with NAVWARN messages. This interface must, provide the following minimum functionality;: The user must be able to classify individual messages in accordance with section 12.12.2; An on-demand listing of all (S-124) NAVWARN messages in the system must be provided allowing sorting according to: installed date and time, issue date and time, warning type, producer, series and message classification (as defined in 12.12.2); The on-demand listing of all NAVWARN messages in the system must include means for viewing an abbreviated view of any NAVWARNPart, warningInformation attributes present. This should consist of a minimum of the issue date, warning type and the first 64 characters of the warningInformation attributes.; An indication must be provided when a new NAVWARN message is loaded until it has been displayed or 24 hours have passed. This indication may be suppressed if the NAVWARN message does not meet filtering criteria set by the mariner or automatically applied (as defined in 12.12.2).; Means must be provided to view the most recent message and past messages. | DONE |
| Main | IT | 11.12.2. | 3 | ed | The full stop is missing. | Even though the full extent of a navigational warning may not be portrayed, it must still be available and discoverable in a list of NAVWARNs that can be recalled by user action at any time. | DONE |
| Main | IT | 12.2 | 4 | ed | The full stop is missing. | If the selected display time is outside the interval [(startDateTime – discriminationInterval), (endDateTime+discriminationInterval)] then the data must not be displayed. | DONE |
| Main | IT | 14.3.1 | 2 | ed | The full stop is missing. | In lieu of this method, the OEM may employ an alternate consistent and logical sorting method for the display of pick report results. | DONE |
| Main | IT | 14.4 | 3 | ed | The full stop is missing. | Multiple instances of Tidal Stream Panel Data features must be shown as separate tables. | DONE |
| Main | IT | 18.4.1 | 6 | ed | Comma missing | Consequently, these tolerances should not be applied to the night table; and for type-approval purposes they are restricted to the bright sun table. | DONE |
| Main | IT | 18.5 |  | ed | The full stop is missing. | The ECDIS Chart 1 and its indexing list of symbol names and meanings arranged numerically, together with the colour differentiation test diagrams, are intended for the Mariner’s use. The use of the Colour Differentiation Test Diagrams is described in 19.6.2. | DONE |
| Main | IT | Appendix D | 1 | ed | There is no 12.6.1 in the document | In these processes an end user selects: - Suppression of specific S-101 features by S-102.; - A Safety Contour value of their choice. When a user selects a safety contour value, outside areas of S-102 coverage the source for the displayed safety contour is the existing S-101 ENC (or S-57 ENC in dual-fuel mode) as defined in 12.6.1. | DONE |
| Main | IT | Appendix D | 1.1 | ed | Semicolons are missing | Sun illumination may be provided. Where provided: - The mariner must be able to select whether it is enabled or not.; - Selection of Base or Standard Display shall disable sun-illumination; - The ECDIS must use a consistent sun azimuth and elevation values for all data portrayed; - The mariner may be able to vary the sun azimuth and elevation values. | DONE |
| Main | IT | Appendix D | 1.2 | ed | Two colons and semicolons missing. | - Overlaps are strictly defined as those between grid cells containing data, as opposed to grid cells with no-data (fill) values. Datasets may contain overlapping grid cells, and different datasets may overlap, most commonly at national borders, but also in areas where there are multiple vertical datums within a single dataset. In these cases: - Overlaps within the same dataset (in areas with multiple vertical datums) the overlap must be resolved by taking the shoalest value for each grid cell (including vertical uncertainty when the Uncertainty selector is enabled) within the intersection. This is also used to resolve situations when the Mariner Selected Viewing Scale (MSVS) means that multiple grid cells are to be portrayed at the same screen position (the common point rule) – most often when the grid is being portrayed at a smaller zoom scale than the dataset resolution.; - Overlaps between different producers. The end user must be able to select which dataset (or data producer) they wish to use. | DONE |
| Main | IT | Appendix D | 2.7 | ed | Full stop missing | The same user-specified distance must be used for the check of safety contour, prohibited areas, geographic areas for which special conditions exist and navigational hazards and equivalent requirements when route monitoring. … When WLA is based on the planned schedule and own ship is not keeping to schedule a Caution must be raised to indicate the water level being experienced may be different to that being applied by the ECDIS. | DONE |
|  | Sperry | 8.1.2 |  | Ed | This is the only instance in the document where “black adjust” Is not hyphenated; consequentially text searches for the term do not always find it | Correct to “black-adjust” | DONE |
|  | Sperry | 8.3 |  | Ed | The references to clauses 12.12.1 and 20.4.4 are incorrect | Reword to “are given in clauses 11.11.1 12.12.1 and 20.4.4” | DONE |
|  | Sperry | 8.3 |  | Ed | The reference to clause 12.12.4 is incorrect | Correct reference | DONE |
|  | Sperry | 11.3 |  | Ed | Traceability to the requirement in the IMO PS for an ‘overscale indication’ should be improved. | Reword to “This overscale indication is required by IMO Performance Standards (MSC.530(106)/Rev.1 clause 6.1.1.)” | DONE |
|  | Sperry | 11.3.2 |  | Ed | There are multiple stage 3b in Appendix E | Specify sub-clause of Appendix E when referencing stage 3.b | Row 3b E-1.3, clarifined. DONE |
|  | Sperry | 11.3.2 |  | Ed | The table has alignment issues | Correct alignment of the first column | Tried. Needs more work. DONE |
|  | Sperry | 11.3.3 |  | Ed | The most recent change to this section lost the traceability to the requirement in the IMO PS | Add “, as required by MSC.530(106) Rev.1 clause 6.1.2.” at the end of the sentence. | DONE [check] |
|  | Sperry | 11.4.2 |  | Te | Change to be consistent with the para 3.3 statement that “Non-mandatory requirements or recommendations to implementers are indicated using the word “should”.” | Change ‘shall’ to ‘must’ | DONE [check] |
|  | Sperry | 11.5.1 |  | Ed | Reference to 9.1.1 is incorrect. Is this intended to reference 11.5.2? | Correct reference. | It’s 8.1.1 (non-ENC, but I believe this is non-OFFICIAL, needs correcting] DONE |
|  | Sperry | 11.5.1 |  | Te | The concept of “of other than ENC data” would benefit from clarification. Is this unofficial ENCs, non-S-101 data, both? Or can this be deleted? | Clarify | ONG: Yes, needs clarifying. |
|  | Sperry | 11.5.2 |  | Ed | The reference to 9.1.1 is incorrect. The correct reference is 8.1.1, but this doesn’t provide a definition of “official” data, instead it describes how to determine whether the data is official. | Reword “The definition of official data as opposed to non official data is contained in 9.1.1” to “Para 8.1.1 describes how the ECDIS determines that the data is official”. | DONE (also 11.5.3) |
|  | Sperry | 11.5.3 |  | Ed | Para 3.3 reserves the word “may” to express a permissible practice for action. It does not express a requirement. | Reword to: “NOTE: For some types of coverage products (for example gridded data), no-data areas may can be easily distinguished by the lack of portrayal | DONE |
|  | Sperry | 11.5.3 |  | Te | There are two references to “no data areas” but no actual requirements to provide such areas. As a result, the statements: - “For gridded data, the no-data area limits do not need to demarcate areas within the grid extent which are covered by fill values”, and - “either this clause should be implemented for all the non-ENC S-100 products, or for none of them” … are challenging to interpret. | Document the requirements for no-data areas. Currently, the only requirement is for what they do not need to do! | ONG: Agree, clarify. TSM |
|  | Sperry | 11.8 |  | Te | The Indication is mandatory. | Reword to “… ECDIS must should provide an indication if information at own ship's position is not displayed because of applying scale minimum for display” | Check w DE comment. DONE. |
|  | Sperry | 11.10.3 |  | Te | Current wording of the note implies that the ‘Value of safety contour’ in the “ECDIS legend’ only needs to display the values for both selected and displayed contour when the displayed contour is 30m (default). This isn’t the intention. Also, this is a requirement, not an informational note. | Reword to ”Note: If the Mariner has selected a contour that is not available in the ENC and the ECDIS displays a default contour, both the contour selected and the contour displayed at the selected position must be displayed quoted” | ONG: Check. More comments on this. Suggest better words. |
|  | Sperry | 11.11.1 | 2nd para | Te | The requirement is to provide “identification” of the manual update in addition to “source of update (i.e. manual update) and that provided by the ENC feature. The requirement would benefit from clarification. Probably there is no additional identification needed. | Reword to: “The following details must be available when the manual update object is selected by the cursor pick feature: - All attributes available from the manual update feature catalogue - Source of update (i.e. manual update) - Date and time of update - Mariner entered text associated with the manual update object” | ONG: Check |
|  | Sperry | 11.11.1a |  | Te | The “must” followed by “should” results in a loss of clarity | Reword to: “The mariner must be able to enter manual updates for presentation on the display as follows:. A capacity should exist to enable the mariner to.” | DONE |
|  | Sperry | 11.11.1 a.2 |  | Te | The requirement “Ensure all update text information relevant to the new condition and to the source of the update, as entered by the Mariner, is recorded by the system for display on demand” would benefit from clarification: “New condition” isn’t clear. “source of update” isn’t entered by the mariner – its always “manual update” “for display on demand” has already been covered by the cursor pick requirement above | Reword a.2 to: “Enter text associated with the manual update object.” | ONG: Ok, review |
|  | Sperry | 11.11.1 b |  | Te | This requirement is that manual update alerts and indications are “using the same criteria as for ENC features”, whereas Appendix A-2 requires that they are as defined in the feature catalogue. There’s the potential for conflict if the feature catalogue differs from the ENC object. Since our understanding is that the feature catalogue doesn’t yet include alerts and indications, there’s real potential for conflicting requirements. | Reword to “The ECDIS should provide indications and alerts using the same criteria as for ENC features defined in Appendix A” | ONG: Needs work. Use S-101 Alert Catalogue applied to features in manual update catalogue to avoid replicating. [review] |
|  | Sperry | 11.11.1. d |  | Te | The two “shoulds” should be converted to “musts”. An explicit requirement to view archived objects should be added. (Currently this is only implicit.) | Reword to: “Archiving of Manual Updates: The mariner must be able to remove any manual update object from display. Removed manual update objects must be archived at least until the commencement of the next voyage. It must be possible to review archived manual update objects.” | DONE |
|  | Sperry | 11.11.2 |  | Te | This paragraph should be aligned with the current draft of IEC 61174 Ed 5. “should” should be “must” | Reword entire section to: “The mariner must be able to add: - Mariner’s caution note (!) and mariner’s information note [i] used to call up a note on the text display by cursor picking - Mariner’s information lines and mariner’s information areas without colour fill, set up for cursor picking to give explanatory notes in the text display. - Mariner’s text notes - Mariner entered danger IEC 61174 adds further requirements for Additional Mariner’s Information. Additional Mariner’s Information must be presented according to IEC 61174 or IEC 62288, as applicable.” | DONE |
|  | Sperry | 11.11.5 |  | Ed | IMS is an undefined abbreviation | Spell out term. | DONE – IMS is gone |
|  | Sperry | 11.11.5 |  | Ed | Typo | Reword to “… actually updated …” | DONE |
|  | Sperry | 11.11.5 |  | Te | IMS is an undefined term in S-100 and S-101. Some requirements are needed to guide OEMs otherwise they won’t know what to implement. | Add IMS related requirements | IMS is deleted. DONE |
|  | Sperry | 11.12.1 | 2nd bullet | Te | Is “installed date and time” essential to the minimum standard? There’s a risk of cluttering the display. | Consider removing “installed date and time” | ONG: Check with 124 PT |
|  | Sperry | 11.12.1 | 2nd & 3rd bullets | Te | The term in S-124 is “publication time”, not “issue time” | Replace “issue time” with “publication time” in 2nd and 3rd bullet. | Agree – this will use dataset, not CATALOG.XML DONE |
|  | Sperry | 11.12.2 |  | Ed | The title is “classifying” only but the contents additionally covers filtering | Reword title as “Filtering and classifying navigational warnings” | Ok |
|  | Sperry | 11.12.2 |  | Te | Presumably the first paragraph refers to avoiding a cluttered graphical display However, it’s unclear whether the intention is that the filtering applies to both the graphical display and the “on-demand listing of all NAVWARN messages”. Is there an unstated implication that the ECDIS must provide both an “on-demand listing of all NAVWARN messages” and an “on-demand listing of filtered NAVWARN messages”? If so, it’s preferable to introduce this concept in 11.12.1. | Reword to “To ensure only relevant information is displayed, and to ensure an uncluttered screen, the mariner must be able to classify NAVWARNs which are then filtered for graphical display by the ECDIS” If there’s a requirement for an “on-demand listing of filtered NAVWARN messages” this should be explicitly stated. | Check |
|  | Sperry | 11.12.2 | Bullet 1 | Te | “Must always be displayed” is presumably not the intention. - If the S-124 layer is off the symbols won’t be displayed, although the on-demand listing will remain available - Other filters may prevent the symbols from being displayed. | Needs clarification that will be dependent on the outcome whether filtering applies to both the graphical display of Navwarn symbols and the “on-demand listing”. Probably, the filtering paragraph needs to be updated to include the classification as one of the filter criteria, as this is not currently explicit. | DISCUSS |
|  | Sperry | 11.12.2 | Bullets 1 to 3 | Ed | Presumably there’s no intention to distinguish between “visualised” and “displayed” | Replace “visualised” with “displayed” | Agree |
|  | Sperry | 11.12.2 | Bullet 4 | Ed | The fourth bullet is not a classification option | Convert 4th bullet to body text. | ONG: Ok, DONE |
|  | Sperry | 11.12.2 | 3rd para | Te | Filtering is by time or maximum distance from own ship, not time and maximum distance from own ship Proximity and distance are the same quantity so “proximity or distance” needs correction. Probably its minimum distance that a line or area gets filtered for, not maximum distance, but also probable that this detail is unnecessary. | Reword to: “A means must be provided for the mariner to enter criteria for automatic filtering of NAVWARN messages based on time or and maximum distance from own ship, monitored route or planned route. When automatic filtering is applied by proximity time or distance any messages without spatial extent must be classified as Information.” | ONG: Agree. Review w/124PT |
|  | Sperry | 11.12.2 | 4th para | Te | Given that the user manually classifies and the ECDIS automatically filters (according to user settings), the statement “When automatic filtering is applied by proximity or distance any messages without spatial extent must be classified as Information” doesn’t really reflect the intention. Classification, by the user, takes place before the filtering, not as a result of it. Put differently, S-98 can’t place requirements on what the mariner does while operating the ECDIS! Additionally, the sentence “When automatic filtering is applied by proximity or distance any messages without spatial extent must be classified as Information.” suggests that the classification of “Information” applies to NAVWARNs without spacial extent only. If, as suspected, this is the intention, the manual assignment of “Information” classification, referenced in the third bullet, is unnecessary as the ECDIS can apply this automatically. More precisely, the manual classification of “on chart” or “off chart” only applies to NAVWARNs with special extent. | Rewrite to clarify that the classification of “On chart” and “Off chart” apply to navigational warnings with special extent only. As a consequence, the classification of Information becomes redundant. | ONG: CHECK |
|  | Sperry | 11.12.2 |  | Te | Since the classifications of “On chart” and “Off chart” relate to a route, the question arises – what happens when the mariner selects a new route (during planning or monitoring)? Presumably the classification should be cleared (unless it was linked to a particular route, but there is no requirement to do so). Perhaps the concept of “relevant to the route” is not exactly as intended, and this is more about the mariner marking individual navigational warning symbols for display or not (although the equivalent for ENCs is expressly prohibited). | Consensus needed on the concept of classification. | ONG: Review w/124PT |
|  | Sperry | 11.12.2 | 5th bullet | Te | The concept of “Navigational warning topic” is not described in S-98 or S-124 | Delete | ONG: Check w / Ed. May need to ref the FC (warning type) |
|  | Sperry | 11.12.2 | 6th bullet | Te | “Date range of the hazard” would benefit from clarification wrt what S-124 attribute this refers to. Hazard has a specific meaning on an ECDIS that is distinct from Navigational Warning, so is not the correct term here. | Reword to “Date range (temporal extent) of the hazard Navigational Warning; | ONG: As long as we’re explicit it’s ok. Probably comes from dataset, not CATALOG.XML |
|  | Sperry | 11.12.2 | 7th bullet | Te | “Valid time of the navigational warning” would benefit from clarification wrt what S-124 attribute it refers to | Clarify | ONG: Get attribute from dataset. (should be the attribute name) |
|  | Sperry | 11.12.5 |  | Ed | Suggest a simple introduction to the NAVWARNAreaAffected feature is provided. | Add “The NAVWARNAreaAffected feature marks areas impacted by a navigational warning outside the immediate vicinity where the navigational warning itself is placed” | ONG: Ok |
|  | Sperry | 11.12.5 |  | Te | An explicit requirement not to portray NAVWARNAreaAffected is needed. The highlight symbol should be specified. Note here that IEC TC80 MT7 will discuss documenting such a symbol in their February meeting. | Reword to: The NAVWARNAreaAffected feature does not have a portrayal defined and must not normally be portrayed to avoid causing significant cluttering on the navigation screen. The feature must be highlighted by the system if selected from a pick report for interrogation by user using the “Navigational Warning Area Affected Highlight” symbol defined in IEC 61174 or IEC 62288, as applicable”. | ONG: Review |
|  | Sperry | 14.4 | Table 6 | Te | Given that this is a normative table (see second paragraph) some adjustments are needed | - Column heading should read “Rate” not “Rates” - Rate units should be in each of the blue cells, and not hard coded as the column heading (show as “knots” or “kn” or compliance with IEC 62288 - Degrees symbol should be shown against each direction value and “(degrees)” can be removed from the column title - “high water” and “springs” should be shown in blue - Stream depth should have units | ONG: Check. Should be ok. Needs agreement. |
|  | Sperry | 17.1 |  | Te | First bullet: “should” should be “must” | Replace “should” with “must” | DONE |
|  | Sperry | 18.6.4 |  | Te | This requirement that “An ECDIS must have the black-adjust symbol displayed whenever the mariner is adjusting the display” conflicts with the requirements in 18.3.6 that the symbol must simply be available (as per S-52). | Reword to: “An ECDIS must make have the black-adjust symbol available displayed whenever the mariner is adjusting the display …” | Check S-52. DONE |
|  | Sperry | 18.6.4 |  | Ed | Incorrect reference to clause 19.3 | Correct reference. To 18.3.6? | 18.3 – check. DONE |
|  | Sperry | 19.1 |  | Te | Should the mariner be able to uninstall a catalogue to “free up room”? | Add requirement | ONG: It’s inferred – you only need to keep catalogues for which datasets exist. Related to minimum requirement. |
|  | Sperry | 19.1 | Last para | Te | Should there be an indication if a dataset is called up for display and a required catalogue is not found? | Add requirement | ONG: I believe this is in the SSE codes. We can clarify in text. |
|  | Sperry | 19.1 | Last para | Ed | The last sentence duplicates same requirement as the first sentence in this paragraph. | Delete | ONG: Take this last para out. |
|  | Sperry | 19.2 | First para | Ed | The first sentence triplicates the requirement in the first and last sentences of 19.1 | Delete | ONG: Reword |
|  | Sperry | 19.1. 19.2 |  | Te | Specify the minimum number of catalogues to be supported. | Specify the minimum number of catalogues to be supported. | ONG: In discussion. I believe this is the “minimum requirement for the maximum number to be supported.” |
|  | Sperry | 19.2 |  | Te | It is not entirely clear what each edition/revision/clarification numbers is being referred to in all cases. Possibly the statement can be simplified and thereby clarified. | Reword to: - Compatible datasets have identical dataset Edition and Revision numbers - Portrayal Catalogues compatible with Feature catalogues also have identical catalogue Edition and Revision Numbers. - Dataset and catalogue Editions with different clarification numbers but identical Edition and Revision numbers are all compatible. - So, it is possible to process datasets conforming to a one catalogue Clarification number with the Feature and Portrayal Catalogues for an earlier Clarification number but within the same Edition and Revision number. Preferably this can be reduced to: “A dataset and associated feature catalogue and portrayal catalogue are compatible when their Edition and Revision numbers are all the same. When the Edition or Revision numbers are different for any of the dataset, feature catalogue or portrayal catalogue, compatibility cannot be assumed. A difference in Clarification number between the dataset, feature catalogue and portrayal catalogue has no bearing on compatibility.” | Agree, this came from a paper so edited first set of words would be better… DONE |
|  | Sperry | A-1 | 1st para | Te | Wrt “the symbols and attributes listed in Table 8, Table 9 and Table 10 must be supported.” The tables don’t define the attributes. A-2 provides information on attributes. | Reword to ““the symbols and attributes listed in Table 8, Table 9 and Table 10 must be supported.”” | DONE |
|  | Sperry | A-1 | 3rd para | Te | It is presumably intended that the portrayal catalogue is used, but there is no requirement to do so. | Reword to “In order to provide a harmonised and consistent approach to manual editing and update on the ECDIS the a set of portrayal symbols is supplied alongside this publication must be used.” | ONG: Use symbols from portrayal catalogue |
|  | Sperry | A-2 |  | Te | The clause states “The supplied feature catalogue contains a restricted set of features and attributes compatible with those used for S-101 electronic charts which enable portrayal and required alert and indication processing to be carried out on manually entered updates.” However, no alert or indication specifications have been found in the catalogue. | Either add alert and indication support to the catalogue, or define them textually in A-2, or liaise with IEC TC80 MT7 to get them added to IEC 61174. | ONG: Share definitions from S-101 Alert catalogue. ? |
|  | Sperry | D-1.1 | Bullet 2 | Te | Change to be consistent with the para 3.3 statement that “Non-mandatory requirements or recommendations to implementers are indicated using the word “should”.” | Change ‘shall’ to ‘must | DONE: Check 4th bullet too.? |
|  | Sperry | D-1.1 |  | Te | Ed 2.5.5 saw “Provision of Sun Illumination” moved to D-1.1. Para 6.1.1 states that S-102 can only be used for Enhanced Safety Contour and Water Level Adjustment, and references Appendix D in that context. So, sun illumination remains permitted, but now only because it is considered part of the ESC. The corollary is that, if sun illumination were not considered part of the ESC it would not be permitted to be used (i.a.w. 6.1.1). Is the intention to allow sun illumination only when ESC is in use? In which case clarification or a new requirement is needed in D-1.1. Or is the intention to allow the ESC & sun illumination independently of each other? In which case D-1.1 is not really the correct location for the sun illumination requirements. | Clarify/rework as required, depending on the intention. | ONG: MTG – Discuss. |
|  | Sperry | Feature Catalogue |  | Ed | <S100FC:definition> should be consistent for all Manual Update Feature Types. | Add “Manually entered by a mariner.” at the end of the following <S100FC:definition> : Other, Buoy, Beacon. | ONG: Check |
|  | Sperry | Annex A / Feature Catalogue |  | Te | Symbols should be consistently named in Annex A Table 10 and the Manual Update Feature Catalogue. | Align symbol names by renaming “Restricted” to “Restricted Area” and “Caution” to “Caution Area” in the FC | ONG: Check |
| S-98 | DE1 |  | Whole Table of comments |  | All yellow marked comments are old (already submitted) and were not considered. |  | Ok. |
| S-98 | DE2 | 2.1 | 1st paragraph | ge | Add IEC 63173-1 | Add IEC 63173-1 Maritime navigation and radiocommunication equipment and systems – Data interfaces – Part1: S-421 route plan based on S-100, Edition 1, 2021. | ONG: Check |
| S-98 | DE3 | All |  | ge | MSC.530(106) should be MSC.530(106)/Rev.1 Check the whole document. | MSC.530(106) should be MSC.530(106)/Rev.1 | DONE |
| S-98 | DE4 | 4.1 | 1st paragraph | ed | They were introduced in IMO MSC 530(106) and… MSC.530(106) should be MSC.530(106)/Rev.1 | MSC.530(106) should be MSC.530(106)/Rev.1 | DONE |
| S-98 | DE5 | 4.2 | b) | ed | b) …nautical chart and nautical publications carriage requirement required by regulations V/19 and V/17 of the 1974 SOLAS… Change regulation to regulations and V/17 to V27. | Change to: …nautical chart and nautical publications carriage requirement required by regulations V/19 and V/27 of the 1974 SOLAS… | DONE |
| S-98 | DE6 | 6.1.1 | 1st paragraph | ed | 13.1, 12.12, 12.4 Wrong references | If appropriate change to 12.1, 11.12, 11.4 |  |
| S-98 | DE7 | 6.1.4 | 1st paragraph | ed | The MSC.530(106) should be MSC.530(106)/Rev.12. MSC.530(106)/Rev.1 | MSC.530(106) should be MSC.530(106)/Rev.1 | DONE |
| S-98 | DE8 | 6.1.4 | 2nd paragraph | ge | A combination of S-101, S-102 and S-104 is used to develop the safety contour (if already selected by the user) In D-1.3 1st paragraph The Enhanced Safety Contour must be enabled by default. Because of the statement in paragraph D-1.3, may be it should be better to write (if not deselected by the user) | Change to A combination of S-101, S-102 and S-104 is used to develop the Enhanced Safety Contour (if not deselected by the user). . | Needs discussion. |
| S-98 | DE9 | 6.1.4 | 2nd paragraph 1st bullet | te | How can all other S-100 data products be suppressed when the user selector settings are preserved? The presentation of other S-100 data products are controlled by user selector settings. | Reverse order of bullets and change: “User selector settings must be preserved” to “All other user selector settings must be preserved” | discuss |
| S-98 | DE10 | 6.2 | 1st paragraph | ed | Section 8 In the whole document sometimes, we are using the term clause and sometimes we are using the term Section. | To be consistent may be we should use the same term everywhere. | Clause is now used, or section when a high level section is being referred to.. Has been changed DONE |
| S-98 | DE11 | 6.2 | 2nd paragraph | ed | data overload Add a full stop after overload. | data overload. | DONE |
| S-98 | DE12 | 9.2.1 | 1st paragraph | te | Add IEC 61174 for symbols not yet available in the IEC 62288. | Add following statement before the last sentence: Requirements for additional symbols not yet available in the IEC 62288 are defined in IEC 61174. | ONG: Check w/IEC |
| S-98 | DE13 | 11.1 | 1st paragraph | ed | … ENC features is contained in… Broken link. | Fix the broken link. | DONE |
| S-98 | DE14 | 11.1.1 | Last paragraph | ge | -- prior to S-101 portrayal is defined fully in… Broken link | Fix the broken link. | DONE |
| S-98 | DE15 | 11.3.2 | 1st and 2nd paragraph |  | Two broken links. | Fix the broken links. | DONE |
| S-98 | DE16 | 11.4.2 | 2nd paragraph | te | The display of each supported data product shall be graphically differentiated. Change shall to must. | Change to : … data product must be graphically differentiated. | ONG: Check. I think this was meant to be optional |
| S-98 | DE17 | 11.8 | 2nd paragraph | te | The IMO Performance Standard for ECDIS MSC.530(106)/Rev.1, clause 6.1.3 states that ECDIS should provide an indication if information at own ship's position is not displayed because of applying scale minimum for display. | Change to: ECDIS must provide an indication if information at own ship's position is not displayed because of applying scale minimum for display. | DONE, using agreed words. |
| S-98 | DE18 | 11.10.2 | 1st paragraph | ed | ….in Appendix D-1.2 When the.. Add a full stop after D-1.2. | Add a full stop after D-1.2. | DONE |
| S-98 | DE19 | 11.12.1 | 1st paragraph Bullets 1 & 2 | ed | 12.12.2 should be 11.12.2. | Replace 12.12.2 by 11.12.2. | DONE |
| S-98 | DE20 | 11.12.3 | Note | ge | EGC message. EGC is not included in the list of abbreviations and Notation | Change to EGC (Enhanced Group Call) message | DONE |
| S-98 | DE21 | 14.4 | 2nd paragraph | ed | Table 6 should be Table 5 | Change Table 6 to Table 5 | DONE |
| S-98 | DE22 | 14.4 | 4th paragraph | ed | Broken link after table | Fix the link. | DONE |
| S-98 | DE23 | All Appendixes | Figures and Tables | ge | Tables and figures should be numbered according to their appendix (e.g., Table A-X-X, Figure A-X) to avoid duplicate numbering throughout the document. Ensure that the references in body text are correct. | Tables and figures should be numbered according to their appendix (e.g., Table A-X-X, Figure A-X) to avoid duplicate numbering throughout the document. Ensure that the references in body text are correct. | ONG: ONGOING |
| S-98 | DE24 | A.1 | 1st paragraph | ed | Table 8, Table 9 and Table 10 should be Table 7, Table 8 and Table 9. Use terminology as mentioned above. | Change Table 8, Table 9 and Table 10 to Table 7, Table 8 and Table 9. Use terminology as mentioned above. | DONE |
| S-98 | DE25 | A.1 | 1st paragraph | ed | Proposal for the table: Table 8 should be Table A-8 or Table A.1 |  | ONG: As per above |
| S-98 | DE26 | B.1 | 1st paragraph | ge | Add Error Code SSE 136 and SSE 137 | Add error code SSE 136 and SSE 137 | DONE |
| S-98 | DE27 | B.1 | Table B-1 | ed | X509 should be X.509 A full stop is missing after X. | Change X509 to X.509. | DONE |
| S-98 | DE28 | B.2 | 1st paragraph | ge | Error code descriptions for SSE 127 and SSE 130 are missing. | Add error code descriptions for SSE 127 and SSE 130. | ONG: Needs discussion |
| S-98 | DE29 | B.2 |  |  | SSE 129 …(as identified according to section 9.1). Wrong reference. | If appropriate replace section 9.1 by section 8.1.1. | DONE |
| S-98 | DE30 | B.3 | Figure 6 | te | 134 should be 135 | Change 134 to 135. | ONG: IMG edit required |
| S-98 | DE31 | C-1 |  | ge | Product Independence: 2nd bullet S-128 datasets are “official” data by definition and have a data producer status (as defined in section ) Which section? The reference is missing. | Which section? The reference is missing. | DONE |
| S-98 | DE32 | C-1 |  | ge | Data Server content tables - …….as identified by Section 9.1.1) Wrong reference. | If appropriate replace section 9.1 by section 8.1.1. | DONE |
| S-98 | DE33 | C-1 | - Figure | ge | Figure 1 and Figure 2 should be Figure C-1-1 and Figure C-1-2. Figure 1 and 2 are already covered in other clauses. | Change Figure 1 and Figure 2 to Figure C-1-1 and Figure C-1-2. The cross-reference in the text should also be updated | DONE |
| S-98 | DE34 | C-1 | Table 3 | ed | Table 3 should be Table C-1-1. Table 3 is already covered in another clause. | Change Table 3 to Table C-1-1. The cross-reference in the text should also be updated. | DONE |
| S-98 | DE35 | C-1 | Table 4 | ed | Change Table 4 to Table C-1-2. | Change Table 4 to Table C-1-2. The cross-reference in the text should also be updated. | DONE |
| S-98 | DE36 | D-1.2 |  | ge | (S-100 Part 8, clause 8-7.1.4), … Clause 8-7.1.4 not found | Fix the reference | DONE – 8..7.7.3 |
| S-98 | DE37 | D-1.3 | Last sentence | ed | ..MSC 530(106) 11.4.3, which the OEM must implement… | MSC.530(106) should be MSC.530(106)/Rev.1 | DONE |
| S-98 | DE38 | D-1.4 | Whole paragraph | ge | # D-1.4 Vertical Uncertainty in the calculation of depth values. Either split this clause or move it to the D-2 WLA. Because of the mentioned S-104 uncertainty. | Either split this clause or move it to D-2 WLA. | ONG: CHECK |
| S-98 | DE39 | D-2.7 | 1st paragraph | ed | …the user as specified by IMO MSC 530(106) 11.3.7. | MSC.530(106) should be MSC.530(106)/Rev.1 | DONE |
| S-98 | DE40 | D-2.7 | 4. | ed | …across the time period (ti-tu2) and (ti+tu2), In… Replace the comma by the full stop. | Replace the comma by the full stop. | DONE |
| S-98 | DE41 | D-2.7 | 8th paragraph | ed | …“limit of check area” as specified by IMO MSC 530(106) 11.3.7… | MSC.530(106) should be MSC.530(106)/Rev.1 | DONE |
| S-98 | DE42 | D-3.1 | 2nd paragraph | te | Will WLA also adjust the z value of soundings as for ESC? Add sounding to this paragraph as in paragraph before. | Change to depth attribution (that is, the attribute valueOfSounding bound to them in the S-101 Feature Catalogue and the z value of soundings) | Check DONE – this is already the text? |
| S-98 | DE43 | E-1.4.1 | Footnote | ed | Footnote 8. 12.3 should be 11.3. Wrong reference | Change 12.3 by 11.3. | DONE |
| S-98 | DE44 | G-1.1 | 3rd paragraph | ge | 3 broken links. | Fix the broken links. | Deleted and replaced with references. DONE |
|  | NOAA1 |  |  |  | 12.9.10.3 Check area extension value – S-102 depth substitution and S-104 adjustment: "When the depth values in the S-101 are substituted for S-102 and also adjusted by the S-104 values (as documented in Appendix D) then the total value for the extension of the check area, as defined in the previous clause must then be further adjusted by adding the uncertainty value in the S-104 feature attribute table zoneOfConfidence.horizontalPositionUncertainty.uncertaintyFixed if defined." Should this beS-102 feature attribute table? Don't think S-104 has this value. |  | ONG: Check. |
|  | NOAA2 |  |  |  | D-2.7 and Figure D-2-14: " S-102 point x is adjusted using values drawn from S-104 points a and b (because its extent intersects the extents of points a and b) and S-102 point y is only adjusted with values from S-104 point b." Why is S-102 point x only adjusted using values drawn from S-104 points a and b? S-102 point x coverage includes a small sliver of the S-104 points to the left of a and b extents, so should S-102 x be adjusted by all 4 S-104 points? Figure D-2-14 - Selection of S-104 data |  | DONE. Diagram is incorrect. |