| S-98 | OSI | 6.1.4 |  |  | “All other S-100 data products listed in 6.1.1 must be suppressed.” This statement can be deleted as it is redundant and potentially in conflict with the definitions of ‘Standard Display’ which turns on / off various viewing groups. All S-100 products (Phase 1, Phase 2, 8211, HDF5, or GML) use viewing groups. Simply following the VGs defined in the standard display (0-29999) should ensure the correct result. |  | This was requested for clarification previously [SM/AU?] – the idea is that standard display does not include any non-S-101 data. This shouldn’t conflict with any other requirements but agree it is, at worst, redundant. |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 12.4 |  |  | My notes from our TSM side discussion are: “Jonathan clarified this means select the display of the chart index”. My engineers re-reading this are confused by the use of “individually select” to mean toggle individual index rectangles one by one. This would be inconsistent to how other layers of information are handled (ie: toggle all or none of the same object type). To avoid ambiguity or misunderstanding this clarification if offered. | It must be possible to toggle individually select the display of the graphical index entries for the installed S-57 and S-101 ENCs. | Someone requested not to use the word “toggle” already? Could we change to “select” from “individually select”? |
|  |  | 12.4.2 |  |  | The sentence “The display of each supported data product shall be differentiated” is highlighted with the question “is this a requirement?”. My interpretation is each product type is differentiated by showing the dataset cell name which includes the product number in the name Therefore it is possible to differentiate between index rectangles from different products. |  | Suggest methodology with examples. |
|  |  | 12.7.2 |  |  | Permanent indication “Display Not Current Time” conflicts with the requirement in IEC 61174 5.3.3 which is sourced from S-52 Annex A, Part I/10.4.1 and states ‘Display Not Real Time’. | Settle on a consistent term and adjust either S-98 or IEC 61174 / S-52 Annex A. A similar issue in 12.5.1 was resolved by aligning the wording in S-98 to IEC 61174. | SM proposal (I think). Can be discussed, I believe if we change this then 61174 can be changed retrospectively |
|  |  | 15.3.1 | Sorting order of results | te | Recommend deleting this section “Sorting order of results” as it strays into design space and prevents innovative or alternative ways of presenting the pick report results. Sorting by drawing order is not the most intuitive way to present the results to the end user. | Add sentence “In lieu of the above, the OEM may employ an alternate consistent and logical sorting method for the display of pick report results.” | This seems ok to me. Can add as a proposal for 2.4.0. |
|  |  | 17 | Use of Context Parameters | te | Add words “, which may require an ECDIS software update / upgrade.” | Context parameters may be added or removed by updates to the Portrayal Catalogue, which may require an ECDIS software update / upgrade. | Needs some discussion. If CP are only used by portrayal what are the barriers to their dynamic update if the correct validation is done….?? |
|  |  | 20.1 |  |  | Table column header “DPS” likely should be “PS” (typo) |  | agree |
|  |  | 20.1 |  |  | This section does not specify any limit on the number of different versions of each S-100 product that must be supported. At TSM there was discussion of a 2 year window for HOs to upgrade their data. Discussion was also around the current major version and the previous major version. It can’t be unbounded. | Add some wording on the maximum number of edition, revision, and clarification versions that must simultaneously be supported. | Needs to be talked through with IHO Sec. Have passed it on. |
|  |  | A.1 |  |  | Need the Manual Update FC and PC |  | One is in development. |
|  |  | A.1 |  |  | There is an “Other” object for lines and areas, but not for points. Recommend adding this for consistency and to have a place for features that do not fit into the other point features. |  |  |
|  |  | B.3 |  |  | When installing Permits (S-98 B-3 Figure 7), it is perfectly OK to install and use expired permits (SSE 115 & SSE 125). However, when attempting to install chart with an expired (but otherwise valid) digital signature, the flowchart fails with SSE 122 (S-98 B-3 Figure 8). SSE 122 failures can occur if any of the SA certificates, DS certificates, or catalog certificates are expired. This failure will prevent the dataset from being displayed. I think that despite SSE 122, the end user should be able to load (but of course no longer update) the dataset in the same way they can when the permit is expired. Can we get the flowchart updated and SSE 122 downgraded from a failure to a warning like SSE 125? |  | Agree – propose new words for review. |
|  |  | C-1 |  | ed | Typo, change MSC.532(106) to MSC.530(106) |  |  |
| S-98 Annex A | OSI | General |  |  | Ensure HDF5 data constraints in S-98 section 13 are included in S-98 Annex A: Coverage and time series features are encoded in the HDF5 format (see S-100 Part 10c). S-100 provides for the following types of coverage and time series data: - Gridded data with different types of spatial grid coverages; - Data at a set of discrete fixed points; - Data at a set of moving platforms; - Time series data at a set of fixed points. For S-102 and S-104 datasets, ECDIS must support gridded data in DCF 2 format only. For S-111 datasets, ECDIS must support gridded data in DCF 2 or DCF 3 format only Gridded data will specify either continuous or discrete interpolation (no interpolation between points) between grid points. Data for discrete fixed points, moving platform and time series at fixed points is intrinsically discrete. |  |  |
| S-98 Annex A |  | 3 |  |  | Please take into account OSI inputs on GML format presented at TSM https://portal.iho.int/api/files/AAAAAAAAqDY/Issues%20on%20S-100%20and%20Product%20Specifications%20Raised%20by%20the%20Implementer%20(OSI)_Rev1/EN_S-100%20standards%20issues%20(rev%202).pdf | Please consider topics G1, G3, and G4 | Yes, in progress. |
|  |  | 12.5.3 |  |  | This shouldn’t require a NODATA03 pattern for the other data products. NODATA03 should only apply for ENC. |  |  |
|  |  |  |  |  | Can we use “select” instead of “individually select”? |  |  |