S-98

Published by the

International Hydrographic Organization

4b quai Antoine 1er

Principauté de Monaco

Tel: (377) 93.10.81.00

Fax: (377) 93.10.81.40

info@iho.int

www.iho.int

Annex C

Harmonised User Experience for ECDIS and INS

Edition 1.3.0 – May 2024

© Copyright International Hydrographic Organization 2024

This work is copyright. Apart from any use permitted in accordance with the Berne Convention for the Protection of Literary and Artistic Works (1886), and except in the circumstances described below, no part may be translated, reproduced by any process, adapted, communicated or commercially exploited without prior written permission from the International Hydrographic Organization (IHO). Copyright in some of the material in this publication may be owned by another party and permission for the translation and/or reproduction of that material must be obtained from the owner.

This document or partial material from this document may be translated, reproduced or distributed for general information, on no more than a cost recovery basis. Copies may not be sold or distributed for profit or gain without prior written agreement of the IHO Secretariat and any other copyright holders.

In the event that this document or partial material from this document is reproduced, translated or distributed under the terms described above, the following statements are to be included:

“Material from IHO publication [reference to extract: Title, Edition] is reproduced with the permission of the IHO Secretariat (Permission No ……./…) acting for the International Hydrographic Organization (IHO), which does not accept responsibility for the correctness of the material as reproduced: in case of doubt, the IHO’s authentic text shall prevail. The incorporation of material sourced from IHO shall not be construed as constituting an endorsement by IHO of this product.”

“This [document/publication] is a translation of IHO [document/publication] [name]. The IHO has not checked this translation and therefore takes no responsibility for its accuracy. In case of doubt the source version of [name] in [language] should be consulted.” The IHO Logo or other identifiers shall not be used in any derived product without prior written permission from the IHO Secretariat.

# CONTENTS

C-1	Introduction	1

C-2	References	1

C-2.1	Normative references	1

C-2.2	Informative references	2

C-3	Abbreviations and Notation	3

C-3.1	Abbreviations	3

C-3.2	Notation	3

C-4	System Concepts and Limitations	4

C-4.1	The concepts of ENC, ENDS and System Database	4

C-4.2	ECDIS concept, limitations, and challenges	4

C-4.2.1	Depth Related Functionality	6

C-4.3	Integrated Navigation System (INS) concept, limitations and challenges	6

C-5	User Interface Design	7

C-5.1	General principles	7

C-5.2	User interface elements	8

C-6	Data Layers	8

C-6.1	Data products and information layers	9

C-6.1.1	Basic products and layers	9

C-6.1.2	Other data products	9

C-6.1.3	Future data products (informative)	9

C-6.1.4	User selection of displayed data products	10

C-6.2	Mitigation of data overload	10

C-7	Portrayal Process, Catalogue Elements, and Alerting	10

C-7.1	Overview of the portrayal process	10

C-7.2	Elements of portrayal catalogues	11

C-7.2.1	Pixmaps	11

C-7.2.2	Colour coding scheme	11

C-7.2.3	Symbols	11

C-7.2.4	Area fills	11

C-7.2.5	Line styles	11

C-7.2.6	Text	12

C-7.2.7	Style sheets	12

C-7.2.8	Display planes	12

C-7.2.9	Display priorities	12

C-7.2.10	Viewing groups	12

C-7.2.11	Viewing group layers	12

C-7.2.12	Display modes	12

C-7.2.13	Foundation mode	13

C-7.2.14	Rules	13

C-7.2.15	Context	13

C-7.2.16	Validity times	13

C-7.3	Alerts	13

C-8	Visual Interoperability	13

C-8.1	Phased implementation of interoperability	13

C-8.2	Symbols and icons with interoperability ON	14

C-8.3	User interface functions for interoperability	14

C-8.3.1	General navigation functions	14

C-8.3.2	Control of chart display functions	14

C-8.3.3	Functions for control of chart functionality	15

C-8.3.4	Database functions	15

C-8.3.5	Function groups	16

C-8.3.6	Function accessibility	16

C-8.3.7	Condition-dependent enhancements to user interface	16

C-8.3.8	Status display	17

C-8.3.9	Settings	17

C-8.3.10	Display overload indications	18

C-9	Display Organisation and Operation	18

C-9.1	Display of non-S-100 information	18

C-9.1.1	Distinguishing between official data and additional data	18

C-9.1.2	Mariner’s information on the route monitoring display	19

C-9.1.3	Manufacturer’s information on the route monitoring display	20

C-9.1.4	Supplemental display items	20

C-9.1.5	Units	21

C-9.1.6	Legend	21

C-9.2	Priority of information	21

C-9.2.1	Priority layers	21

C-9.2.2	Radar priority	22

C-9.3	Display categories	22

C-9.3.1	IMO Categories	22

C-9.3.2	Selecting features for the route monitoring display	23

C-9.3.3	Selecting text for the route monitoring display	23

C-9.3.4	Linkages between ECDIS options	23

C-9.4	Drawing priorities	24

C-9.5	Viewing groups	25

C-9.5.1	Viewing group layers	25

C-9.5.2	Viewing groups for display base	26

C-9.5.3	Viewing groups for Standard Display	28

C-9.5.4	Viewing groups for other chart information	30

C-9.5.5	Viewing groups for mariner’s and manufacturer’s information	33

C-9.5.6	ECDIS viewing group layer implementation	33

C-9.6	Display modes	34

C-9.7	Displaying ECDIS updates	34

C-9.7.1	Automatic updates	35

C-9.7.2	Manual updates	35

C-9.8	Display functions	35

C-10	General Rules for Symbols and Text	37

C-10.1	Symbol Specifications	37

C-10.2	Guidelines for reproduction of lines, symbols and text	37

C-10.2.1	Introduction (informative)	37

C-10.2.2	Guidelines for size and resolution	38

C-10.2.3	Fonts	38

C-10.2.4	Zooming	38

C-10.3	Mariner's options in symbols and linestyles	39

C-10.4	Display orientation	39

C-10.5	Common text information attributes	39

C-10.5.1	Feature names	39

C-10.5.2	Generic attribute “information”	39

C-10.6	Queries	40

C-11	Text and Graphics	40

C-11.1	Text as part of the display	40

C-11.2	Viewing groups used for text	40

C-11.3	Viewing group layers used for text	43

C-11.4	Abbreviations	44

C-11.4.1	Text abbreviations	44

C-11.5	Additional information in text form	44

C-11.6	Graphics	46

C-12	Miscellaneous Display Elements	46

C-12.1	Elements related to data and display scales	47

C-12.1.1	ENC scale	47

C-12.1.2	Overscale	47

C-12.1.3	Scale boundary	47

C-12.1.4	Overscale area at scale boundary	47

C-12.1.5	Larger scale data available	48

C-12.2	Graphical indexes	48

C-12.2.1	Graphical indexes of ENCs	48

C-12.2.2	Graphical indexes of other S-100 products	49

C-12.3	Limits of data	49

C-12.3.1	Limit of HO S-101 data and data from non-HO sources	49

C-12.3.2	Limits of other S-100 product data	49

C-12.3.3	No data areas	50

C-12.4	Data quality indicators	50

C-12.5	Special ECDIS chart symbols to identify unsafe depths	50

C-12.5.1	Safety Contour	50

C-12.6	Other ECDIS symbols and their use	51

C-12.6.1	Mariners Caution Notes	51

C-12.6.2	Unknown feature	51

C-12.6.3	Change of horizontal (geodetic) datum	51

C-12.6.4	Manual chart correction	51

C-12.6.5	Manual corrections to non-ENC S-100 products	51

C-12.6.6	One-sided linestyle for boundaries	51

C-12.6.7	Special identifiers	51

C-12.7	Date-dependent features	51

C-12.7.1	Display of date-dependent features by mariner-selected date	52

C-12.7.2	Highlighting of date-dependent features	52

C-12.7.3	Indication of date adjustment	52

C-12.8	Scale-dependent features	52

C-12.9	IMO presentation elements	52

C-12.9.1	Scale bar and latitude scale	52

C-12.9.2	North arrow	53

C-12.9.3	Graticule	53

C-12.9.4	Display mode	53

C-12.9.5	Shallow water pattern	53

C-12.9.6	Black level adjustment symbol	54

C-12.9.7	Detection and notification of navigational hazards	54

C-12.9.8	Detection of areas for which special conditions exist	55

C-12.9.9	Visualisation of the safety contour	55

C-12.9.10	Detection of safety contour	55

C-12.9.11	User Selected Safety Contour and Water Level Adjustment.	56

C-12.9.12	Range indicator	56

C-12.9.13	Detection of route-based conditions	56

C-12.10	Data Producer specified display features	57

C-12.10.1	Supplementary information	57

C-12.10.2	Display in languages other than English	57

C-12.10.3	ECDIS Legend	57

C-12.11	Displaying manual and automatic updates and added information	59

C-12.11.1	Manual update	59

C-12.11.2	Identifying automatic chart corrections on demand	59

C-12.11.3	Updating non-official chart information	60

C-12.11.4	Other non-official data	61

C-12.12	Contours from datasets other than S-101	61

C-13	Coverages and Time Series	61

C-13.1	Discrete coverages	62

C-13.2	Continuous coverages	62

C-13.3	Contours from continuous coverages	63

C-13.4	Transparency	63

C-13.5	Thinning	63

C-13.5.1	Thinning algorithms (informative)	63

C-13.6	Temporal variation	67

C-13.7	Conflicting displays	67

C-14	Colours	68

C-14.1	General	68

C-14.2	Selection of colours (informative)	68

C-14.3	The colour palettes	68

C-14.4	Transparency	68

C-14.4.1	Use of transparency (informative)	68

C-14.5	Colour assignment for navigation systems	69

C-14.5.1	Introduction to colours (informative)	69

C-14.5.2	Colour assignments	69

C-14.6	Colour tokens, profiles and palettes	70

C-15	Cursor Pick Reports and Displays in Interface Panels	75

C-15.1	Cursor pick rules	75

C-15.2	Pick Report descriptions	75

C-15.2.1	User defined cursor pick parameters	76

C-15.2.2	Sorting order of results	76

C-15.2.3	Hover-over function	76

C-15.2.4	Unknown attributes	76

C-15.3	Cursor queries on coverage data	76

C-15.4	Tidal stream data panels	77

C-15.5	Schedules, contact Information, and vessel characteristics	78

C-15.6	Special Pick Report formats	78

C-16	Alerts and Indications	79

C-17	Use of Context Parameters	79

C-17.1	Context parameters for ECDIS functionality	80

C-18	Dual-fuel systems	80

C-18.1	Display of data available in both new and legacy formats	80

C-18.2	Display of additional information layers	80

C-18.3	Concurrent applicability of S-52 and S-57	81

C-19	Type approval considerations	81

C-20	Specifications for the display screen	82

C-20.1	Physical display requirements	82

C-20.2	Colour reproduction	82

C-20.3	Display requirements for colours	82

C-20.3.1	General	83

C-20.3.2	Bright sunlight or night viewing - use of filters, etc	83

C-20.3.3	Display calibration and verification	83

C-20.4	Colour display capability	85

C-20.4.1	Colour conversion tolerances and tests	85

C-20.5	Colour differentiation test diagram	87

C-20.6	ECDIS Chart 1	88

C-20.7	Use of ECDIS Chart 1 and Colour Test Diagram	89

C-20.7.1	Specification for ECDIS Chart 1 and the Colour Test Diagram	89

C-20.7.2	Displaying the Colour Test Diagram	90

C-20.7.3	Grey Scale	91

C-21	Dataset Management	92

C-21.1	Multiple product versions and portrayal	92

C-21.2	Dataset overlaps and gaps	93

C-21.2.1	Overlaps and gaps in ENC coverage	93

C-21.2.2	Overlaps and gaps in other data products	93

C-21.3	Guidance on updating S-100 datasets on ECDIS	93

C-21.3.1	Introduction	93

C-21.3.2	General requirements	94

C-21.3.3	Automatic Update	95

C-21.3.4	Manual Update	96

C-21.3.5	Support file updates	96

C-21.4	New editions, re-issues, and updates of datasets	96

C-22	Support File Formats	96

Appendix C-1 - Relationship to S-52	99

C-1	Relationship to S-52 provisions	99

Appendix C-2 - S-100 Data Import Error Codes and Explanations	101

Appendix C-3 - ECDIS Update Status Reports	107

Appendix C-4 - User Selected Safety Contour and Water Level Adjustment	113DATA_PRODUCER

C-1	User Selectable Safety Contour	113

C-1.1	Introduction	113

C-1.2	Data Constraints	114

C-1.3	Implementation	114

C-2	Water Level Adjustment	115

C-2.1	General	115

C-2.2	Constraints on input data	115

C-2.3	User inputs	116

C-2.4	Implementation - general	116

C-2.5	Implementation of WLA Option 1 – WLA for a single datetime instant	117

C-2.6	Implementation of WLA Option 2 – WLA for a datetime range specified by the user as a time period	118

C-2.7	Implementation of WLA Option 3 – linking of WLA to a defined route with planned waypoints and times	119

C-3	Treatment of depth and water level related S-101 features	123

C-3.1	Substitution and adjustment of depth values	123

C-3.1.1	General	123

C-3.1.2	Areas covered by S-102 only or by both S-102 and S-104	123

C-4.3.1.3 Areas covered by only S-104	125

C-3.2	Adjustment of heights and vertical clearance values	125

C-3.3	Alerts and indication details	125

C-3.4	Legend details	126

Document History

Changes to this Specification are coordinated by the IHO S-100 Working Group. New editions will be made available via the IHO website. Maintenance of the Specification shall conform to IHO Resolution 2/2007 (as amended).

| Version Number | Date | Approved By | Purpose |
| --- | --- | --- | --- |
| 0.0.1 | 28 Aug 2020 | RM | First draft. Reviewed by EM. |
| 0.0.2 | 01 Nov 2021 | J.Powell | Numerous revisions to take into account the S-98 Correspondence Group adjudication work. |
| 1.0.0 | May 2022 | S-100WG | Submission to HSSC14 for approval. |
| 1.0.0 | May 2022 | HSSC | Initial published version for evaluation and testing. |
| 1.1.0 | August 2023 |  | Added MSC.530(106) in place of MSC.232(82) and updated text accordingly (“SENC” to “System Database”- various clauses); updated tidal stream panel presentation (C-15.4); fixed heading 1 and heading 2 styles; added new clause listing allowed support file formats for ECDIS and elaborated rules for additional information in text (C-11.5); struck alternate labels in Tables C-2 and C-3; |
| 1.2.0 | February 2024 |  | Collection of issues identified by subgroup and external stakeholder. |
| 1.3.0 | May 2024 |  | Incorporating stakeholder inputs, GitHub discussions and TSM 2024 outputs. |

# Introduction

This Annex contains guidance for the implementation of harmonised portrayal and other user interaction functionalities for S-101 Electronic Navigational Chart (ENC) and other S-100 based data products in an Electronic Chart Display and Information System (ECDIS). It describes how S-100 products are to be used and displayed simultaneously on the navigation screen. It does not address the portrayal processes or architectures, which are addressed in S-100 Part 9. It is based on the general principles described in S-100 Part 16A, and the requirements for ECDIS specified in the relevant International Maritime Organization (IMO), International Hydrographic Organization (IHO), and International Electrotechnical Commission (IEC) standards.

This Annex is the successor to the IHO S-52 Standard for chart content and display in ECDIS. It contains material from S-52 that has been updated for S-100, but does not include symbol specifications (the S-52 “presentation library”) or portrayal rules because symbols and portrayal rules are now defined in the IHO GI Registry and portrayal catalogues for individual product specifications. This document focuses on the principles for portraying S-101 ENC and S-100 based data on ECDIS. The principles described herein are intended to be compatible with the corresponding requirements for ECDIS portrayal of S-57 ENCs.

This Annex does not revoke nor does it supersede S-52 in its applicability to the use of S-57 information on ECDIS. It assumes ECDIS will support both S-52/S-57 and S-101 ENCs for the foreseeable future. Such systems should continue to follow the S-52/S-57 standards for S-57 information.

ECDIS presentation and user interactions are determined by the following standards:

- IMO standards control presentation, performance, and user experience. They include standards and guidelines for display and user interaction, including alerts.

- IHO standards provide the framework for data content, primarily in S-100. S-100 also provides an abstract specification for visual interoperability; for ECDIS, details about interoperability are specified in S-98.

- IEC standards describe methods and required results for equipment and system testing.

- Data product specifications describe the content, data formats, symbols, portrayal rules, packaging, and delivery, of individual data products. For ECDIS, the basic data product is S-101 (Electronic Navigational Charts). Other products describe additional information relevant to navigation, for example, bathymetry, currents, water levels, regulated areas, services and weather.

The standards that are current at the time of writing of this Annex are listed in the References section. More detailed information about the various standards is provided in S-100 Part 16A.

Application developers should obtain an up-to-date set of applicable standards and specifications from the relevant organisations. Developers must conform to the mandatory requirements of the particular standards which apply to an application or system. In case of a conflict between this Annex and a mandatory requirement in an applicable standard, the requirement in the standard supersedes the guidance in this Annex.

The specifications in this Annex also apply to navigation displays in an Integrated Navigation System (INS) which fulfil the role of an ECDIS.

Users of this document should refer to IEC publications when dealing with implementation testing.

# References

## Normative references

A.1021(26)	Code on Alerts and Indicators (2009), IMO Resolution A.1021(26), 2009.

IEC 60945	Maritime navigation and radiocommunication equipment and systems – General requirements – Methods of testing and required test results, International Electrotechnical Commission (IEC), Fourth Edition, 2002.

IEC 61174	Maritime navigation and radiocommunication equipment and systems – Electronic chart display and information system (ECDIS) – Operational and performance requirements, methods of testing and required test results, International Electrotechnical Commission (IEC), Edition 5.0, 20XX.

IEC 62288	Maritime navigation and radiocommunication equipment and systems – Presentation of navigation-related information on shipborne navigational displays – General requirements, methods of testing and required test results, International Electrotechnical Commission (IEC), Edition 3.0, 2021.

MSC.191(79)	Amendments to the Performance Standards for the Presentation of Navigation-Related Information on Shipborne Navigational Displays, IMO Resolution MSC.191(79), 2004. As amended by MSC 466(101).

MSC.252(83)	Performance Standards for Integrated Navigation Systems (INS), IMO Resolution 252(83), 2007.

MSC.302(87)	Adoption of Performance Standards for Bridge Alert Management, IMO Resolution 302(87), 2010.

MSC.466(101)	Amendments to the Performance Standards for the Presentation of Navigation-Related Information on Shipborne Navigational Displays, Resolution MSC.191(79), 2019.

MSC.530(106)	Performance Standards for Electronic Chart Display and  Information Systems (ECDIS), Resolution MSC.530(106), 2022.

MSC.1593	Interim Guidelines for the Harmonized Display of Navigation Information Received via Communication Equipment, IMO MSC.1/Circ.1593, 2018.

MSC.1/CIRC.1609	Guidelines for the Standardization of User Interface Design for Navigation Equipment, IMO MSC.1/Circ 1609, 2019.

S-97	IHO Guidelines for Creating S-100 Product Specifications, IHO Publication S-97 1.1.0 (June 2020).

S-100	Universal Hydrographic Data Model, IHO Publication S-100, Edition 5.2.0 ( 2024).

SN.1/Circ.243/2	Guidelines for the Presentation of Navigational-Related Symbols, Terms and Abbreviations, IMO SN.1/Circ.243/Rev.2, 2019.

## Informative references

ISO 19117	Geographic Information – Portrayal, ISO Standard 19117 Edition 2, 2012.

S-4	Regulations of the IHO for International (INT) Charts and Chart Specifications of the IHO, IHO Publication S-4, Edition 4.8.0, October 2018. (Parts B and C in particular.)

S-52	Specifications for Chart Content and Display Aspects of ECDIS, IHO Publication S-52, Edition 6.1.1, June 2015.

SVG-Tiny	Scalable Vector Graphics (SVG) Tiny 1.2 Specification, W3C Recommendation 22 December 2008, <http://www.w3.org/TR/2008/REC-SVGTiny12-20081222>.

# Abbreviations and Notation

## Abbreviations

AIS	Automatic Identification System

ARPA	Automatic Radar Plotting Aid

CRS 	Coordinate Reference System

CSS	Cascading Style Sheets

EBL	Electronic Bearing Line

ECDIS	Electronic Chart Display and Information System

ECS	Electronic Chart System

ENC	Electronic Navigational Chart

ENDS	Electronic Navigational Data Service

ENP	Electronic Navigational Publication

GML	Geographic Markup Language

HO	Hydrographic Office

INS	Integrated Navigation System

IEC	International Electrotechnical Commission

IHO	International Hydrographic Organization

IMO	International Maritime Organization

SD	System Database

SENC	System Electronic Navigational Chart (obsolete)

SVG	Scalable Vector Graphics

VRM	Variable Range Marker

XML	EXtensible Markup Language

XSLT	EXtensible Stylesheet Language Transformations

## Notation

The following notation is used throughout the document to describe various portrayal aspects.

(Name of feature or information type)

Features and information types are referenced by their names in camel-case or space-separated names in initial capitals (“LandArea” or “Land Area”).

(Name of attribute)

Attributes are referenced by their names in camel-case with initial letters in lower case, or space-separated names in all lower case (“visuallyConspicuous” or “visually conspicuous”).

(Portrayal register item)

Items from the IHO portrayal registry are referenced by their registered name and item type (“line style SCLBDY51”). The portrayal registry is a component of the IHO Geospatial Information Registry (https://registry.iho.int).

## Normative vs Informative within this document.

This document uses the word “must” to describe mandatory functionality for S-100 ECDIS. In many cases within this document S-100 ECDIS behaviour is described which is mandatory, but which is illustrated using examples which are contained in operational/run-time S-100 elements such as feature or portrayal catalogues.

In many cases this document describes functionality which is mandatory for the S-100 ECDIS, but its implementation is fulfilled by the implementation of S-100 itself. For example there is a requirement to implement standard display layers Base, Standard and Other. This document does not define those layers, they are implemented by the S-101 portrayal catalogue, thus the requirement is fulfilled by implementation of S-100 Part 9 and the ability to read and correctly portray ENC data. Where this is the case functionality will be described using examples drawn from individual product speciications but the S-100 product specification catalogues remain normative. Where this is the case the product specification feature and portrayal catalogues remain the normative reference and this document will describe the required functionality as normative.

Where these differences exist, they are noted in section heading footnotes. Where this document refers to mandatory functionality defined elsewhere it

# Background - System Concepts and Limitations

## The concepts of ENC, ENDS and System Database

The Electronic Navigational Data Service and System Database concepts include multiple products for use in navigation systems. They were introduced in IMO MSC 530(106) and update the previous concept of SENC defined in IMO MSC.232(82).

IMO MSC.530(106) defines Electronic Navigational Chart (ENC), Electronic Navigational Data Service (ENDS), and System Database as follows:

3.2 Electronic Navigational Chart (ENC) means the database, standardized as to content, structure and format, issued for use with ECDIS by or on the authority of a Government, authorized hydrographic office or other relevant government institution, and conforming to IHO standards. The ENC contains all the nautical chart information necessary for safe navigation.

3.3 Electronic Navigational Data Service (ENDS) means a special-purpose database, compiled from nautical chart and nautical publication data, standardized as to content, structure and format, issued for use with ECDIS by or on the authority of a Government, authorized hydrographic office or other relevant government institution, and conforming to IHO standards; and, which is designed to meet the requirement of marine navigation and the nautical chart and nautical publications carriage requirements in SOLAS regulations V/19 and V/27. The navigational base layer of ENDS is the electronic navigational chart.

3.4 System Database means a database, in the manufacturer’s internal ECDIS format, resulting from the lossless transformation of the ENDS contents and its updates. It is this database that is accessed by ECDIS for the display generation and other navigational functions, and is equivalent to [an] up-to-date ENDS.

For S-100 purposes, the System Database must be compiled from multiple S-100-based products.

## ECDIS concept, limitations, and challenges

IMO MSC 530(106) also defines Electronic Chart Display and Information System (ECDIS):

3.1 Electronic Chart Display and Information System (ECDIS) means a navigation information system which with adequate backup arrangements can be accepted as complying  with the up-to-date nautical chart and nautical publications required by SOLAS regulations V/19 and V/27, by displaying selected information from a system database with positional information from navigation sensors to assist the mariner in route planning and route monitoring and, if required, display additional navigation-related information.

The following list explains more of the ECDIS concept in detail and provides information about certain limitations.

- ENC is an integral part of ECDIS and therefore is defined as the base layer for the portrayal harmonisation framework. Additional layers are classified as two main types, additional information to that of the ENC or enhanced information to that of the ENC. Additional information is information that is not contained in the ENC, while enhanced information are layers that contain improved, detailed or higher resolution information than the ENC.

- ECDIS, used together with official data, [is] accepted as complying with the up-to-date nautical chart and nautical publications carriage requirement required by regulation V/19 and V/17 of the 1974 SOLAS Convention amended in 2009. Electronic chart systems not meeting these ECDIS specifications of IHO and IMO, or ECDIS using non-official data, are known as ECS (Electronic Chart Systems).

- Chart information may be used in conjunction with a radar overlay on ECDIS. Integration of tracked radar targets provided for collision avoidance radar (ARPA) or targets reported by AIS (Automatic Identification System) can be integrated into the ECDIS display, as well as other navigational information which may be added to the ECDIS display. With the advent of S-100, additional types of data may also be occasionally added to the ECDIS display, especially high-density bathymetry, surface current and water levels data or predictions, maritime safety information (MSI), and under keel clearance area data.

- Colours and symbols defined in the S-101 portrayal catalogue are conceptually based on the symbology of conventional paper charts. However, due to the special conditions of the ECDIS chart display as a computer-generated image, the ECDIS presentation of ENC data may differ from the appearance of a conventional paper chart, especially when simplified portrayal mode is used. There may be considerable differences in symbology in shape, colour and size, and in the placement of text in particular.

- ECDIS combines chart and real-time navigational positioning information. Modern navigation systems (i.e. GNSS) may offer a more accurate positioning than was available to position some of the surveys from which the digital chart data ENC was derived. Further, other products may not be encoded to the same accuracy or precision as ENC data or ship’s positioning, therefore there is a potentially greater reliance on metadata covering data quality and precision in a digital navigation context

- The display categories specified in the IMO Performance Standards and the IHO priorities of the various types of chart information (alerts, updates, mariner's and non-HO chart data, etc.) are applied to features by the display plane and drawing order assignments in individual portrayal catalogues. The drawing order may be modified if interoperability is activated.

- For definition of ‘single operator action’ and ‘simple operator action’, see IMO MSC.252(83).

- In the initial period of S-100 roll-out, S-100 compatible ECDIS must be “dual-fuel” in that they contain both S-57 and S-101 ENCs. It is possible that there will be situations where the navigation screen must display both S-57 and S-101 ENCs. The user experience aspects of such situations are discussed in Clause C-18.

The versatility of ECDIS poses several challenges for ECDIS display design. These challenges are discussed below.

The diversity of information displayed on an ECDIS may include:

- physical chart information, for example, coastline, depth contours, buoys;

- traffic routeing; specified areas; cautions; etc.;

- supplementary HO information from light list, etc.;

- mariner's notes; additional local chart information; manufacturer's information;

- chartwork such as planned route; electronic bearing lines and range rings; etc.;

- own ship's position and velocity vector; ship's head and rate of turn; past track;

- fix accuracy, or position check from secondary positioning system;

- possibly, ship handling options, based on ship’s characteristics, alphanumeric navigation information (ship’s latitude, longitude, heading, course, etc.);

- information from radar and other sensors;

- information from AIS;

- navigational indications and alerts generated by ECDIS;

- possibly, telemetered information from shore authorities (traffic, real-time tides etc.);

- possibly, ice information;

- reminders (for example, time to contact pilot station);

- possibly, messages from other displays.

The flexibility of portrayal may include:

- displaying/removing various types of chart and non-chart information,

- selecting standard chart display or a thinned out display, and either paper chart symbols or simplified symbols,

- using cursor interrogation for further detail,

- overlaying/removing radar video or radar target information (in order to: confirm ship’s positioning; aid radar interpretation; show the entire navigation situation on one screen),

- overlaying/removing various other sensor information, or information telemetered from shore,

- changing the scale or orientation of the display,

- selecting true motion or relative motion,

- changing screen layout with windowed displays, text information in the margins, etc.,

- possibility of pull-down menus and other operator interaction devices being alongside the operational navigation display and so interacting with it,

- giving navigation and chart warnings such as “too close approach to safety contour”; “about to enter prohibited area”; “overscale display”; “more detailed (larger scale) data available”; etc.,

- possibly, a diagrammatic representation of a computer evaluation of grounding danger,

- possibly, a diagrammatic representation of the immediate vicinity of the ship to aid in close quarters manoeuvring,

### Depth Related Functionality

It must be possible to adjust depth information by water level height. When water level adjustment is provided:

- The system must default to no water level adjustment.

- The mariner must be able to select one for the following methods of depth adjustment:

- Current date and time;

- A mariner specified date and time;

- Where the ECDIS supports schedules, the depth at the predicted date and time of transit in each area along a route (Comment: This is the feature that will reduce the mariner workload. Additionally, manually setting the time is prone to errors).

- When water level adjustment is applied (full details provided in Appendix C-4):

- The safety contour, depth zone shades, safety depth and indication of isolated dangers must use the adjusted depth;

- The pick report must indicate both adjusted and unadjusted depth;

- The applied water level adjustment method must be provided in the legend;

- Other details of the water level adjustment must be readily available, such as the data source, temporal extent, and applicable areas;

- It must be possible to remove all water level adjustment via simple operator action;

- There must be a permanent indication “Water level adjustment”.

- ECDIS voyage recording must include:

- The state of water level adjustment (method applied);

- All other information necessary to reconstruct depths as presented to the mariner.

## Integrated Navigation System (INS) concept, limitations and challenges

The concept of an Integrated Navigation System (INS) is outlined in the IMO Performance Standards MSC 252(83). INS workstations have multifunctional displays providing at least route monitoring and collision avoidance functions, and may provide manual or automatic navigation control functions. In addition to these functions, an INS generally also provide route planning, navigation control data and status, and alert management functions. INS integrate sources, data, and displays into one navigation system. An INS may consist of multiple task stations.

Since the concept of ECDIS is included in the INS concept, the principle described in clause C-6.1.1 of having ENC as the base layer applies – but in the context of an INS it applies to components playing the role of an ECDIS. Components playing other roles will generally use ENC or similar data, but this may depend on function and task.

The considerations related to portrayal that are in addition to the requirements already embedded in the INS standards are outlined below:

- An INS may substitute under some carriage requirements for certain navigational equipment. The INS is required to fulfil the requirements for the systems it replaces. For example, an INS component used for the tasks of route monitoring and route planning must meet the requirements for an ECDIS, which are described IMO MSC 530(106). This means that the portrayal and user interaction considerations for an ECDIS described in C-4.2 apply to the workstation playing the role of an ECDIS.

- All tasks of an INS must use the same electronic chart data and other S-100 navigational databases such as routes, maps, tide information. If ENCs are available, they must be used as a common data source for INS.

IMO Performance Standards (MSC.252(83)) state that for each task it is used for, the INS should fulfil the relevant modules of the performance standards for standalone equipment for the task. An INS display being used for route monitoring will be subject to the ECDIS performance standards, and therefore the relevant INS display will have largely the same limitations and challenges described in clause C-4.2. The potential availability of other workstations in an INS allows manufacturers somewhat more flexibility in designing solutions, but this is limited by human factors constraints related to compatibility and divided attention.

Given the complexity of the issues and the necessity for compatibility, this document does not distinguish between INS and ECDIS displays for the purpose of harmonised portrayal. If the data products are shown on the same screen, the same rules apply for INS and ECDIS displays, and the treatment should be the same for INS as for ECDIS. If the data products are on different INS screens, the treatment should be compatible in the sense of the guidelines in IMO Circ.1609.

# User Interface Design

## General principles

The guidelines in IMO  MSC.1/CIRC.1609 (Guidelines for the Standardization of User Interface Design for Navigational Equipment) apply to the design of user interfaces for navigation systems such as ECDIS and INS. The general principles are described in  MSC.1/CIRC.1609 and are summarised in S-100 Part 16A.

## User interface elements

IMO Circulars SN.243/Rev.2 and  MSC.1/CIRC.1609 define symbols for navigation-related features and functions. For ECDIS and INS, this entails the following requirements and recommendations:

- For feature and information data objects in S-100 datasets, the symbols defined in the IHO GI registry and specified in the respective data product specifications must be used, subject only to such modifications as may be specified by an approved interoperability catalogue conforming to S-98 (ECDIS Interoperability).

- Deleted

- Deleted

- For functions and symbols to use when S-98 interoperability is activated on the display, see Sections C-8.2 and C-8.3 of this Annex.

- Where a standard symbol, icon or term is not available, manufacturers may define a new symbol, icon or term, but such symbols, icons and terms should not conflict with those defined by IMO or IHO. New abbreviations should be defined only for new symbols, icons, or terms, and should not conflict with existing abbreviations. Manufacturers are encouraged to discuss the need for new symbols, icons, terms or abbreviations with IMO and IHO.

- Logical grouping of information should be consistent with the groups specified in Appendix 3 of  MSC.1/CIRC.1609. Note that Appendix 3 does not specify where the groups should appear on the screen or the order of information elements within groups, nor is the list exhaustive.

- While many symbol and icon definitions in SN.243/Rev.2 and  MSC.1/CIRC.1609 do not specify colours for the standard symbols and icons, the general principles of user interface design and likely environmental and lighting conditions (e.g., use of red light on the bridge, day/night/dusk lighting, prominence vs. distraction tradeoffs) should be kept in mind when determining colours for these symbols and icons. The colour assignment table in Part 16A may be consulted to assess possible colours, but developers should note that assignment of a colour in that table does not necessarily preclude its use for navigation-related symbols or functions.

- Deleted

- Deleted

# Data Layers

Since there is no restriction on which data layers can be displayed other than IMO Performance Standards’ requirement that the data does not degrade the displayed System Database information and should be clearly distinguishable from the System Database information, navigation systems mjust be able to display any published, operational S-100 data product that satisfies the relevant technical standards.

## Data products and information layers

### Basic product specifications and layers

Navigation displays such as the ECDIS main screen must be able to display at least the following data layers:

- S-101 ENC data as the base layer, alongside S-57 ENC data

- S-102 Bathymetric Surface;

- S-104 Water Level information for Surface Navigation;

- S-111 Surface Currents;

- S-124 Navigational Warnings;

- S-129 Underkeel Clearance;

- S-421 Route Plan.

- S-128 Nautical Product Catalogue;

- Additional mariners information

### Other data products

In addition, navigation displays such as the ECDIS main screen must allow the mariner to toggle the display of the following data products (when operational product specification versions are available from IHO):

- S-122 Marine Protected Areas;

- S-123 Marine Radio Services;

- S-127 Marine Traffic Management;

- S-412 Weather and Wave Hazards;

- S-411 Sea Ice

### Future data products

Other data products are in early stages of development, and are unlikely to need display capabilities for the next several years. These include:

- S-125 Marine Navigational Services;

- S-126 Marine Physical Environment;

- S-128 Nautical Product Catalogue;

- S-413 Weather and Wave Conditions;

- S-414 Weather and Wave Observations.

### User selection of displayed data products

The user interface must provide a way for the mariner to select data layers from non-ENC S-100 data products to allow them be added to or removed from the navigation display window.

## Mitigation of data overload

Each additional data layer multiplies clutter and the potential for hiding critical information. General user interface principles as well as IMO guidance (MSC.191(79), MSC.1/Circ1609) recommend avoiding cluttering screens with too much information. To reduce the likelihood of screen clutter when multiple data products are loaded on the screen, the ECDIS should limit the number of simultaneous layers to those necessary to perform a specific task.

Detection of and response to possible data overload should be guided by the role a particular system is playing at the moment (e.g., a display being used for planning may be able to treat potential overloading in a laxer manner than navigation displays being used for route monitoring and collision avoidance). See clause C-8.3.10 for user interface recommendations relating to possible data overload.

# Portrayal Process, Catalogue Elements, and Alerting

This section provides a brief, informative summary of the S-100 portrayal process, the main elements of Portrayal Catalogues, and the alert model. More details about the process and definitions of Portrayal Catalogue elements and the alerting model are available in S-100 Parts 9 and 9a.

Later sections of the present document describe how the portrayal elements should be used in an ECDIS implementation.

## Overview of the portrayal process

The system has feature data within its internal database that needs to be portrayed. The System Portrayal Engine transforms the feature data into drawing instructions according to the portrayal rules defined in the Portrayal Catalogue. The drawing instructions are intermediate data used by the rendering engine to produce the portrayal output. Drawing instructions include such things as references to symbol definitions, priority and filtering information. The symbol definitions contain the details of all graphical elements used for the portrayal. The drawing instructions are processed by the rendering engine to produce the final display according to the output device.

S-100 describes two different portrayal mechanisms, one based on XSLT templates and the other on a scripting architecture based on Lua. The basic portrayal process is the same for both, and is described in S-100 Part 9; variations to the process and input/output that are required for Lua Portrayal Catalogues are described in Part 9a. Both XSLT and Lua portrayal are required for S-100 ECDIS and must be implemented.

When interoperability is activated and there is an interoperable product loaded to the display, either feature data or drawing instructions (depending on the implementation architecture) are further filtered and/or have their priorities adjusted as specified in the interoperability catalogue. Interoperability processing is described in S-100 Part 16 and S-98.

In the absence of interoperability, portrayal is as described in S-100 Part 9. When interoperability is activated the portrayal is modified as specified by the Interoperability Catalogue. Interoperability processing is described in S-100 Part 16 and S-98.

## Elements of portrayal catalogues

Much of the content of this section is included for information as it is described definitively in S-100 Part 9 and Part 9a already.

### Pixmaps

Pixmaps can be used for defining pattern fills for areas that are not sufficiently described to be symbolised, or for which no symbol exists in the IHO Geospatial Information (GI) Registry.

### Colour coding scheme

The Portrayal Catalogue uses a colour scheme, which classifies colours by their usage. Each colour usage is represented by a five-letter colour token. Each colour token corresponds to a colour definition given in CIE and/or sRGB coordinates in one of a set of colours; each set is intended for a different bridge lighting condition. Each such set of colours is referred to as a “palette” and is generally identified by a descriptive name, such as “Day-Bright”, “Dusk” and “Night”. A particular colour token is often assigned different CIE/sRGB coordinates in different palettes (especially day/night/dusk palettes).

A “colour profile”, as the term is used in S-100, is a collection of different palettes within a single XML file. An ECDIS must, at a minimum, have palettes for day, night, and dusk conditions available on the system (MSC 530(106) / A10.4.

Symbols, fill styles and line styles refer to the colour tables by using the standardized colour tokens as part of the symbol definition.

### Symbols

The Portrayal Catalogue provides a set of symbols, which are generally based on the traditional paper chart symbols. The symbols for point features are generally based on the traditional paper chart symbols and, in addition a set of more compact, but more visible, 'simplified' buoy and beacon symbols are provided for use under difficult viewing conditions.

### Area fills

The Portrayal Catalogue offers various ways to fill areas. They can be filled with an opaque colour; with a colour shown with some transparency; or with a pattern of symbols (fill pattern) or with a centred symbol. Fill patterns and centred symbols are introduced as a solution for the symbolization of areas in special situations. A fill pattern showing arrows does not have a certain position on the chart like the paper chart arrow symbol. It shows up as long as any part of the traffic separation lane can be seen on the screen. A centred symbol moves to the centre of the part of the area that remains in the display window. Fill patterns or centred area symbols may be used to symbolize the case when the entire display window lies within an area.

The Portrayal Catalogue provides two options for area boundaries, referred to as “plain” and “symbolised”.

Centred symbols must be used with symbolized boundaries to symbolize the case when the entire display window lies within an area.

Area boundaries should not be visible on the edge of the display window when the display window is enclosed by an area. However, if the boundary is actually on the edge of the display window, it should be visible.

In the context of S-100 area fills effect surface geometry portrayal.

### Line styles

The Portrayal Catalogue uses two types of line styles: simple line styles and complex line styles. Simple line styles are solid, dashed or dotted lines with varying colour and thickness (width or stroke width). Complex (or “composite”) line styles are composed of repeating line patterns.

Simple line styles are generally described by continuity, width, and colour. The full specification of a simple line style may also include other elements such as dash interval, cap and join types (see S-100 clauses 9-12.4 and 9a-11.2.2.3), defaults for which may be set in the Portrayal Catalogue. Complex line styles consist of additional elements, described in S-100 9-12.4 and 9a-11.2.2.3.

Complex linestyles may be one-sided (symbols, text, etc., which are part of the line extend to only one side of the line) or two-sided (symbols, text, etc. extend on both sides of the line).

In the context of S-100 line styles effect curve geometry portrayal.

[

]

### Text

S-101 and other Product Specifications utilize a cartographic feature called TextPlacement that is used in association with a feature to optimise placement of labels (specifically, either feature name or light characteristic description string).

There are three types of text instructions possible in portrayal catalogues:

- Text relative to a point;

- Text that will be drawn along a line; and

- Text placed relative to an area uses a local coordinate reference system. Note that this can cause the text to be drawn at multiple locations.

However, explicit cartographic placement along curves or relative to area/local CRS cannot be explicitly encoded in S-101 datasets, though it can be encoded in portrayal rules in a Portrayal Catalogue (S-100 Part 9a, clause 9a-11.2).

### Style sheets

Cascading Style Sheets (CSS) files are used to provide different sets of stroke and fill style instructions to be applied to symbols. This mechanism allows changing colours and line weights used in the symbols by swapping CSS files according to the desired colour scheme.

In principle, any style attribute can be set in a CSS file, but the CSS files in IHO Portrayal Catalogues will affect only a limited set of style attributes, namely, stroke and fill colours, line cap and join styles. Note also that the style precedence rules will affect whether the style attribute in a CSS file will actually be applied to a displayed element.

### Display planes

Display planes are used to split the output of the portrayal functions into mutually exclusive lists. An example of this is the separation of chart information drawn under a radar image and chart information drawn over a radar image.

### Display priorities

Display priorities control the order in which the output of the portrayal functions is processed by the rendering engine. Priorities with smaller numerical values will be processed first.

The display priority is a value between '00' and '99' (proposed scale - see Table C-9), where '99' identifies the highest priority. The display priority applies irrespective of whether a feature is a point, curve or surface. If the display priority is equal among features, curve features have to be drawn on top of surface features whereas point features have to be drawn on top of both. If the display priority is still equal among features of the same type of geometry (curve, surface or point) the given sequence in the data structure of the System Database, or some other neutral criterion, should be used for a decision as to which feature is drawn on top. Text must be drawn last (except for own ship etc.), in priority 80-89.

Display priorities are also called “drawing priorities” in S-100 Parts 9 and 9a.

### Viewing groups

The viewing group controls the content of the display.  It provides an on/off switch in the Portrayal Catalogue for any drawing instruction assigned to the corresponding viewing group.

### Viewing group layers

A viewing group layer defines a collection of viewing groups whose visibility can be simultaneously toggled on/off by an application.

### Display modes

A display mode defines a collection of viewing group layers which can be simultaneously toggled on/off by an application.

### Foundation mode

The foundation mode defines a collection of viewing groups that forms the foundation of the portrayal and cannot be removed from the display. The content of these viewing groups should comply with Display Base rules in IMO MSC.530(106) Appendix 2.

### Rules

A portrayal rule is a set of machine processable statements that describes the transformation of feature data into drawing instructions.

### Context

The context is a set of parameters passed into portrayal processing at the top level. Context parameters are accessible only in processing rules in the same portrayal catalogue, but can be accessed by any of the rules in that portrayal catalogue. These parameters can be used to provide contextual information to the execution of transformation rules in portrayal processing. Examples are settings of depth values for the safety depth, shallow and deep contours.

### Validity times

Drawing instructions can be designated as being valid only during a specified interval, which may be open-ended at either beginning or end, or a single time point (with beginning and end the same). Intervals may be specified in terms of date, time (of day), or date-time endpoints.

The start and end instants are defined by their date/time component of the smallest granularity. For example, if the end instant is specified a date without a time of day, and the interval is “right-closed”, the end instant is midnight at the end of the specified day (240000 in ISO 8601 terms). This is consistent with the S-100 treatment of dateStart and dateEnd attributes (see S-100 Part 3 clause 3-8-3 (Interpretation of models of time intervals and period), but note that the open/closed nature of the interval affects the interpretation for Part 9 time intervals — if the same interval is specified as “right-open” the start time point is midnight at the beginning of the specified day (000000 in ISO 8601 terms).

## Alerts

The Alerts Catalogue is a component of the Portrayal Catalogue; it describes each alert event along with its associated message(s) and highlighting rules.

The alerting model allows product developers to associate alerts with features (optionally satisfying specified conditions on attribute values) by encoding rules in the Portrayal Catalogue.

Alerts are associated with drawing instructions output by the portrayal, and are triggered when the vessel route (either actual track, during route monitoring, or planned, during route planning) intersects the geometry (which may be restricted or augmented) of a feature. The events are alerts, warnings, cautions, or indications as described in IMO MSC.252(83).

# Visual Interoperability

Ensuring visual interoperability in the presence of simultaneously displayed layers is an important aspect of user interfaces for navigation screens. IHO Publication S-98 (Data Product Interoperability in S-100 Navigation Systems) specifies the structure of the interoperability catalogue for ECDIS and INS.

While the Interoperability Catalogue provided by IHO will provide a normative Interoperability Catalogue for ECDIS, S-98 permits extensions, substitutes and customizations provided the minimum functions provided by the IHO Catalogue are maintained and interoperability is not degraded (S-98 clauses 12.1.1–12.1.3).

The preference is for interoperability to be permanently on at the highest level supported by the equipment. But should this solution not be acceptable, then the following applies:

- The ECDIS should default to Interoperability = On and Level = the highest interoperability level supported by the ECDIS

- There should be a permanent indication when default interoperability is not selected.

- The existing single operator action to select the ECDIS Standard Display should select the default interoperability settings.

- Access to the interoperability selections should be consistent with access to the existing S-57 viewing groups selections – i.e. no need for single or simple operator actions. Consequentially, there should no new requirements for single or simple operator actions for interoperability.

Equally, if it is not agreed that interoperability is permanently on at the highest level supported by the equipment, then the following comments on interoperability apply.

## Phased implementation of interoperability

S-98 is designed with four levels of interoperability, of increasing power and complexity.

ECDIS implementations of interoperability must be capable of applying Level 2 interoperability and shall therefore comply with the “S-98 – Main” document plus S-98 Part B. Since Level 2 interoperability functionality is a superset of Level 1 functionality, it must be possible for the mariner to select between Level 2, Level 1 and Level 0 Interoperability levels.

Implementers should consult S-100 Part 16 for the formal specification of the Interoperability Catalogue model and XML schema, which are common to all Interoperability Catalogues.

## Symbols and icons with interoperability ON

When interoperability is turned on, symbols and icons for chart display of features in interoperating products are selected by interoperability operations or rules, and will be one of the following:

- The standard symbol defined in the portrayal catalogue for the individual product;

- The substitute symbol, if any, defined in the substituteSymbolization sub-element of an S100_IC_DrawingInstruction element of the interoperability catalogue; or

- A symbol defined in the hybrid portrayal catalogue (only for Level 3 or 4 interoperability).

## User interface functions for interoperability

The terms and abbreviations defined in this clause are to be treated like the standard terms and abbreviations as described in SN.243/Rev.2 Annex 2. In particular, as required in that document, terms should be shown in lower case and abbreviations in upper case.

### General navigation functions

This document does not define any general navigation functions in addition to those in IMO/IEC standards

### Control of chart display functions

The mariner must be provided with the functions for control of chart display listed in the table below. If the case of conflict, the terms, abbreviations and icons given in MSC.1/Circ.1609 shall take precedence over those in this document

| Explanation | Term | Abbreviation | Hydrographic Symbol | Icon (“hot key”) |
| --- | --- | --- | --- | --- |
| Selector for interoperability level | Level 1 visual interoperability | VINTOP L1 | (none) |  |
|  | Level 2 visual interoperability | VINTOP L2 | (none) |  |
|  | No visual interoperability | VINTOP OFF | (none) |  |
| Selector for predefined combination in the currently active interoperability catalogue (see notes 1, 2) | Visual interoperability ruleset | VINTOP RULES | (none) |  |

Table C-2 - Additional functions for control of chart display

Notes:

- This function should be disabled when interoperability is OFF or the interoperability level of the Catalogue as a whole (attribute interoperabilityLevel of S100_IC_InteroperabilityCatalogue) is 1. Level 1 Catalogues do not include predefined combinations.

- This function should display a list of predefined combinations in the currently active Interoperability Catalogue. Names of predefined combinations can be found in the name attribute of S100_PredefinedCombination in the Interoperability Catalogue. The name should be displayed; other information (description, use condition, level) should either be displayed at the same time or accessible from the display.

### Functions for control of chart functionality

The following functions should be added to the user interface, extending Table 3 (Control of Chart Functionality) in MSC.1609:

| Explanation | Term | Abbreviation | Icon (“hot key”) |
| --- | --- | --- | --- |
| To toggle interoperability | Visual interoperability Layer merging | VINTOP LYR MRG |  |

Table C-3 - Additional functions for control of chart functionality

### Database functions

The following functions should be added to the user interface, extending Table 4 (Database Functions) in  MSC.1/CIRC.1609.

| Function | Term | Abbreviation | Icon (“hot key”) |
| --- | --- | --- | --- |
| To review available interoperability catalogues | Rulesets for layer merging | LYR MRG INDX |  |

Table C-4 - Additional database functions

### DELETED

### Function accessibility

The following functions should be added to the list in  MSC.1/CIRC.1609 of functions accessible by single or simple operator action (Table 2 in  MSC.1/CIRC.1609 App. 4):

| Function | Equipment | Access |
| --- | --- | --- |
| Disable visual interoperability without loading or unloading data products to the display | ECDIS | Single operator action |
| Load data products in a predefined combination and activate interoperability | ECDIS | Simple operator action (S-100 Part 16, clause 16-4.4.2.6) |
| Change interoperability level when interoperability is active | ECDIS | Simple operator action (S-100 Part 16, clause 16-C-5) |
| Change predefined combination to a different predefined combination covering the currently loaded products on the display at the current level | ECDIS | No operator action other than selecting predefined combination. Interoperability rules from the new predefined combination must be applied automatically (S-100 Part 16, clause 16-C-5.) |
| Load additional product to the display and apply interoperability rules when there is a predefined combination covering the new set of loaded products at the current level | ECDIS | No operator action other than loading product. Interoperability rules from the applicable predefined combination must be applied automatically (S-100 Part 16, clause 16-C-5.) |

Table C-6 - Additional functions accessible by single or simple operator action

### Deleted.

### Status display

Interoperability status (indicating whether visual interoperability rules are being applied to the display) and current interoperability level should be indicated by a status indicator in the user interface. The selector for interoperability level can serve this purpose when visible.

### Settings

The default and user ECDIS settings related to interoperability are listed in Table C-8. The principles in  MSC.1/CIRC.1609 Appendix 5 also apply to interoperability settings.

| Function | Setting |
| --- | --- |
| Interoperability toggle | Interoperability Off |
| Selector for interoperability level | Remain unchanged if previously set (selector disabled, because the default setting is interoperability Off) |
| Predefined combination | Remain unchanged if previously set |
| S-100 data products displayed on screen (see IMO MSC.1/CIRC.1609 for AIS, Radar, Own Ship, and Track settings) | S-101 + any active data products in overlay mode |
| Selected interoperability catalogue | IHO-provided Catalogue |

Table C-8 - Additional settings configured in response to “Default” selection

The defaults in Table C-8 supplement the defaults specified in MSC.530(106), MSC.252(83) and IEC 61174.

If a system does not implement an interoperability level, the corresponding user interface elements must either be omitted from the interface, or disabled as specified in C-8.3.7.

### Display overload indications

Clause C-6.2 describes possible problems with data overload and the likely practical limits on the number of data products simultaneously loaded to a navigation display.

If the number of layers loaded on the display appears to be excessive, the ECDIS should provide a noticeable but unobtrusive (that is, non-blocking) visual indication of possible visual overload.

The criteria for deciding when the number of layers appears to be excessive are left to manufacturers, but a trigger level of 4 S-100 data products is suggested. (In the absence of sufficient data products and applications to experiment with, it is hard to be more definite.) Manufacturers may vary the trigger level depending on experience or particular conditions; for example, displays being used for route planning may allow one or two more products before triggering the indicator.

# Display Organisation and Operation

## Display of non-S-100 information

This clause describes the display of information that is not from official S-100-based data products, S-57, radar, or AIS.

### Distinguishing between official data and additional data

IMO Performance Standards (MSC.530(106) section 1.5) states that ECDIS should enable the mariner to execute all route planning, route monitoring, and positioning and sections 3.3-3.4 mean that the ENDS and System Database may contain information from nautical publications as well as ENCs. This specification requires that ECDIS distinguish between official data and such additional data from users (mariners) and manufacturers. The colour and symbol usage for mariners’ and manufacturers’ data in the IHO GI Registry and the Portrayal Catalogues for S-100 data products are designed to implement this while ensuring the display remains clear and uncluttered.

The official status of data delivered to the ECDIS is defined by the value of a role. This is encoded in the ST field of the X.509 certificate referenced in the mandatory digitial signature accompanying content in the exchange set catalogue. Digital signatures are mandatory for any data imported into the ECDIS, whether official or not. The mandatory fields within every authenticating certificate are defined as:

- C (Country) = ISO Country Code of state making request

- ST (State or Province) = A code reflecting the role of the signing entity (below)

- O (Organisation) = member state organisation name (text) or other organisation name

- CN (Common Name) =  IHO data producer code integer and alpha code (as part of a colon separated MRN), e.g. urn:mrn:iho:aa:1810 or urn:mrn:iho:GB:540. For datasets the CN data producer integer and alpha code must match those contained in the corresponding dataset and its CATALOG.XML entry.

For ECDIS, the ST code holding the role determines whether a dataset or supporting resources is unofficial or official. The following codes are supported by the IHO data protection scheme:

- For S-100 datasets or supporting resources the following roles define official data.

  - DATA_PRODUCER Data Producers - producing data content for live navigation under SOLAS. This data is "official"

  - DATA_AGGREGATOR RENCs/Aggregators - validate, distribute and (sometimes) digitally sign data on behalf of their members. These organisations do not create data content but can sign data as “official”

- S-128 datasets used for ECDIS Update Status reports. These datasets may also be authenticated by:

  - AGGREGATOR S-128 producers – aggregate data together for the purposes of running a service for end users. They create and digitally sign S-128 datasets which support their service offering and the production of update status reports

- Catalogues

  - SCHEME_ADMINISTRATOR The Scheme Administrator, the IHO. Only digitally signed catalogues with a certificate authenticated by the scheme administrator may be loaded onto the ECDIS.

ST field values using the role OTHER_DATA_PRODUCER (or any other value, or those not conforming to the CN MRN format above) represent data or catalogue content which is "unofficial".

IMO Guidelines (SN.243/Rev.2 and  MSC.1/CIRC.1609) describe "Mariner's Navigational Features" for route planning and route monitoring chartwork. The descriptions may be in the same format as chart features, in order to avoid the ECDIS having to deal with two differently coded types of data.

Mariners may alter the IMO categories for Mariner's Features (but not for ENC features). Note, however, that IMO MSC.530(106) section 11.4.1 requires that own ship’s position and selected planned route should always appear when the display covers either, and these “features” must therefore remain in Display Base.

Viewing groups assigned to mariner’s and manufacturer’s features will not be provided in any Portrayal Catalogue; it is the responsibility of application developers to implement support for these features:

- Using viewing groups as specified in C-9.5.5;

- Using viewing group layers as specified in C-9.5.6.

Note that Mariner's Features should be kept independent of data from official or non-official producers in the System Database, and that mariners' information does not need to be split into datasets.

In referring to Mariner's Features it is important to distinguish between:

- "Add/Enter", "Revise" or "Delete" mariner's or manufacturer's information (this refers to the contents of the System Database); and

- "Display" or "Remove" the information (this refers to the ECDIS display).

### Mariner’s information on the route monitoring display

In addition to the ability to enter manual chart corrections and to carry out route planning and route monitoring chartwork, the mariner should be provided with the capability of adding at least the following symbols, lines and areas to the System Database; and should be able to revise or delete them:

- The caution “(!)” or information “[i]” symbol, used to call up a note on the text display by cursor picking;

- Simple lines and areas with or without colour fill, set up for cursor picking to give an explanatory note in the text display;

- Any of the chart symbols from the Portrayal Catalogue;

- Text notes.

Information added by the mariner should be in normal chart colours as specified in the Portrayal Catalogue.

Other information added by the mariner should be distinguished by the colour orange (colour token NINFO) except for colour fill, which should use transparent yellow (colour token ADINF).

Mariner's information should not overwrite official information.

### Manufacturer’s information on the route  display

ECDIS must also be capable of accepting updates to the ENDS data entered manually with simple means for verification prior to the final acceptance of the data. They must be distinguishable on the display from ENDS information and its official updates and not affect display legibility

If the manufacturer should add non-chart information to the System Database, they should use the following symbols, lines and areas:

- The circled “!” caution symbol CHINFO11, or boxed “i” information symbol CHINFO10, used to call up a note on the alphanumeric display by cursor picking;

- Simple lines, or areas without colour fill, set up for cursor picking to give an explanatory note in the alphanumeric display (colour fill should not be used);

- Non-chart information entered by the manufacturer should be distinguished by the colour yellow (colour token ADINF). It should not overwrite official ENC information.

If the manufacturer should add non-official information to the System Database, it should be symbolised in the same way as official chart information and distinguished from official chart information as described for the various cases below:

- Limited non-official data is added to existing official data to augment the chart information: Each feature should be marked by the special identifiers.

- An area of non-official data is located in waters for which official chart data exists: It is superimposed on the official data. In some cases the non-official data may be more appropriate for the intended purpose, for example it may be more detailed.

If the mariner selects the non-official data, the boundary of this data should be identified on the ECDIS display by the line style NONHODAT and the warning “Unofficial data displayed; refer to official RNC or paper chart” should be displayed. The diagonal stroke of the line should be on the non-HO data side.

- An area of non-official data is located wholly outside the area covered by official data (although it may share a boundary with the official data) but is shown on the same display as official data.

- The entire display contains nothing but non-official data: The warning “No official data available; refer to official RNC or paper chart.” should be displayed. In this case, special identifiers need not be used.

The mariner should be able to remove all manufacturer's information if the need should arise.

### Supplemental display items

The following information should ibe shown on demand on the same screen as the chart display or on an additional graphic or text display:

- Positional data and time;

- Legend (See C-9.1.6);

- Feature description and associated attributes (result of "cursor query") in human readable language, including the meaning given in the Portrayal Catalogue for any symbol selected by cursor-pick; textual information from ENC, for example, dataset name, compilation date, date of issue;

- Record of updates for all data products;

- ECDIS Chart 1 (available from IHO S-164);

- Colour differentiation diagram;

- Black adjust symbol for contrast adjustment;

- List of categories which are removed from Standard Display;

- Edition and update numbers of S-101 and other Products in use;

- Details of navigational warnings, accessible in the following categories (this is the minimum set; manufacturers may add other categories or access methods, or substitute equivalent methods):

  - Result of “cursor query” on MSI feature(s);

  - Selection from a list of navigational warnings provided elsewhere (outside the chart graphic) on the same screen, or on an additional graphic or text display;

  - Within a user-specified distance from the vessel’s current position;

  - Within a user-specified distance from the planned route (the same distance as the previous item);

  - In the local NAVAREA, and neighbouring area when the vessel is near a NAVAREA boundary.

[Refer to NAVIGATIONAL WARNINGS SUBSECTION]

### Units

The units listed below should be indicated in the display legend:

Position:	Latitude and longitude in degrees, minutes and decimal minutes.

Depth:		Metres and decimetres.

Height:		Metres.

Distance:	Nautical miles and decimal miles; or metres.

Speed:		Knots and decimal knots.

### Legend

A standard legend of general information relating to the area displayed, applicable to the position selected by the mariner, should be shown on a graphic or text display. This legend should contain at minimum:

- Units for depth;

- Units for height;

- Scale of display; in addition overscale indication where appropriate;

- Data quality indicator;

- Sounding/vertical datum;

- Horizontal datum;

- The value of the safety depth if used;

- The value of the safety contour selected by the mariner, as well as the value of the safety contour displayed (which may be different from that selected by the mariner);

- Magnetic variation;

- Date and number of last update(s) affecting the datasets currently in use;

- Edition number and date of issue of the datasets;

- Chart projection.

See clause C-12.10.3 for details about the listed items.

Since attempting to display all the above items for all displayed data products may lead to an unduly large legend, manufacturers may suppress information from data products other than ENCs. If this is done, the suppressed information should be available through simple operator action, such as a temporary expansion of the legend activated by clicking on a target in the legend. (Note that some of the items will be the same for all data products - in particular, units and datums should be the same for all products, or converted to present the same display result, in order to reduce the chances of user error.)

## Priority of information [Informative]

### Priority layers

The IMO Performance Standard divides System Database information into three categories that determine what data is to be on the display: Display Base (always present on the display); Standard Display (the default display); and Other Information (displayed on demand). (MSC.530(106), Appendix 2).

There are 10 basic priority layers for the drawing sequence of the data on the display, ordered from higher to lower priorities in the list below:

- ECDIS visual alerts/indications (for example caution, overscale);

- Official-data: Points/lines and areas + official updates;

- Notices to Mariners, manual input and Navigational Warnings;

- Official-caution (ENC and other cautions);

- Official-colour-fill area data;

- Official on demand data (for example, water levels, surface currents, underkeel clearance);

- Radar and AIS information;

- Mariners data: Points/lines and areas;

- Manufacturer's data: Points/lines and areas;

- Mariners colour-fill area data.

This list is not intended to indicate a drawing sequence, but to specify that the information content of category n+1 must not obscure the information content of category n, or any higher-priority category (n-1 etc.).

It must be possible to remove the radar information, AIS information and other navigational information by single operator action

### Radar priority

Radar priority for a given display plane is described by the order attribute within the Portrayal Catalogue.

RadarOverlay Portrayal Catalogue context parameters must be updated whenever the radar image is toggled on or off.

When present, the radar image should be written over Portrayal Catalogue display planes with a negative order attribute; and below display planes with a positive order attribute.

Opaque area fills should generally be assigned to display planes with negative order attributes so that they do not obscure the radar image.

Symbolization of curves and points should generally be assigned to display planes with positive order attributes so that they are not obscured by the radar image.

In order to meet the requirements of IMO MSC.530(106) section 11.4.17 to adjust the ship’s position, the ECDIS may incorporate the capability of temporarily changing the radar image priority during the adjustment..

As a fail-safe, features are automatically assigned to an above Radar display plane if the displayPlane field in the drawing instruction is empty. A warning should be recorded by the system in this case.

## Display categories

### IMO Categories

The IMO "Display Base" is that part of the Standard Display that should never be removed. It is a list of basic features which the IMO consider to be required at all times, in all geographic areas and under all circumstances. The IMO do not intend the Display Base to be sufficient for safe navigation on its own, therefore it should not be possible to "Show Display Base" without any additions.

The IMO "Standard Display" is a list of features that the mariner may either add further features to or remove features from (except Display Base), in deciding what is to be displayed. As soon as any feature on this list is removed from the display, or any feature not on this list is added to the display, the display no longer shows the IMO "Standard Display".

The IMO category "Other Information" contains every feature in the System Database which is not classed as "Standard Display".

The mariner should be able to remove information selectively from "Standard Display", except that they cannot remove any feature of the "Display Base". The mariner should also be able to add selectively to the Standard Display any items of the "Other" category.

The portrayal generates one or more drawing instructions for each feature. Each drawing instruction has a display mode, which may correspond to an IMO category. The mariner may override the category for mariner's features, but not for chart features.

### Selecting features for the route monitoring display

Section 5.5 of the IMO Performance Standard (MSC.530(106)) requires that "It should be easy to add or remove information from the ECDIS display". For example, the mariner might want to retain "caution areas" from the Standard Display (see Table C-12) but remove the other areas, including features such as protected areas (for example, "game preserve"). Another example is that they might want to add soundings from "Other", but omit submarine cables (which cause serious clutter at certain ocean terminals).

As an aid to adding and removing information from the display, the Portrayal Catalogue for a product also assigns every feature in the product to a viewing set, and these sets are arranged in groups which are related to the lettered groups of INT 1 for the paper chart, a classification which is familiar to the mariner.

The manufacturer is responsible for organising System Database features, viewing sets and viewing groups for display in such a way that the mariner has reasonable flexibility in selecting what he sees without the selection process becoming too complex. Note that it is not necessary to provide access to each feature, viewing set or even viewing group individually.

Note that combined groups which include features from different IMO categories should not be linked permanently, since doing so would, in effect, drag additional features into the Display Base or Standard Display. For example, if a combined group includes soundings (which are "Other") and lights (which are "Standard Display"), this link should not cause soundings to be shown with the Standard Display on first switching on the ECDIS.

### Selecting text for the route monitoring display

In S-100 based products, the display of text is controlled as specified in clause C-10.3.2. There is no special provision for display of text unless specified within a particular Product Specification.

### Linkages between ECDIS options

Because vessels vary widely in their requirements and their operational procedures, linkages between options which reduce the flexibility of the ECDIS must be avoided.

### Independent Mariner Selections

- Selecting a Display Category must turn ON all VG/VGL allocated to that Display Mode in S-98 with the exception of IMS Objects.

- Objects allocated to a 'higher' Display category (i.e Other) can be added to a lower category (i.e. Standard) by individually activating their associated VGL. This action must not load all other layers allocated to their default Display Category (i.e. Turning ON 'Accuracy' when in Standard Display must not switch the display mode to 'Other').

- When additional VG/VGL are manually added by the user to a Display category, ECDIS should add the character '+' at the end of the name of the Display Mode (i.e. Standard+ when Accuracy is added to the Standard Display category).

- If a VG/VGL is active, IMS related to the objects in that VG/VGL can be activated by the Mariner without triggering the loading of all VGL related to the Display category the IMS relates to [i.e. if LIGHTS have been manually added to Base Display (Base+)], 'Date Dependent' and 'Highlight Date Dependent' symbols must be shown if they apply to LIGHTS).

- Once a Display Mode is selected by the Mariner, it shouldn't be possible to turn 'core' VG/VGL OFF. Mariners should load a lower Display category first (i.e. Base) and then add additional layers as required to build their preferred view.

## Drawing priorities [Informative]

Display priorities control the order in which the output of the portrayal functions is processed by the rendering engine. Priorities with smaller numerical values will be processed first. In the S-100 portrayal model the display priority is known as the drawing priority.

The drawing priority should be used to ensure that features that overlap each other are drawn in the right sequence. Thus, a feature with a higher priority should be drawn after (on top of) a feature with a lower drawing priority.

The drawing priority applies irrespective of whether a feature is a point, curve or surface. The following rule applies:

- The drawing of manual chart correction lines is an exception to the rule for suppressing overlapping lines. The manual chart correction lines (line styles CHCRID01 and CHCRDEL1) should coexist with the underlying line. Both these manual chart correction line styles and the underlying line should be drawn.

The following table gives an indication of how drawing priorities are allocated. Within each group priorities are adjusted to meet specific cases. Italics indicate S-100 data products other than ENC (only the “basic” and “other” products from clauses C-6.1.1 and C-6.1.2 are included). The Table below is indicative; definitive priorities are assigned in the Portrayal Catalogues.

| Data Groupings (italics - data other than S-101) | Drawing Priorities (0-9 scale) | Drawing Priorities (0-27 scale – IHO GI Registry) | Drawing Priorities (proposed) |
| --- | --- | --- | --- |
| No S-101 data filled area pattern | 0 | 0 | 0 |
| Reserved | -- | -- | 1-8 |
| S-101 Skin of the earth filled areas S-102 bathymetry coverages | 1 | 3 | 10–19 |
| Superimposed areas S-123 radio service areas S-412 weather (non-hazardous) | 2, 3 | 6, 9 | 20–39 |
| Restricted area S-122 protected areas | 5 | 15 | 50–59 |
| Traffic areas VTS areas S-129 go/no-go areas S-127 traffic areas | 6 | 18 | 60–69 |
| land features | 4, 5 | 12, 15 | 40–59 |
| Water features S-111 surface currents coverages S-104 water levels coverages S-411 sea ice | 3, 4, 5, 6 | 9, 12, 15, 18 | 30–69 |
| Coastline features | 5, 6, 7 | 15, 18, 21 | 50–79 |
| Routeing lines S-127 routeing lines | 5, 6, 7 | 15, 18, 21 | 50–79 |
| Symbols for lines and areas | 4, 5, 6 | 12, 15, 18 | 40–69 |
| Hazards (bridge, safety contour) S-124 navigational warnings S-102 safety contour S-412 hazards and storms | 8 | 24 | 80–89 |
| Aids to navigation | 8 | 24 | 80-89 |
| Text Operating times (schedules, etc.) | 8 | 24 | 80–89 |
| Mariners VRM & EBL S-421(?) | 9 | 27 | 90–98 |
| Own ship | 9 | 27 | 99 |

Table C-9 - Drawing priorities [Informative]

NOTE: The rule of thumb for conversion between the new scale (z) and the 0-9 scale (x) and 0-27 scale (y) can be described by z ≈ 10x+5 ≈ 10y/3 + 5. The “No ENC data” and “Own ship” categories are placed to the extreme ends of the new scale and a reserved range inserted for priorities 1-8.

## Viewing groups

The viewing group controls the content of the display. It provides an on/off switch in the Portrayal Catalogue for any drawing instruction assigned to the corresponding viewing group. A viewing group can be regarded as a filter on the list of drawing instructions.

Viewing groups are 'on' or 'off' switches for use by the Mariner to control the information appearing on the display, as required by IMO Performance Standards (MSC.530(106) App.2 section 3).  An item in the viewing group table may be a chart feature; a Mariners' or other time-variable feature; a special symbol such as the "depth less than safety contour" pattern; or a non-ENC feature such as the shallow water pattern. Additional 'symbol viewing groups' have been added to allow auxiliary symbols such as contour labels, the 'low accuracy' symbol, etc., to be switched on or off without affecting the primary symbolisation of the feature.

Viewing groups can be aggregated into Viewing Group Layers and Viewing Group Layer can be aggregated into Display Modes. Both aggregations are part of the Portrayal Catalogue

Items in the viewing group tables are arranged in numbered groups (for example, group 26230 consisting of the items pipeline area and cable area) which in turn are arranged in layers (for example, layer 26000 consisting of cautionary areas). The groups are arranged by IMO Category, in the sequence of INT 1; upper-case letters following group numbers correspond to symbolisation sections in INT 1.

Viewing groups from different IMO categories must not be combined under a single display selector.

[ Needs reference Independent Mariner Selections . See reference Portrayal Issue https://github.com/S-101-Portrayal-subWG/Working-Documents/issues/18 ]

### Viewing group layers [Informative]

Table C-10 shows the layers and numbering scheme for viewing groups. Groups for Mariner’s information are in italics.

The viewing groups in this table cover the “Basic” and “Other” products listed in clauses C-6.1.1 and C-6.1.2. Viewing groups for other S-100 products will be assigned later.

| DISPLAY BASE | STANDARD DISPLAY | OTHER INFORMATION |
| --- | --- | --- |
| 00000–09999 reserved for administrative purposes |  |  |
| 10000 reserved 40000 reserved | 20000 reserved 50000 reserved | 30000 reserved 60000 reserved |
| 11000 A,B information about the chart display 41000 tools | 21000 A,B 51000 tools | 31000 A,B 61000 tools |
| 12000 C, D, E, F land features | 22000 C, D, E, F | 32000 C, D, E, F |
| 13000 H, I depths & currents | 23000 H,I | 33000 H,I S-102 bathymetry S-104 water levels S-111 surface currents |
| 14000 J,K,L obstructions, pipelines | 24000 J,K,L | 34000 J,K,L |
| 15000 M traffic,routes | 25000 M | 35000 M S-127 traffic routes |
| 16000 N special areas | 26000 N | 36000 N S-122 protected areas S-129 underkeel clearance areas |
| 17000 P,Q,R,S buoys, beacons, lights, radar | 27000 P,Q,R,S 57000 reserved | 37000 P,Q,R,S 67000 reserved |
| 18000 T,U services & small craft facilities | 28000 T,U S-127 pilotage features 58000 reserved | 38000 T,U S-123 radio services 68000 reserved |
| 19000–19999 reserved 49000 49999 reserved | 29000 29999 reserved 59000 59999 reserved | 39000 39999 reserved 69000 69999 reserved |
| 70000–70999 reserved 71000–71999 reserved | 72000–72999 reserved 73000–73999 reserved | 74000–74999 Weather and natural conditions (S-412) 75000–75999 reserved |
| 76000–76999 reserved 77000–77999 reserved | 78000–78999 reserved Navigational Warnings (S-124) 79000–79999 reserved | 80000–80999 Navigational warnings (S-124) 81000–81999 reserved |
| 82000–99999 reserved for future use |  |  |

Table C-10 - Viewing group layers and numbering scheme [Informative]

### Viewing groups for display base

The groupings in Table C-11 list the contents of the Display Base. All features of this category should be permanently retained on the ECDIS display (subject to the application of interoperability when interoperability is activated).

Features are S-101 features except where designated as being from other products.

| Group | Contents |
| --- | --- |
| RESERVED |  |
| 10000-10999 | Reserved for chart information |
| A, B | CHART FURNITURE |
| 11000 | Information about the Chart Display |
| 11010 | cursor [symbol CURSRA01] |
| 11020 | na (not assigned) |
| 11030 | scalebar [symbol SCALEB10], latitude scale [symbol SCALEB11] |
| 11040 | north arrow [symbol NORTHAR1] |
| 11050 | no data [solid fill with colour token NODTA, area fill NODATA03], unsurveyed (UNSARE), incompletely surveyed area |
| 11060 | Non-HO data boundary [line style NONHODAT] |
| C, D, E, F | TOPOGRAPHY AND INFRASTRUCTURE |
| 12000 | Land area |
| 12010 | land area |
| 12200 | Dangers above water |
| 12210 | bridge, pylon, overhead cable, conveyor, overhead pipeline, offshore platform |
| 12400 | Shoreline |
| 12410 | coastline, ice shelf, glacier, shoreline construction, tie-up wall, dolphin, gate, pile, crib, wellhead, ice boom, floating dock, hulk, pontoon, oilboom, log boom, flood barrage |
| 12420 | dock , lock , canal , river |
| H, I | HYDROGRAPHY |
| 13000 | Safety Contour |
| 13010 | safety contour (from portrayal rule DEPCNT03) |
| 13020 | na |
| 13030 | depth area , dredged area , |
| J, K, L | SUBSEA FEATURES |
| 14000 | Dangers under water |
| 14010 | isolated underwater dangers in water deeper than the displayed safety contour (rocks, wrecks, obstructions, mooring cables from portrayal rule CableSubmarine) |
| M | TRACKS AND ROUTES |
| 15000-15999 | na (not assigned) |
| N | SPECIAL AREAS |
| 16000 16999 | na (not assigned) |
| P, Q, R, S | AIDS TO NAVIGATION |
| 17000-17999 | na (not assigned) |
| T, U | SERVICES |
| 18000 18999 | na (not assigned) |
| RESERVED |  |
| 19000-19999 | Reserved for chart information |

Table C-11 - Viewing groups and their contents for Display Base [Informative]

### Viewing groups for Standard Display

The groupings in Table C-12 illustrate the contents of the Standard Display. Features are S-101 features except where designated as being from other products.

| Group | Contents |
| --- | --- |
| RESERVED |  |
| 20000 | Reserved for chart information |
| A, B | INFORMATION ABOUT THE CHART DISPLAY |
| 21000 | Information about the Chart Display |
| 21010 | Unknown feature (magenta question mark) (retained as failsafe) |
| 21020 | Generic Feature |
| 21030 | Chart scale boundary, overscale data [area fill OVERSC01] |
| 21040 | na |
| 21050 | na |
| 21060 | Place-holder for geographic names (LNDRGN, SEAARE) |
| C, D, E, F | NATURAL & MAN MADE FEATURES, PORT FEATURES |
| 22000 | Major Coastal Features |
| 22010 | Lake, sloping ground, slope top, dyke, causeway, dam |
| 22200 | Conspicuous landmarks: |
| 22210 | Radar conspicuous feature |
| 22220 | Visually conspicuous feature |
| 22230 | na |
| 22240 | Built up area |
| H, I | DEPTHS, CURRENTS, etc. |
| 23000 | Depths |
| 23010 | Area of depth less than the safety contour (DIAMON01 pattern) |
| 23020 | na |
| 23030 | Swept area |
| J, K, L | SEABED, OBSTRUCTIONS, PIPELINES |
| 24000 | Seabed dangers |
| 24010 | Mooring cables, tunnel on seabed, sandwaves |
| M | TRAFFIC ROUTES |
| 25000 | Routes and Tracks |
| 25010 | Leading line, clearing line, traffic lane, deep water route, traffic separation area, traffic separation line, traffic roundabout, traffic crossing, precautionary area, traffic separation scheme boundary, deep water route centre line, two way route part, inshore traffic zone |
| 25020 | Recommended track, recommended traffic lane, recommended route centreline |
| 25030 | Ferry route |
| 25040 | Radar line, limit of shore radar |
| 25060 | Radio calling-in point |
| N | SPECIAL AREAS |
| 26000 | Restricted and Cautionary Areas |
| 26010 | Restricted area, both Navigational and Regulatory |
| 26020 | na |
| 26030 | na |
| 26040 | Submarine transit lane, military practice area, sea plane landing area, offshore production area |
| 26050 | Fairway |
| 26150 | Caution area |
| 26200 | Information Areas, Protected Areas |
| 26210 | Fishing ground, marine farm, Fishing Facility |
| 26220 | Anchorage area, anchor berth |
| 26230 | na |
| 26240 | Dumping ground |
| 26250 | Cargo transhipment area |
| 26260 | Archipelagic sea lane |
| P, Q, R, S | BUOYS & BEACONS, LIGHTS, FOG SIGNALS, RADAR |
| 27000 | Buoys, Beacons, Topmarks, Lights, Fog Signals, AIS AtoNs |
| 27010 | Buoy, light float, mooring buoy |
| 27011 | Light vessel |
| 27020 | Beacon |
| 27025 | Daymark |
| 27030 | Distance mark |
| 27040 | Direction of buoyage IALA buoyage regions |
| 27050 | Topmarks for paper chart symbols |
| 27060 | Gridiron |
| 27070 | Light |
| 27080 | Fog signal, retro-reflector |
| 27200 | Radar |
| 27210 | Racon |
| 27220 | na |
| 27230 | Radar reflector |
| 27240 | Physical and Virtual AIS Aids to Navigation |
| T, U | SERVICES & SMALL CRAFT FACILITIES |
| 28000 | Services |
| 28010 | Pilot boarding place, pilotage district |
| 28020 | Signal station traffic, signal station warning |
| 28030 | Vessel traffic service area |
| 28040 | S-127 pilotage features |
|  | NAVIGATIONAL WARNINGS |
| 78100 | Navigational warnings types [A, B, C] |
| 78110 | Navigational warnings types [D, E, F] |
| RESERVED |  |
| 29000 | reserved for chart information |

Table C-12 - Viewing groups and their contents for Standard Display [Informative]

### Viewing groups for other chart information

The groupings in Table C-13 illustrate viewing group usage for other information. Features are S-101 features except where designated as being from other products.

| Group | Contents |
| --- | --- |
| RESERVED |  |
| 30000 | Reserved for chart information |
| A, B | INFORMATION ABOUT THE CHART DISPLAY |
| 31000 | Information about the Chart Display |
| 31010 | Quality of non-bathymetric data, quality of survey, quality of bathymetric data for ENC data |
| 31011 | Symbol LOWACC01, identifying low accuracy data, applied to the spatial feature of point and area wrecks, rocks and obstructions and to point land areas |
| 31012 | Quality of non-bathymetric data, quality of survey, quality of bathymetric data for non-ENC data |
| 31020 | Reserved (retained for backward compatibility in dual-fuel displays - was M_NPUB in S-52) |
| 31021-31029 | Reserved for supplementary information in products other than S-101 |
| 31030 | Supplementary Information - S-101 feature InformationArea; NauticalInformation instances associated to S-101 features. |
| 31031 | Reserved for Additional Documents (referenced by fileReference attributes). |
| 31040 | DataCoverage |
| 31050 | na |
| 31060 | na |
| 31070 | na |
| 31080 | Magnetic variation , local magnetic anomaly |
| 31090 | Update information features |
| C, D, E, F | NATURAL & MAN MADE FEATURES, PORT FEATURES |
| 32000 | Natural Features |
| 32010 | Dunes, hills, ridge, clifftop, contours and elevation |
| 32020 | na |
| 32030 | Trees, vegetation, mangrove, marsh |
| 32040 | na |
| 32050 | River or lake; also rapids, waterfall |
| 32060 | na |
| 32070 | Tideway |
| 32080 | na |
| 32200 | Shore Structures |
| 32210 | na |
| 32220 | Any of the following not classified as visually conspicuous: landmark, building, tank, silo, water tower, wall, fort |
| 32230 | na |
| 32240 | Airport, runway |
| 32250 | Railway, road, tunnel, control point |
| 32260 | na |
| 32270 | Quarry, refinery, power station, tank farm, wind farm, factory, timber yard |
| 32280 | na |
| 32400 | Port Features |
| 32410 | Harbour type, customs check point [note: "small craft facilities" is in group 38210] |
| 32420 | na |
| 32430 | na |
| 32440 | Berth number, mooring facility (such as bollard), gate (such as lock gate), dry dock, crane |
| 32450 | na |
| 32460 | na |
| H, I | DEPTHS, CURRENTS ETC |
| 33000 | Depths, Currents, Tide Rips, etc |
| 33010 | Soundings |
| 33020 | Depth contours other than the safety contour |
| 33021 | Label for the safety contour |
| 33022 | Label for contours other than the safety contour |
| 33030 | na |
| 33040 | Water turbulence |
| 33050 | Reserved (retained for backward compatibility in dual-fuel displays) |
| 33060 | Current and tidal stream information |
| 33100 | S-102 Bathymetry features |
| 33200 | S-104 Water level features |
| 33300 | S-111 Surface currents features |
| J, K, L | SEABED, OBSTRUCTIONS, PIPELINES |
| 34000 | Seabed Information: Rocks, Wrecks and Obstructions; Pipes and Cables |
| 34010 | Nature of seabed |
| 34020 | Spring, sea weed |
| 34030 | Pipeline area, cable area |
| 34040 | na |
| 34050 | Rocks, wrecks, obstructions, which are not a danger to own ship's navigation (these are all Display Base if a danger to own ship) |
| 34051 | Non-dangerous rocks, wrecks and obstructions which have a valueOfSounding attribute and are not a danger to own-ship’s navigation (these features are all Display Base if a danger to own-ship) |
| 34060 | na |
| 34070 | Submarine cable, submarine pipeline |
| M | TRAFFIC ROUTES |
| 35000 | Routes |
| 35010 | na |
| 35100 | S-127 Marine Traffic Management traffic routes & associated features |
| N | SPECIAL AREAS |
| 36000 | Administrative Areas, (by cursor enquiry) |
| 36010 | Continental shelf, |
| 36020 | Harbour area, free port area, customs zone |
| 36030 | na |
| 36040 | Fishery zone |
| 36050 | Contiguous zone, exclusive economic zone, national territorial area, territorial sea, territorial sea baseline, administration area |
| 36100 | S-122 Marine Protected Areas features |
| 36200 | S-129 Underkeel Clearance features |
| P, Q, R, S | BUOYS & BEACONS, LIGHTS, FOG SIGNALS, RADAR |
| 37000-37999 | na |
| T, U | SERVICES & SMALL CRAFT FACILITIES |
| 38000 | Services |
| 38010 | Radar station, radio station |
| 38020 | na |
| 38030 | Coastguard station, rescue station |
| 38200 | Small Craft Facilities |
| 38210 | Small craft facility |
| 38300 | S-123 Marine Radio Services features |
|  | WEATHER AND NATURAL CONDITIONS |
| 74100 | Weather hazards (S-412) |
| 74110 | Wave hazards (S-412) |
|  | NAVIGATIONAL WARNINGS |
| 80100 | Navigational warnings types [X, Y, Z] (S-124) |
| 80110 | Navigational warnings types [P, Q, R] (S-124) |
| 80200 | Navigational warnings (other) (S-124) |
| RESERVED |  |
| 39000 | Reserved for chart information |

Table C-13 - Viewing groups and their contents for other information [Informative]

### Viewing groups for mariner’s and manufacturer’s information

Table C-14 illustrates the contents of viewing groups for mariner’s information.

| Groups | Contents |
| --- | --- |
| MARINERS' FEATURES |  |
| 53000 | Mariners' Features |
| 53010 | Danger highlight (dnghlt) |
| 53030 | Mariners' information note (marnot catnot 1) |
| 53040 | Mariners' cautionary note (marnot catnot 2) |
| 53050 | Mariners' feature (marfea) |

Table C-14 -Viewing groups and their contents for mariners' information - Standard Display [Informative]

### ECDIS viewing group layer implementation

For standardization of the ECDIS Human Machine Interface (HMI) and to facilitate generic training of Mariners as a minimum the following viewing group layers should be implemented to control display of charted features. An ECDIS may provide more display on/off controls than are available in this table, but OEMs should use the viewing group layer names contained in Table C-15. The viewing group names are derived from the IMO Performance Standards.

| Viewing Group Layer | Name of viewing group layer in the ECDIS | Viewing groups included |
| --- | --- | --- |
| 1 | Display Base | 10000–19999 |
| Standard Display |  |  |
| 2 | Drying line | 22010 |
| 3 | Buoys, beacons, aids to navigation | 21020, 22200–22240, 27000, 27010, 27011, 27020, 27025, 27040, 27050, 27070, 27080, 27200, 27210, 27230, 27240, 27030, 27060 |
| 3.1 | Buoys, beacons, structures | 21010, 21020, 22200–22240, 27000–27050, 27080, 27200–27230, 27030, 27060, 27240 |
| 3.2 | Lights | 27070 |
| 4 | Boundaries and limits | 23030, 26050, 26220, 26240, 26250 |
| 5 | Prohibited and restricted areas | 26000, 26010, 26040 |
| 6 | Chart scale boundaries | 21030 |
| 7 | Cautionary notes | 26150 |
| 8 | Ships’ routeing systems and ferry routes | 25010–25060 |
| 9 | Archipelagic sea lanes | 26260 |
| 10 | Miscellaneous | Switches on and off features in viewing groups 20000–29999 (other than the groups listed in layers 2–9 above) and features in mariners’ information viewing groups 50000–56999 |
| Other |  |  |
| 11 | Spot soundings | 33010 |
| 12 | Submarine cables and pipelines | 34030, 34070 |
| 13 | All isolated dangers | 34050, 34051 |
| 14 | Magnetic variation | 31080 |
| 15 | Depth contours | 33020 |
| 16 | Seabed | 34010, 34020, 33040 |
| 17 | Tidal | 33050, 33060 |
| 18 | Miscellaneous | Switches on and off features in viewing groups 30000–39999 (other than the groups listed in layers 11–17 above) and independent mariner selections (mariners’ tools) other than those listed in Table C-16 (- Mandatory and optional display functions). |

Table C-15 - Recommended set of viewing groups for ECDIS [Informative]

For text groups see clause C-11.2.

## Display modes

See IMO MSC.530(106) and clause C-9.3.1).

## Displaying ECDIS updates

The strategy for displaying ECDIS updates is derived from the IMO Performance Standard MSC.530(106). The citations below are to sections in that Standard.

MSC.530(106) 4.4,	Automatic and semi-automatic updates: These should be displayed in the same manner as ENC information, using standard colours and symbols.

MSC.530(106) 4.5	Manual updates should be distinguished as specified in clause C-14.11

MSC.530(106) 4.7	The mariner should be able to display updates for review as described in clauses C-9.7.1 and C-9.7.2.

### Automatic updates

The manufacturer must provide a means of distinguishing automatic updates. The S-101 Feature Catalogue provides encoders with an Update Information feature that is used to represent a change to the information shown.

EXAMPLE:  The ENC update contains updates to various features that adjoined the feature that was actually updated. In this case, the producer will use an Update Information feature to indicate the actual feature that was affected and a brief description of the changes.

[]

### Manual updates

The manual updates should be distinguishable from official information and its official updates.

## Display functions

Display functions are described in IMO MSC.530(106), MSC.252(83),  MSC.1/CIRC.1609, and clause C-8.3 of this document. Each of these functions has a display mode assigned but should only be shown in that display mode if the Mariner has decided to select this option. Listed in Table C-16 below are the Mariner selectors that should be available in ECDIS, the name of the selector in the ECDIS GUI and a functional description in terms of S-101 information and viewing groups. The Table is not a complete set of ECDIS user functions; there are a number of other Mariner selectors that can be included in ECDIS, listed in the cited IMO documents.

This Table adds a user function for displaying information from additional products to the IMO list.

Context parameters associated with functions listed in Table C-16 are given in the IHO GI Registry. Certain functionality may be implemented by means of viewing groups. Manufacturers may use other means of conveying settings for ECDIS functions to portrayal processing, but should note that some of the context parameters are used by the official IHO S-101 Portrayal Catalogue.

Context parameters may be added or removed by updates to the Portrayal Catalogues. Context parameters which cannot be automatically associated with ECDIS display functions or user interface features must be exposed to the mariner for use.

If a context parameter is removed, the mariner should be notified during implementation of the update of the Catalogues of any associated ECDIS display functions or user interface features that functionality is degraded or deprecated.

All ECDIS-capable products are expected to map the user functions/selectors to viewing groups, layers, and context parameters in their respective Portrayal Catalogues. The ECDIS user interface should combine the effects for all on-screen products, modifying the combination process as guided by the Scope column or equivalent settings.

EXAMPLE 1: Function applicable to all on-screen products: The display should switch to the same national language for all on-screen products when the National Language function is activated by the Mariner.

EXAMPLE 2: Function applicable to S-101 only: The “Accuracy” selector displays accuracy symbols from only S-101 datasets. Accuracy from other S-100 data products is not displayed. This is the default action for this user function; manufacturers may allow switching to displaying accuracy for other data products by further or separate user action.

EXAMPLE 3: Function applicable to S-101 only, because no other product qualifies: The “Full light lines” selector displays full light sector lines for lights encoded in S-101. (There is no question of it applying to any other product, because S-101 is the only product that contains the relevant light features.)

[This table will be updated after S-101 Edition 2.0.0 is published]

| Clause | ECDIS Implementation | Name of Selector in ECDIS | Function Description | Remarks |
| --- | --- | --- | --- | --- |
| C-9.5.4 | Mandatory | Accuracy | Independent pattern selection of data quality (Quality of Bathymetric Data), viewing group 31010. Symbol LOWACC01, viewing group 31011 | According to MSC.1609 “Accuracy” is the function “To show accuracy related symbols”. |
| C-12.7 | Mandatory | Date dependent | Date Dependent Features – to turn on and off the display of temporal features by viewing a date range. |  |
| -- | Mandatory | Full light lines | Full Light Sector Lines | See Note 1 |
| C-12.7.2 | Mandatory | Highlight date dependent | Indication of date dependent features – to turn on and off the display of symbol CHDATD01 |  |
| C-9.5.4 C-12.10.1 | Mandatory | Highlight info | Additional Information - viewing group 31030 | See Notes 2 & 4 |
| C-9.5.4 C-12.10.1 | Mandatory | Highlight document | Additional Information - viewing group 31030 / 31031. | See Notes 2 & 4 |
| C-9.5.4 C-12.10.1 | Mandatory | Highlight overlay info | Additional information in products other than ENC - viewing group 31020-31029 |  |
| C-9.5.3 C-10.1 C-12.6.2 | Mandatory | Unknown | Unknown Features - to turn on the display of features in S-101 datasets which are not specified in the S-101 Product Specification – viewing group 21010 |  |
| C-12.11 | Mandatory | Update review | Review of Updates – This function turns on colour highlighting for the S-101 UpdateInformation feature which have undergone modification in the process of the latest accepted correction. |  |
| C-12.8 | Mandatory | Scale min | Turn SCAMIN Off |  |
| C-12.9.5 | Mandatory | Shallow pattern | Shallow Water Pattern |  |
| C-12.5 | Mandatory | Shallow water dangers | Isolated Dangers in Shallow Water – to turn on the display of isolated danger features which are located in the unsafe waters - viewing group 24050 |  |
| C-9.5.4 | Optional | Contour label | Contour Labels – to turn on the display of contour labels (incl. label of safety contour) – viewing group 33021, 33022 | See Note 3 |
| C-14.6 (Table C-27) | Optional | Four shades | Four Colour Depth Shades |  |
| C-12.10.2 | Optional | National language | National Language, text group 31 |  |
| -- | Mandatory | Paper chart / simplified symbols | Selection of Point symbol style |  |
| -- | Mandatory | Plain / Symbolized boundaries | Selection of Line symbol style |  |

Table C-16 - Mandatory and optional display functions

NOTES:

- Assumes that S-101 or the equivalent is the only data product containing the relevant features. If in the future another product containing relevant features is introduced, this guidance may need to be revised to restrict the scope.

- Other data products also have these attributes and features, but since turning that on may unduly overload the display, this selector is restricted to S-101.

- S-102 contours are displayed only if a contour generated from S-102 data is available and S-102 is activated on the display. See clause C-12.9.11

- “Highlight document” and “Highlight info” produce the same result because the information may be provided in either a text sub-attribute or an external file named by a fileReference attribute. Additional information for features is provided via the feature instance itself or an information association to the information type Nautical Information, which binds the complex attribute information. Distinguishing “highlight document” and “highlight info” may therefore involve a multi-level query. Manufacturers may implement such a query if desired; if implemented, viewing group 31031 must be used for “Highlight document”.

# General Rules for Symbols and Text

## Symbol Specifications

All symbols are specified in the Portrayal Catalogue for the respective Product Specifications and are defined according to S-100 Part 9 appendix B.

Some feature classes do not have a symbol (for example, territorial sea). Such "no symbol" features may be picked up by cursor interrogation of the area.

Should an "unknown feature" occur in the System Database which is not adequately defined or for which no symbol exists, its presence should be indicated on the display by symbol QUESMRK1 (a magenta "?") with the IMO category "Standard Display".

Some features are symbolised differently depending on circumstances (for example the symbol for a contour depends on whether it is the safety contour).

## Guidelines for reproduction of lines, symbols and text

### Introduction (informative)

In a navigation system the viewing distance will be about 70 cm for route planning, but experience to date indicates that the viewing distance for important features during route monitoring may be several metres.

Human factors experts recommend that symbols and characters subtend 20 arc minutes at the observer’s eye. (For example, a symbol viewed from 70 cm for route planning should be about 4mm in size, 1.5 times the size of a normal chart symbol. Two times chart size is a good general rule.) Symbols and characters important for route monitoring may have to be significantly bigger.

For clear representation, symbols require a minimum number of screen units (pixels), depending on their complexity. A simple chart symbol of height 4mm should extend at least 13 pixels for a screen that just meets the current minimum standards for chart display size and resolution (clause C-20.1).

The minimum resolution is defined by IEC 61174 and/or IEC 62288.

### Guidelines for size and resolution

Lines, symbols and text should be large enough that they can be easily interpreted at the operational viewing distance. Also, for clear representation, symbols require a minimum number of screen units (pixels), depending on their complexity. The ensuing requirements for size in absolute and pixel terms are described in this clause.

The minimum sizes for all symbols should be as shown in the Portrayal Catalogue.

Enough "picture units" (pixels) must be used to draw small features and symbols clearly and allow viewers to distinguish similar symbols.  Symbols should therefore always be drawn with at least the same number of pixels as are required to draw the symbol at the size defined in the Portrayal Catalogue for the minimum resolution and minimum chart display area.

NOTE (informative): This requirement means the minimum height in pixels of a symbol is: (symbol height in mm) divided by the "pixel size" for the minimum size chart display (see clause C-20.1).

The text on the ECDIS should be readable from 1 metre,. Pending reconciliation of the two rules of thumb cited in S-52, this Annex adopts the IEC 60945 recommendation of a minimum character height of 3.5mm for a 1-metre viewing distance.

Since viewing conditions and anticipated viewing distances may vary between installations, manufacturers should provide for configuring ECDIS software for viewing distances on different bridges. Operating conditions (for example low illumination, bright daylight, etc.) may also need to be considered.

The CHKSYM01 symbol may be used for comparison; this symbol should measure 5mm×5mm when displayed at its nominal size (that is, scaled to 100%), within the tolerance specified by the IEC testing standard (recommended tolerance: 1 physical pixel width in each of the X and Y dimensions).

Since the CHKSYM01 symbol is quite small for the purpose of on-site system configuration by ordinary users, manufacturers may use an enlarged version or an equivalent method for that purpose.

### Fonts

The portrayal specifies the font for chart data. For non-chart data, sans serif, non-italic fonts should be used. The font should be plain and clearly readable. The computer 0 (‘slashed zero’) should not be used.

Because several appropriate commercial fonts are available, this document does not prescribe a specific font. Manufacturers should make their own arrangements for the use of a font.

### Zooming

When the display scale is enlarged by zooming in, it should be possible to hold symbol size constant. The same applies to text. Symbol and text size should never be decreased when zooming out.

## Mariner's options in symbols and linestyles

At large scale, it may be difficult to identify the area or see on which side of the boundary the area lies. The S-101 Portrayal Catalogue therefore provides, as a mariner's option, symbolised area boundary linestyles for use on large scale displays. These make the areas easier to figure out than the plain linestyles recommended for small scale displays, where symbolised lines would cause clutter. Two options are provided within the portrayal catalogue, to display either symbolised or plain area boundary linestyles.

The mariner should be given the option of selecting the buoy symbols and area boundary linestyles (plain or symbolised) that best fit the situation, without linkages. For example, boundary linestyle should be selectable independent of the choice of buoy symbol, and independent of the actual display scale. See Table C-16: Selection of Point symbol style, and Selection of Line symbol style.

## Display orientation

It should always be possible to display the System Database information north-up (IMO MSC.530(106) section 8.1), but other orientations are allowed.

If the display is oriented course-up, the orientation should not be altered too frequently, in order to avoid jitter from frequent rewriting of chart information.

The north arrow is always required on the display, as part of the IMO Performance Standards Display Base.

## Common text information attributes

Several Product Specifications use similar structures for feature names and information attributes. The following guidelines should be used when this document mentions displaying the “feature name” or “information” attributes.

### Feature names

Attribute “feature name” (featureName) is a complex attribute consisting of the mandatory sub-attribute “name” (the actual name of the feature) and sub-attributes “language” and “display name”. The sub-attribute “display name” is a boolean indicating which one of multiple instances (in the same feature) of the complex attribute featureName should be displayed. Only one instance of featureName attribute of a feature can have displayName=TRUE. Selecting which instance of the featureName attribute should take into account the value of displayName.

### Generic attribute “information”

Attribute “information” is a complex attribute consisting of the following sub-attributes:

- fileLocator: Indicator for location of text within the file named by fileReference;

- fileReference: Name of text file;

- headline: Heading relevant to the content in co-attribute “text” or the file named by fileReference (specifically, the section indicated by fileLocator, if used). The length of “headline” will generally be much shorter than “text”.

- language: The language of the text or referenced file;

- text: A text string providing the information content.

At least one of “fileReference” or “text” must be populated (usually only one of the two).

If headline is populated, it may be used as needed by the manufacturer, for example as a short title in a collapsible sidebar, collapsible menu item, etc.; or as a paragraph heading for the longer content in attribute text or the content of the file named by fileReference.

## Queries

ECDIS must be capable of performing spatial queries on ENC and other data during import and symbolisation. Spatial query is defined as the ability to inspect geographical position and numerical value of spatial coordinates associated with a charted feature.  Spatial query could be available by means of cursor pick or as an independent function.

# Text and Graphics

## Text as part of the display

Text information should be used on the route monitoring display only when unavoidable, since it has to be written large to be readable and so causes clutter.

The complex attribute featureName has a Boolean sub-attribute displayName. When the ENC data is encoded with a displayName as TRUE then it is intended for that instance of featureName to be displayed on the ECDIS. This is intended to declutter much of the text display on an ECDIS. The other alternative featureNames must be able to be queried by the ECDIS Pick Report.

Soundings are treated by the Portrayal Catalogue as symbols to ensure they are legible and correctly located.

In addition the following guidance should be used for text:

- Text is normally coloured black, to give best readability under all light conditions.

- Text should only be displayed when the feature it applies to is displayed.

- Text should always have sufficiently high drawing priority, to ensure it is readable, independent of the feature it applies to.

- The manufacturer should provide the capability to select “Important Text” and “Other Text”, and they may also provide further text groupings if they so wish.

The display of text must be controlled independently of the display of the feature it applies to and the Mariner must have full control over the display of text.

## Viewing groups used for text

Viewing groups for text are listed in Table C-17 below. The content of these tables is purely informative as their precise specification, fulfilling the mandatory requirements are contained within the published Portrayal Catalogue(s).

| Viewing Group | Description | S-101 Features and Attributes (attributes in lower case; indentation indicates attribute or sub-attribute) |
| --- | --- | --- |
| 00-10 | reserved for future assignment by IHO. |  |
| IMPORTANT Text |  |  |
| 10 |  |  |
| 11 | Vertical Clearance of Fixed Spans | Span Fixed vertical clearance fixed vertical clearance value |
|  | Vertical Clearance of Open Spans | Span Opening vertical clearance closed vertical clearance value vertical clearance open vertical clearance value |
|  | Vertical Clearance of Overhead Cable | Cable Overhead vertical clearance fixed vertical clearance value vertical clearance safe vertical clearance value |
|  | Vertical Clearance of Overhead Pipeline | Pipeline Overhead vertical clearance fixed vertical clearance value |
|  | Vertical Clearance of Conveyor | Conveyer vertical clearance fixed vertical clearance value |
|  | Vertical Clearance of structures over navigable water | Building (with in the water = TRUE) vertical clearance fixed vertical clearance value |
|  |  | Wind Turbine (with in the water = TRUE) vertical clearance fixed vertical clearance value |
|  |  | Crane (with in the water = TRUE) vertical clearance fixed vertical clearance value |
|  | Bearing of Navline | Navigation Line orientation orientation value |
|  | Recommended Route | Recommended Route Centreline orientation value |
|  | Deep Water Route Centreline Line | Deepwater Route Centreline orientation value |
|  | Recommended Track | Recommended Track orientation value |
|  | Name and Communications Channel of Radio Calling-In Point | Radio Calling In Point feature name communications channel |
| Other Text |  |  |
| 20 |  |  |
| 21 | Names for position reporting |  |
|  | Name or Number of Buoys | Buoy (all types) feature name |
|  | Name or Number of Beacons | Beacon (all types) feature name |
|  | Name or Number Daymarks | Daymark feature name |
|  | Name or Number Light Vessel | Light Vessel feature name |
|  | Name or Number Light Float | Light Float feature name |
|  | Name or Number Offshore Platform | Offshore Platform feature name |
| 22 | NA (Not Allocated) |  |
| 23 | Light Description String | (various features and attributes) |
| 24 | Note on chart data (INFORM) or nautical publication (TXTDSC) | information language text or fileReference |
| 25 | Nature of Seabed | Seabed Area surface characteristics nature of surface |
| 26 | Geographic Names | Anchorage Area feature name |
|  |  | Bridge feature name |
|  |  | Builtup Area feature name |
|  |  | Building Single feature name |
|  |  | Dock Area feature name |
|  |  | Fairway feature name |
|  |  | Landarea feature name |
|  |  | Landmark feature name |
|  |  | Land Region feature name |
|  |  | Sea Area/Named Water Area feature name |
|  |  | Tideway feature name |
|  |  | Pilot Boarding Place feature name |
| 27 | Value of Magnetic Variation | Magnetic Variation value of magnetic variation |
|  | Value of Swept Depth | Swept Area depth range minimum value |
| 28 | Height of Islet Or Land Feature | Land Area height |
| 29 | Berth Number | Berths feature name |
|  |  | Anchor Berth feature name |
| 30 |  | NA |
| 31 | National Language Text | feature name language (other than English) name |
|  |  | information language text or fileReference |
| 33 | Schedules | Service Hours |
|  |  | Non-standard Working Day |
| 34 | Organisations and Contacts | Contact Details |
| 35-49 | Reserved For IHO |  |
| 50-69 | Mariners' Text, Including Planned Speed etc. |  |
| 70-79 | Manufacturer’s Text |  |
| 80-99 | Future Requirements |  |

Table C-17 - Viewing groups for text

## Viewing group layers used for text

To enable the Mariner to make selections regarding the text visible in the chart display the ECDIS should use the individual text groups defined in the portrayal catalogue(s), and described in Table C-17 collected under distinct text group layers.

For standardization of the ECDIS user interface and to facilitate generic training of mariners, at least the following text group layers must be implemented to control the display of text:

- Important text

- Other text

“Important text” consists of the following :

- Communication channels for calling-in points.

- Vertical clearances for obstructions and structures over navigable water.

- Track or route orientations.

“Other text” consists of all other text.

Product Specifications should conform to these definitions in their Portrayal Catalogues.

An ECDIS may provide more textual on/off controls than are available in this table, but manufacturers should use the names of text groups or layers contained in Table C-17. Descriptive names may be combined but should be clearly indicative of the descriptive names in Table C-17.

EXAMPLE: A manufacturer provides more detailed selections. The table has text group layer “Other text”. The manufacturer may subdivide this category into “names”, “light description” and “all other” (Table C-18).

| Viewing Group Layer | Name of Viewing group layer in the ECDIS | Viewing groups included |
| --- | --- | --- |
| 1 | Important text | 11 |
| 2 | Other Text | 20-49, 0-10 |
| 2.1 | Names | 21, 26, 29 |
| 2.2 | Light description | 23 |
| 2.3 | All other | 0-10, 25, 27, 28, 32-49 |

Table C-18 - Example of subdivision of “Other Text” layer to provide more detailed selections

## Abbreviations

The abbreviations listed below must be used on the ECDIS display. The meanings of the abbreviations must be available to the Mariner through the ECDIS Pick Report.

### Text abbreviations

The abbreviations in Table C-19 are used with text instructions and Lua format specifiers. The normative definition of all abbreviations and their implementation is contained in the relevant portrayal catalogue.

| Abbreviations |  |  |
| --- | --- | --- |
| Prefixes | Suffixes |  |
| bn = beacon (INT1) by = buoy clr = overhead clearance clr cl = clearance closed clr op = clearance open sf clr = safe clearance No = number (INT1) Plt = pilot Prod = offshore production (INT1) LtV = light vessel varn = magnetic variation ch = communication channel NMT = not more than “CLEARING BEARING” NLT = not less than “CLEARING BEARING” | kn = knots (INT1) deg = degrees |  |

Table C-19 - Abbreviations and format specifiers used with text instructions

EXAMPLE: The format specifier “%03.0f deg” printing an orientationValue of 60.1 produces the string “060 deg” (without quotes).

## Additional information in text form

The mandatory formats to be supported by ECDIS are defined in Section C-22.

S-101 datasets encode uncategorised additional information in the complex attribute information, which is bound to feature types and the information type Nautical Information in S-101. Other data products may use the complex attribute textContent, which allows encoding of Internet information resources and descriptions of the sources from which the information is derived. The actual information may be encoded in the form of a text string or the name of a support file in text format that contains the information.

Additional information may be displayed in a hover box (see clause C-15.2.3). The size of the box should not be excessive in proportion to the chart window. The hover box should contain the following content, in order of precedence:

- The content of the headline sub-attribute, if not empty or blank.

- The content of the text sub-attribute, if not empty or blank.

- Full or partial content of the file referenced by the fileReference sub-attribute, if any.

If all the information available in the text and referenced file cannot be contained in the hover box, an element should be added indicating the availability of more information than is displayed, and enabling the mariner to view all the information (for example, “…(more)” suffixed to partial content).

It is not necessary to display content derived from more than one of the sub-attributes headline, text, and fileReference. For example, displaying the headline with a link to access the other attributes will be sufficient.

S-100 allows several different formats for support files which may be suitable for displaying information in text form. Several product specifications restrict this set to text, HTML, and XML.

Rendering of additional text information should conform to the following rules:

- The displayed text must be formatted according to its format type (that is, as plain text, rendered according to HTML format specified in the header, styled according to CSS style rules, etc). This rule implicitly means that embedded markup (such as HTML <br> tags for line breaks) should not be displayed to the user in raw form.

- Only text, HTML or XML can be used for support files providing additional information. JPEG may be used for graphics referenced in HTML files. CSS files may be used for styling. If used by a support file, JPEG and CSS files should be included in the exchange set.

- Any dynamic content (for example embedded Javascript, Flash, DHTML, etc) should be ignored. Including scripts or other dynamic content is therefore disallowed. (There is a case for allowing Javascript etc., to facilitate more function displays on non-ECDIS applications while disabling them on ECDIS, but this is disallowed in this edition for simplicity.)

- Any off-system links should be disabled8. The link should still be shown if it would normally be visible, but as an inactive element.

- Off-system resources should not be accessed, but if the manufacturer has provided local copies, they may be used in place of off-site resources. (Note that this prevents accessing any CSS files not available on the local system.)

- Manufacturers should not attempt to display information encoded in information or textContent attributes, or the content of text support files, that is in any format other than plain text, HTML, or XML. Specifically, PDF files should not be opened. Instead the interface should show an indication of a file type that cannot be opened on the ECDIS, for example, the file type with a “not allowed” icon superimposed (Table C-7). Clicking on this indication should provide an appropriate message to the user, for example “This file type cannot be displayed on an ECDIS, but may be readable on other systems.”

The optional sub-attribute fileLocator is intended to indicate the location of information relevant to the current feature within a larger file, and can be used to automatically position the text viewer at the correct place in a large file. Manufacturers should not assume that the content of filelocator will be human-readable. The content of fileLocator should be interpreted according to Table C-20. If this attribute is present and the file is large enough to require scrolling of the text viewer window/panel, its use is encouraged, to save the mariner time in scrolling through possibly large amounts of text; however, in that case plain text files should be converted to HTML or XML, for which fragment identifiers are part of the respective specifications and more amenable to programmatic use than integer offsets for plain text.

| File Format | Interpretation of fileLocator content |
| --- | --- |
| Text | Integer value of fileLocator: The offset of the start of the section relative to the beginning of the file (the first character in the file has offset 0). This can be used to position the view at a specific place in a long text document. |
|  | Text value of fileLocator: Section heading; clause heading or number; page number, etc. This cannot be used to position the view at the section, clause, page, etc. |
| HTML | HTML fragment identifier: This is the value of the name or id attribute of an HTML element in the file (as defined in the relevant HTML specification). Browsers can use this to position the viewer at a unique location in the document. |
| XML | XML fragment identifier as defined in the relevant specification; for example, the value of an xml:id attribute of an element in the file. As in HTML, this identifies a unique |

Table C-20 - Interpretation of fileLocator attribute

Additional information should be displayed in the current language of the display (the language sub-attribute of information is provided). If information in the current language is not encoded in the data, the English version should be displayed.

Final determination of the presentation type (hover box, side panel, etc.), the proper size of the presentation, its styling, and the arrangement and formatting of its content rests with the manufacturer. ECDIS manufacturers should provide appropriate solutions that enable text information to be displayed without affecting the night vision of the user.

## Graphics

S-101 datasets encode graphical information such as diagrams and photographs in the simple attribute pictorialRepresentation, which is bound to feature types and the information type Nautical Information in S-101. The pictorialRepresentation attribute contains the name of a support file in an allowed graphics format. Other data products may use the complex attribute graphic, which allows encoding descriptive information related to the picture, such as a caption, the date a photograph was taken, the aspect from which it was taken, etc. This is in addition to encoding the name of the picture file in pictorialRepresentation.

Pictorial information may be displayed in a hover box (see clause C-15.2.3) or side panel, or auxiliary display. If a hover box is used, its size should not be excessive in proportion to the chart window, and it should be distanced from own ship, planned track, alerts or indications by at least the track buffer distance. If these guidelines cannot be satisfied, a side panel or auxiliary display should be used. Expansion to the full size of the picture (or available display space in the picture side panel/auxiliary display, if smaller) should be allowed; whether the expanded picture is displayed over the chart window or in a side panel or elsewhere is left to manufacturer discretion. If a picture caption and aspect are available, they should also be displayed with the picture. Other information related to the picture should be accessible to the user via a “further information” link in or next to the hover box or panel.

ECDIS manufacturers must provide appropriate solutions that enable pictures to be displayed without affecting the night vision of the user.

Manufacturers should be prepared to handle pictures of resolution 800 x 800 pixels, in conformance with the guideline for picture files in S-101.

The availability of additional information in graphical format is indicated by the same symbolisation as other supplementary information.

# Miscellaneous Display Elements

This section describes additional display elements and special symbols and considerations for ENC data, including chart furniture, contours, mariners’ features, IMO-required elements, and presentation of updates. Cursor pick reports and information displays in off-graphic panels are described in clause C-15.

## Elements related to data and display scales

### ENC scale

The minimumDisplayScale and maximumDisplayScale of the ENC is the range of scales at which the ENC was designed to be displayed. It may not be the same as the scale of the source data.

### Overscale

Overscale is where the mariner has zoomed larger than the largest maximumDisplayScale of the ENC data that is shown in the mariner’s viewing window.

#### Overscale Indication [UNDER DEVELOPMENT]

The overscale indication is intended to remind the mariner that the size of chart errors is magnified when they increase the display scale.  A 1 mm error at maximumDisplayScale of 1/20,000 becomes a 1.3 mm error at a display scale of 1/15,000 and a 2 mm error at 1/10,000.

The overscale factor must be calculated as [denominator of the maximumDisplayScale] / [denominator of the mariners selected viewing scale], expressed as, for example "X1.3", or "X2" (using the figures in the example above.)

This must be indicated on the same screen as the chart display, and treated as display base. Use colour SCLBR.

This overscale indication is required by IMO Performance Standards (MSC.530(106)) whenever the display scale exceeds the maximum intended viewing scale as indicated by maximumDisplayScale.

NOTE 	If the display is compiled from more than one ENC of the same maximumDisplayScale, and if the mariner deliberately chooses to zoom in so that the display scale exceeds the maximumDisplayScale, then only the "overscale indication" must be shown. The "overscale pattern" (area fill OVERSC01) must not be shown.

### Scale boundary

This shows where the maximumDisplayScale of the ENC data available changes. The ECDIS should warn the mariner of upcoming ENC scale change. Only the major changes in maximumDisplayScale resulting from the scale jumping more than three steps in maximumDisplayScales should be shown. The steps are given in S-101.

The "chart scale boundaries", where the maximumDisplayScale of the data changes, must be symbolised on the ECDIS display by a solid 0.32mm wide line using colour token CHGRD. Alternatively linestyle SCLBDY51 may be used. The display priority is 3; over-radar; standard display; viewing group 21030.

When scale boundaries of smaller scale Data Coverage areas overlap larger scale Data Coverage areas, that portion of the scale boundary which intersects the larger scale Data Coverage area should not be visible.

### Overscale area at scale boundary

ECDIS displays all chart data at the same scale. In order to avoid leaving part of the display blank, the display may be rendered using data from multiple datasets. These datasets may contain Data Coverage areas with varying maximumDisplayScales.

The area fill OVERSC01 must be used to indicate Data Coverage areas displayed X2 or more larger than the maximumDisplayScale; provided that the area was displayed automatically by the ECDIS in order to avoid leaving that portion of the display blank.

NOTE: This rule applies only to the automatic overscaling performed by the ECDIS in matching ENCs at different maximumDisplayScales. It should not be applied to an overscale display deliberately requested by the mariner, which should trigger the overscale indication required by IMO Performance Standard MSC 530(106) section 6.1.1.

A different overscale situation arises when the ship approaches a scale boundary from a larger to a smaller scale ENC, typically when leaving harbour. In combining data from the large scale and the small scale ENCs to generate a display at the larger scale, the ECDIS will have "grossly enlarged" the small scale data.

In addition to drawing the scale boundaries, the "grossly overscale" part of the display should be identified with area fill OVERSC01, as illustrated in Figure C-1.

In this context, "grossly enlarged" and "grossly overscale" should be taken to mean that the display scale is enlarged/overscale by X2 or more with respect to the maximumDisplayScale. For example, at the left edge of Figure C-1 the display scale of 1/12,500 is X4 the maximumDisplayScale of 1/50,000, and so the overscale pattern is required.

Figure C-1 - Illustration of overscale display

[The right hand side of the Figure shows the ENC layout with the screen window overlaid, and the left hand side is enlarged to show the ECDIS display on that screen.]

Note that in this situation the OVERSC01 area fill should only be shown on the area compiled from the smaller scale ENC.  If the area from the larger scale ENC is also overscale, this should be indicated by the "overscale indication". The OVERSC01 area fill should not be shown on the part of the display taken from the larger scale ENC.  For example if the display scale of the situation in the data coverage diagram was 1/3,500 the area of compilation scale 1/12,500 would have an overscale indication of X 3.6 but would have no OVERSC01 area fill.

### Larger scale data available

As the mariner’s display window moves and begins to cover an ENC that is of a larger maximumDisplayScale, the ECDIS must indicate that larger scale data will shortly become available, as required by IMO Performance Standards.

## Graphical indexes

### Graphical indexes of ENCs

The system must be capable of displaying a graphical index of ENCs by optimum display scale.

Without cursor enquiry of the chart area it will not always be clear what optimumDisplayScale applies to a given part of a mixed source display. Display of S-101 requires a graphical index of the optimumDisplayScale of the data to clarify the situation. A graphical index of available S-101 data must be shown on demand.

The system must clearly indicate the product for which the graphical index is displayed. This may be done by any suitable means, such as displaying the product (for example, “ENC Index” or other suitable terminology) in a transparent box in the corner of the display, or in a message area off the chart display area. This requirement applies even if the system is capable only of displaying a graphical index of ENCs.

Only datasets actually installed on the system must be included in the graphical index. Discovery metadata from Exchange Catalogue files in Exchange Sets, or compilations derived from such Exchange Catalogue files, may be used if and only if the Exchange Sets have been verified to contain the claimed datasets (this will normally have been done when the Exchange Set is added to the system).

Listings in any general catalogue listing of datasets, for example S-128 (Catalogue of Nautical Products) information, must be used only for folio purposes.

[Coverage Standardisation of GML – here?]

### Graphical indexes of other S-100 products

The system may implement a capability for displaying graphical indexes of non-ENC data products by the maximum display scale.

If graphical indexes of other products are implemented, the mariner should be able to call up a graphical index of S-100 data products by optimum display scale on demand. Scale-independent products should be included in the index, treated as having optimumDisplayScale=1000 (that is, the lower bound of the set of allowed display scale values for S-101 ENCs, corresponding to scale 1:1000).

The system must clearly indicate the product for which the graphical index is displayed in the same way as for ENCs.

If the system displays graphical indexes for different products simultaneously, the indexes should be distinguishable. For example, dataset extent boundaries in the graphical index may be designed so that the dataset boundaries for one product do not hide the dataset extents of another product; or the system may provide a means of emphasizing the extents for a product selected by the mariner.

As for ENCs, the graphical index should be compiled from the information contained in S-128 (Catalogue of Nautical Products), and only datasets actually installed on the system should be included.

## Limits of data

### Limit of HO S-101 data and data from non-HO sources

The limit of HO S-101 data on the graphical index defines the limit of HO ENC coverage.

The limit of HO S-101 data relative to “no data” areas need not be demarcated on the display. The appearance of the “No data” colour (colour token NODTA) and  NODATA03 area fill (see C-12.3.3) will indicate the end of HO data.

If non-HO chart data is shown on the ECDIS display, its boundary should be demarcated by the line style NONHODAT. Note that the NONHODAT line style is a “one-sided” line and the boundary of the non-HO data must be drawn according to S-101 rules to ensure that the diagonal stroke of the line is on the non-HO data side of the line. The non-HO data boundary indicated by the NONHODAT line style serves to separate ENC data from non-HO chart information.

### Limits of other S-100 product data

To reduce screen clutter, “no data” areas and “non-HO data” boundaries for products other than S-101 and S-57 ENCs should be depicted only when specifically called up by the mariner for a product, and then only if the product is actually displayed on the screen. The fill patterns and boundaries should indicate the product to which they apply, for example by mentioning the product in a corner or embedded in the line as for the graphical indexes (see C-12.2.1 and C-12.2.2).

For gridded data, the no-data area limits do not need to demarcate areas within the grid extent which are covered by fill values.

Implementing the depiction of limits for data other than S-101 and S-57 ENCs is optional.

NOTE: For some types of coverage products (for example gridded data), no-data areas may be easily distinguished by the lack of portrayal. Nevertheless, consistent treatment of products is recommended for the convenience of the mariner - either this clause should be implemented for all the non-ENC S-100 products, or for none of them.

### No data areas

The first action of the ECDIS display re-draw should be to fill the entire screen using colour token NODTA, then fill the entire screen using area pattern NODATA03. These will remain to identify any area not subsequently covered by chart information as a no data area. The NODATA03 area pattern and NODTA colour fill are assigned drawing priority 0 (Table C-9) and viewing group 11050 (Table C-11).

An indication "refer to the official chart" is required if the display includes waters for which no ENC at appropriate scale exists.

If a scale boundary is shown on the display, the information in an overscale area should be identified, and should not be relied on.

An indication that the mariner must refer to the official chart should be given whenever line style NONHODAT appears on the display, or whenever the display is comprised of other than ENC data.

## Data quality indicators

A bathymetric data quality indicator will cover the entire area of depth data or bathymetry for the ENC. This indicator is generated from the S-101 meta-feature Quality Of Bathymetric Data by a rule in the Portrayal Catalogue.

Other indicators of data quality (quality of non-bathymetric information, information about survey dates, positional uncertainty for non-bathymetric data, etc) are derived from other quality meta-features in the S-101 dataset and may be displayed by turning on the appropriate viewing groups (Table C-13) and/or by means of cursor queries (“pick reports”).

Quality indicators for other S-100 products may also be displayed by turning on the appropriate viewing layers (Table C-13) and/or by means of cursor queries (“Pick Reports”) on (meta-)features describing data quality. Quality indicators for products other than S-101 and S-57 ENCs should be displayed only when specifically enabled or called up by the mariner.

## Special ECDIS chart symbols to identify unsafe depths

The ECDIS highlights four features that are important for safe navigation. These are the safety contour, depth shades, the safety depth and isolated dangers

### Safety Contour

The own-ship safety contour, selected by the mariner from among the contours in the System Database, is double-coded by a thick line and a prominent change in depth shade.

If the safety contour selected by the mariner is not available in the System Database, the ECDIS must default to next deeper contour and inform the mariner. If, when the ship moves onto a new chart, the safety contour previously in use is no longer available, the ECDIS must again select the next deeper contour, and inform the mariner.

If the mariner does not select a safety contour, the value must default to 30 metres.

## Other ECDIS symbols and their use

### Mariners Caution Notes

Point cautions and notes entered by the mariner and the manufacturer are distinguished by the colours orange and yellow respectively.

### Unknown feature

A magenta question mark ("?") marks the position of a feature which cannot be identified or for which there is no rule set in the Portrayal Catalogue.

### Change of horizontal (geodetic) datum

The use of non-WGS 84 data does not comply with IHO S-101, and the boundary at which the local geodetic datum changes is not symbolized by the Portrayal Catalogue.

### Manual chart correction

Small orange identifiers are used to distinguish hand-entered chart corrections, which are subject to human error, from corrections entered automatically by electronic means. The original chart feature should not be removed or altered.

### Manual corrections to non-ENC S-100 products

Manual corrections to other S-100 products are entered in the same way as chart corrections. They are  not visually distinguished from manual chart corrections. However, for portrayal purposes they are treated as part of the appropriate product data rather than the S-101 data and are displayed or removed from the display along with the appropriate viewing groups from the relevant S-100 product.

### One-sided linestyle for boundaries

A one-sided linestyle is provided for use on large-scale displays to indicate the side of an area boundary on which the area lies, when only a part of the boundary can be seen on the display.

### Special identifiers

Identifiers are provided for low accuracy chart data; and for ENC features which have additional information for cursor picking utilizing the S-101 attribute information on feature types or the information type Nautical Information that is associated to the feature using the information association additionalInformation. The latter may cause clutter, and should only be displayed temporarily.

Similarly, special identifiers may also be provided for other S-100 products and should provide cursor pick functionality for any associated information type using any information association, since other products may use information types other than Nautical Information and information associations other than additionalInformation. These identifiers should only be displayed on request by the user. The symbols for such identifiers are defined in the respective Portrayal Catalogues.

## Date-dependent features

There are a number of features within the Marine environment, which are seasonal, such as racing buoys. These features are only to be displayed over a certain period, S-101 uses the complex attribute periodicDateRange with the sub attributes dateStart and dateEnd to indicate the periodic nature of the feature. Other features, such as traffic separation schemes, use the complex attribute fixedDateRange with the sub attributes dateStart and dateEnd to indicate their introduction or removal. In order for the Mariner to receive important changes to traffic separation schemes before the event, HOs are required to provide updates or new editions containing the alterations at least one month before they come into force. Any S-101 feature with one of the above complex attributes must not be displayed outside its effective dates unless requested by the Mariner.

### Display of date-dependent features by mariner-selected date

To provide the Mariner with effective route planning capabilities and for the look-ahead function during route monitoring ECDIS must display date dependent chart data based on a Mariner selected date or date range (start viewing date and end viewing date).

During route planning and monitoring the Mariner must be able to select a date or date range to display all date dependent chart features. The display of date dependent information is indicated by the symbol CHDATD01.

All features for which any of the values for the complex attributes periodicDateRange and fixedDateRange are within the Mariner selected date range should be indicated using symbol CHDATD01.

EXAMPLE: A new traffic separation scheme is coming into effect on 01.01.2013, it has been encoded by the ENC producer using the attribute date start (DATSTA). The current date is 12.12.2012 and the Mariner is planning a route that will cross this area over the effective start period.

The ECDIS should be capable of providing the Mariner the ability to set the date the vessel will be in the area (02.01.2013) and the system should show the new traffic scheme.

### Highlighting of date-dependent features

The mariner should have the ability to turn on and off the highlighting of date dependent features.

### Indication of date adjustment

When viewing date or date range do not include the current date, the mariner must be informed by a permanent indication on the chart display that the date has been adjusted. The indication must begin with the text “Display Not Real Time” and the selected date or date range must be readily available. The format of the date should be: dd mmm yyyy = Day, Month, Year; for example, 28 Jan 2014.

The use of one of the following formats is recommended:

- Display Not Real Time – Display is based on date dd mmm yyyy

- Display Not Real Time – Display is based on viewing date range from dd mmm yyyy to dd mmm yyyy

## Scale-dependent features

To reduce screen clutter some features within ENC carry the attribute scaleMinimum to specify the smallest display scale at which they are drawn. At display scales smaller than scaleMinimum the feature is not drawn by the portrayal catalogue. For example, a feature with a scaleMinimum value of 49999, indicating a scale of 1/49,999, must not be drawn on an ECDIS display of 1/50,000.

### Portrayal of Shared Edges.

Shared edges portrayed with dash patterns may be presented incorrectly to the mariner when the ECDIS draws the edge in both directions. This issue can occur along the shared edges of features which abut, such as restricted areas or anchorages where adjacent edges are drawn in opposite directions.

Rendering the dash pattern in both directions can alter the pattern from what is intended, and in some cases may result in a solid line.

In the figure below, note the edges under the green highlight:

| Incorrect presentation | Correct presentation |
| --- | --- |

This issue can also affect complex line styles, for example:

| Incorrect presentation | Correct presentation. |
| --- | --- |

## IMO presentation elements

In some cases S-101 does not provide a symbology instruction in the Portrayal Catalogue that specifies how to present a specific feature on the ECDIS screen. The reason is that such a feature cannot be clearly identified as an S-101 feature class or it appears to be illogical to include it to the Mariners' navigational feature classes.

The following presentation instructions are therefore provided in order to assist the manufacturer to set up a satisfactory and comprehensive ECDIS display.

### Scale bar and latitude scale

The IMO Performance Standards require an indication of scale and range as part of the Display Base. The display scale determines which should be used:

- Case 1: for display scales larger than 1/90,000: always display the 1 Nautical Mile scale bar provided in the Portrayal Catalogue

- Case 2: for display scales at 1/90,000 or smaller: always display the 10 Nautical Miles (at latitude) scale provided in the Portrayal Catalogue.

The scale bar or latitude scale should always be drawn vertically at the left side of the chart display, just clear of the border of the display. The symbols and drawing parameters are described in Table C-21.

The placement should be 3mm in from the border of the display. Make sure the symbol is properly sized by your software to represent 1 nautical mile at the scale of the display (for Case 1) or 10 nautical miles at the scale of the display (for Case 2).

The mariner should be able to remove any labels on the scales to avoid clutter.

| Scale range | Symbol | Drawing priority | Display plane | Display category | Viewing group |
| --- | --- | --- | --- | --- | --- |
| 1:89999 and larger scale | SCALEB10 | 90 | OverRADAR | Display Base | 11030 |
| 1:90000 and smaller scale | SCALEB11 |  |  |  |  |

Table C-21 - Scale bar presentation parameters

EXAMPLE 1: For maximumDisplayScale values larger than 1/90,000 (for example, a scale of 1/50,000) draw symbol 'SCALEB10' on the left side of the chart display, bottom justified and 3mm in from the border of the display. Make sure the symbol should be sized  to represent 1 nautical mile (1852 m) at the scale of the display.

EXAMPLE 2: For display scales of 1/90,000 or smaller (for example, 1/250,000) use symbol 'SCALEB11', similarly located, and scaled to represent 10 miles at the scale of the display.

IEC define a requirement for indicating the location at which the scale is calculated under certain circumstances, as follows:

If the displayed area together with the used projection is such that scale is not uniform over the displayed area then the scale bar (more than 5% difference in uniformity for all directions or displayed area is over latitude 70º) or latitude scale (more than 5% difference in uniformity for latitude direction or displayed area is over latitude 70º) shall indicate the scale either at own ship location or at the centre of the displayed area. In such case a permanent indication “at own ship” or “at centre” shall be close to the scale bar or latitude scale. [IEC 61174:2015].

### North arrow

The IMO Performance Standard requires a north arrow as part of the Display Base. The north arrow should always be shown at the top left corner of the chart display, just clear of the scale bar or latitude scale. Other requirements for the north arrow are defined in IEC 61174.

Use symbol 'NORTHAR1' to indicate true north. Place it in the top left corner of the chart display, on the inner side of the scalebar. Rotate the symbol to true north if the display is other than north up, and make sure it is clear of the scalebar even if the latter extends the full height of the display.

The symbols and drawing parameters for the north arrow are described in the IHO GI Registry.

### Graticule

If the ECDIS shows a graticule (listed in “other information” in IMO Performance Standards (MSC.530(106)) the lines should use the colour token CHBLK.

### Display mode

The ECDIS manufacturer should provide the indication of display mode required in the display base by IMO Performance Standards (MSC.530(106) Appendix 2).

### Shallow water pattern

When the entire water area on the ECDIS display is of less depth than the safety contour, it is not possible for the mariner to easily detect this problem. The issue is exacerbated when the ECDIS is set to night mode due to the small differences between the depth area shades in the ECDIS chart display. Therefore S-101 provides a faint lattice pattern DIAMOND1 to distinguish shallow water.

The mariner must be provided with a selection to turn the shallow water pattern on or off from within the ECDIS.

| Symbol | Drawing priority | Display plane | Display category | Viewing group |
| --- | --- | --- | --- | --- |
| DIAMOND1 | 35 | UnderRADAR | Standard | 23010 |

Table C-22 - Shallow water pattern presentation parameters

### Black level adjustment symbol

Unless the brightness and contrast controls of the monitor are properly adjusted there is a danger that information may be lost from the chart display, particularly at night. Symbol BLKADJ is provided for checking correct adjustment and for re-adjusting as necessary. It should be available for call-up by the Mariner as required.

### Detection and notification of navigational hazards

The IMO Performance Standard for ECDIS MSC.530(106), clause 11.3.5 Route planning states:

A graphical indication should also be given if the mariner plans a route closer than a user-specified distance from a user-selectable category of point objects, such as a fixed or floating aid to navigation or isolated danger..

Clause 11.4.6 Route monitoring states:

ECDIS should give a warning or caution or indication as selected by the mariner and related graphical indication if, continuing on its present course and speed, over a specified time or distance set by the mariner, own ship will pass closer than a user-specified distance from a user-selectable category of danger (e.g. obstruction, wreck, rock) that is shallower than the mariner's safety contour or a user-selectable category of aid to navigation.

Clause 11.4.8 Route Monitoring states:

A graphical indication should be given if the current or the next leg of the selected route goes closer than a user-specified distance from the boundary of a user-selectable category of prohibited area or a geographic area for which special conditions exist

The ECDIS must implement support for the Alert and Indications Catalogue which may be provided within each product’s Portrayal Catalogue.

The safety contour value is set by the user; in the absence of a user setting, its default value must be 30m.

The highlight must indicate the intersection between the ship’s look-ahead buffer (computed using speed, course, look-ahead time and cross-track deviation) and the spatial components associated with alert instructions output by the portrayal.

Figure C-2 depicts indication highlights for points, curves, and surfaces.

Figure C-2 - Examples of indication highlights

### Detection of areas for which special conditions exist

The IMO Performance Standard for ECDIS MSC.530(106), clause 11.3.5 (which applies to Route Planning) states:

A graphical indication should be given if the mariner plans a route closer than a user-specified distance from the boundary of a user-selectable category of prohibited area or geographic area for which special conditions exist (see appendix 4).

Clause 11.4.4 Route Monitoring states;

ECDIS should give a warning or caution, or indication, as selected by the mariner, and related graphical indication if, within a specified time or distance set by the mariner, own ship will pass closer than a user-selected distance from the boundary of a user-selectable category of prohibited area or of a geographical area for which special conditions exist (see appendix 4).

The ECDIS must implement support for the Alert and Indications Catalogue which may be provided within each product’s Portrayal Catalogue.

### Visualisation of the safety contour

The safety contour is defined as an edge between safe and unsafe Skin of the Earth features from S-101.

### Detection of safety contour

The IMO Performance Standard for ECDIS MSC.530(106), clause 11.3.4 Route Planning states;

A graphical indication is required if the mariner plans a route closer than a user-specified distance from own ship's safety contour..

Clause 11.4.3 Route Monitoring states;

It should be possible to select that ECDIS gives an alarm and related graphical indication if, within a specified time or distance set by the mariner, own ship will pass closer than a user-selected distance from the safety contour..

Clause 11.4.7 Route Monitoring states:

A graphical indication should be given if the current or the next leg of the selected route passes closer than a user-specified distance from the safety contour.

The ECDIS must implement support for the Alert and Indications Catalogue which may be provided within each product’s Portrayal Catalogue.

The point, line or surface should be graphically indicated using the presentation named as “DNGHLT” in the Portrayal Catalogue, as depicted in Figure C-3.

Figure C-3 - Examples of danger highlights in ECDIS

### User Selected Safety Contour and Water Level Adjustment.

User selected safety contour means the creation of the safety contour from bathymetric grid data based on a value set by the user. With ENCs the user sets a value for the safety contour, but if the exact fit is not found from the available depth information in the ENC then the safety contour defaults to the next deepest which can be substantially deeper than the value requested by the user.

Bathymetric grid data allows the user the ability to define a value and for the system to delineate areas of safe and unsafe water based on that value. The areas defined can then be used for the definition of safety contour and attendant alerts/indications. When this is done contextual ENC features with depth information are also substituted from the bathymetric grid to present the user a harmonised picture.

The combination of S-101 with S-102 and S-104 together enable Water Level Adjustment (WLA), allowing the water level data contained in S-104 to complement the S-102 and S-101 chart data. As with depth information, WLA processes adjust attribute values in the ENC data to present the user a harmonised picture.

Appendix C-4 defines the detailed mechanisms for user selected safety contour and water level adjustment.

### Range indicator

The IMO Performance Standards require an indication of scale and range as part of the Display Base. ECDIS should implement this as an emphasized marker on the course and speed made good vector (see IEC 62288).

### Detection of route-based conditions

The IMO Performance Standard requires that:

A warning should be given by ECDIS when the ship reaches a specified time or distance, set by the mariner, in advance of a critical point on the planned route. [MSC.530(106) 11.4.12]

Critical points on the planned route are defined in S-421. When the ship arrives within the mariner-specified time or distance of the point, the system must generate an alarm.

The IMO Performance Standard requires that:

An alarm should be given when the specified cross track limit for deviation from the selected route, if defined by the mariner when route planning, is exceeded [MSC.530(106) 11.4.5.]

## Data Producer specified display features

### Supplementary information

Additional chart content in S-101 is encoded using the attributes information and textualDescription on feature types or the information type Nautical Information. This information type is associated to S-101 features using the information association additionalInformation. This information must be discoverable by means of a hover box or user query (Pick Report).

Schedules and contact information may also be encoded in datasets and should also use the same symbol. They are encoded using the information types Service Hours, Non-Standard Working Day and Contact Details, associated to features using the additionalInformation information association.

Other data products may also link regulations and specifications of vessel/cargo characteristics to features in their own Feature Catalogues. Such supplementary information should also be indicated using the supplementary information symbol. The names of the associations are defined in the respective Product Specifications; in general, any information association in a feature instance should be treated as a link to additional information for display purposes on ECDIS.

Display or suppression of supplementary information symbols should be under mariner control (see  MSC.1/CIRC.1609 App. 2 Table 2).

Presentation of the content of information is described in clause C-11.5. Presentation of textualDescription is described in clause C-11.6. Presentation for the content of schedules and contact information is described in clause C-15.5.

### Display in languages other than English

To support dataset portrayal using languages other than English, the ECDIS should expose to the mariner any national language context parameter provided in a products Portrayal Catalogue. For other details of national language support the Data Classification and Encoding Guide (DCEG) of the relevant product should be consulted.

### ECDIS Legend

The ECDIS chart legend containing the following elements should be available for display of values derived from a position selected by the Mariner. Table C-23 indicates which ENC data elements should be used.

NOTE: The legend is currently described in terms of elements of the Dataset Identification (DSID) record in dataset headers in the ISO 8211 encoding as well as dataset discovery metadata because neither the dataset header nor discovery metadata contain all the elements of the legend.

| ECDIS Legend Item | Values |
| --- | --- |
| Units for depth | Axis Unit of Measure (AXUM) subfield in the Coordinate System Axes (CSAX) field |
| Units for height | AXUM subfield in the CSAX field |
| Note: Units for depth and height: although the S-101 ENC Product Specification does not allow any units other than metres for depths and heights, these two elements should be stated for clarity for the Mariner |  |
| Scale of display | Selected by mariner |
| Data quality indicator | - zoneOfConfidence.categoryOfZoneOfConfidence (CATZOC) attribute of the Quality Of Bathymetric Data (M_QUAL) meta-feature. NOTE: When multiple temporal attributes are present: - If a single attribute value is valid for the selected viewing date range, that value should be displayed. If multiple values are valid for the selected viewing date range, the worst-case value should be displayed. NOTE2: When multiple features are present (to indicate bathymetric data quality at various depths): - The feature which intersects the specified safety contour value should be used. (b) Total horizontalPositionUncertainty of the Quality Of Non-Bathymetric Data (M_ACCY) meta-feature if available. |
| Note: Due to the way quality is encoded in the ENC, both values (a and b) should be used |  |
| Sounding/vertical datum | The verticalDatum attribute of the SoundingDatum feature and VerticalDatum feature when available. (verticalDatum attributes of individual features should not be used for the legend.) |
| Horizontal datum | WGS84 |
| Value of safety depth | Selected by Mariner. Default is 30 metres |
| Value of safety contour | Selected by Mariner. Default is 30 metres |
| Note: If the Mariner has selected a contour that is not available in the ENC and the ECDIS displays a default contour, both the contour selected and the contour displayed should be quoted |  |
| Magnetic variation | MagneticVariation (MAGVAR) feature, attributes: referenceYearForMagneticVariation (RYRMGV), valueofAnnualChangeInMagneticVariation (VALACM), and valueOfMagneticVariation (VALMAG) Item should be displayed as: VALMAG RYRMGV (VALACM) For example, 4°15W 1990 (8’E) |
| Date and number of latest update affecting chart datasets currently in use | Issue date and update number from the dataset discovery record (S100_DatasetDiscoveryMetadata) of the last update dataset applied. (See S-100 Part 17) |
| Edition number and date of the ENC | Edition number and issue date from the dataset discovery record (S100_DatasetDiscoveryMetadata) of the current base issue of the ENC dataset. (See S-100 Part 17) |
| Chart projection | Projection used for the ECDIS display (For example, oblique azimuthal). This should be appropriate to the scale and latitude of the data in use |

Table C-23 - Legend elements

The list above is the minimum that should be available, but the complete list need not always be shown. Individual items may be picked by the mariner for display for a period; examples are magnetic variation, data quality for depths, etc.

## Displaying manual and automatic updates and added information

### Manual update

Manual updates of ENC information should be displayed using the same symbology as ENC information and should be distinguished from ENC information as described in the following sub-clauses.

#### Added feature (manual)

Point feature: 	Superimpose symbol CHCRID01.

Line feature: 	Overwrite with line style CHCRID01.

Surface feature:	Overwrite area boundary with line style CHCRID01 and superimpose symbol CHCRID01 on any centred symbol.

#### Deleted feature (manual)

The feature should remain on the display and should be marked as follows:

Point feature: 	Superimpose symbol CHCRDEL1.

Line feature: 	Overwrite with line style CHCRDEL1 (do not remove the original line).

Surface feature:	Overwrite area boundary with line style CHCRDEL1 and superimpose symbol CHCRDEL1 on any centred symbol.

#### Moved feature (manual)

As for deleted feature, followed by added feature.

#### Modified feature (manual)

There are three cases, which are treated as follows:

- If the only modification is an addition (for example, an existing buoy has a retro-reflector added with no other change):

Superimpose symbol CHCRID01 or line style CHCRID01.

- If the only modification is a deletion of a part (for example, an area has a «fishing prohibited» restriction removed), then this creates both a change and a deletion and both should be symbolized:

Point: 	Superimpose symbol CHCRID01 and symbol CHCRDEL1.

Line: 	Overwrite with line styles CHCRID01 and CHCRDEL1.

Area: 	Overwrite the boundary with line styles CHCRID01 and CHCRDEL1 and also superimpose symbols CHCRID01 and CHCRDEL1 on any centred symbol.

- If the modification is an addition and a deletion then it is handled as in (b) above.

A deleted feature should appear on the display only when its IMO category and viewing group are displayed.

A manually updated feature must be capable of the same performance in feature selection, response to cursor-picking, etc, as an ENC feature. In addition, it should provide updating information (identification and source of update, when and by whom entered, etc) on cursor picking.

### Identifying automatic chart corrections on demand

The ECDIS manufacturer should provide a means of identifying chart corrections to the System Database on demand by the Mariner.

On Mariner demand automatic chart corrections of ENC information should be highlighted as described in the following sub-clauses.

#### Added Feature (automatic)

Point object: 	Superimpose symbol CHRVID01.

Line object: 	Overwrite with line style CHRVID02.

Area object:	Overwrite area boundary with line style CHRVID02 and superimpose symbol CHRVID01 on any centred symbol.

#### Deleted Feature (automatic)

Point object: 	Superimpose symbol CHRVDEL1.

Line object: 	Overwrite with line style CHRVDEL2 (do not remove the original line).

Area object:	Overwrite area boundary with line style CHRVDEL2 and superimpose symbol CHRVDEL1 on any centred symbol.

#### Moved Feature (automatic)

As for deleted feature, followed by added feature.

#### Modified Feature (automatic)

Point: 	Superimpose symbol CHRVID01 and symbol CHRVDEL1.

Line: 	Overwrite with line styles CHRVID02 and CHRVDEL2.

Area: 	Overwrite the boundary with line styles CHRVID02 and CHRVDEL2 and also superimpose symbols CHRVID01 and CHRVDEL1 on any centred symbol.

#### Non-HO (Non-ENC) Chart Information

Limited non-HO data added to existing HO ENC data to augment the chart information must be distinguished from the HO ENC information as follows:

Point object:	Superimpose symbol CHCRID01.

Line object:	Overwrite with line style CHCRID01.

Area object:	Overwrite area boundary with line style CHCRID01 and superimpose symbol CHCRID01 on any centred symbol.

#### Information about automatic updates

S-101 defines an Update Information feature to describe automatic updates and corrections. A single instance of Update Information may spatially cover multiple spatially dispersed ENC features. Update Information may also be associated with one or more features using the updatedInformation association to indicate features affected by the update. An Update Information feature can be either a point, curve or surface, and contains a description what has been updated. The presentation parameters are described in Table C-24.

| Primitive | Symbol/Style | Drawing priority | Display plane | Display category | Viewing group |
| --- | --- | --- | --- | --- | --- |
| Point | [TBD] | [TBD] | [TBD] | Other | 31090 |
| Curve | Solid 0.32mm wide line, colour token CHGRD |  |  |  |  |
| Area | Solid 0.32mm wide line, colour token CHGRD |  |  |  |  |

Table C-24 - Presentation parameters for Update Information feature

### Updating non-official chart information

Limited non-official data added to existing official ENC data to augment the chart information should be distinguished from the official information as follows:

Point feature:	Superimpose symbol CHCRID01.

Curve feature:	Overwrite with line style CHCRID01.

Surface feature:	Overwrite area boundary with line style CHCRID01 and superimpose symbol CHCRID01 on any centred symbol.

Non-official data should be distinguished from manually updated chart information, which uses the same identifiers, by cursor picking.

Non-official chart information may be updated by any systematic procedure. A record of updates should be maintained.

The mariner should be able to remove all non-official chart information if the need should arise.

### Other non-official data

Non-official data must be distinguished from manually updated chart information, which uses the same identifiers, by cursor picking.

See clause C-9.1.3 for information on how to symbolize other cases of non-HO data appearing on the ECDIS display.

Non-official chart information may be updated by any systematic procedure. A record of updates must be maintained.

The Mariner must be able to remove all non-HO chart information if the need should arise.

## Contours from datasets other than S-101

Whether algorithms for determining contours from bathymetry, water levels, etc, should be implemented on the ECDIS is undetermined at this time. It is possible that the relevant data products will include contours in (or with) datasets delivered to ECDIS. If contours are appropriate but unavailable in the dataset, manufacturers may devise their own methods for creating contours, but should not substitute the results for contours determined from S-101 ENCs.

Contours computed from other datasets may be displayed as “Other information” in the IMO categorisation, but should be clearly distinguishable from the contours derived from S-101 ENC data. The default method for distinguishing such contours is to use a linestyle with one or both of the following characteristics:

- A line colour that is clearly distinguishable from the colours used for safety, shallow, and deep contours, coastlines, tracks and routes.

- A complex linestyle with embedded or offset symbology identifying the data product (in terms understandable by mariners) or otherwise sufficient to distinguish the contour from safety, shallow, and deep contours, coastlines, tracks and routes.

A distinguishable colour is recommended even if the other characteristic is also used.

The effect on display clutter should be taken into account if display on the navigation window during route monitoring and collision avoidance is anticipated.

### Display and Management of Navigational Warnings

S-124 Navigational Warnings (NAVWARN) portrayal is provided by a portrayal catalogue that includes a symbol set and symbol instructions for the feature and attribute combinations.

### Additional Portrayal requirements of the Graphical User Interface

A dedicated interface is required to provide users with interaction with NAVWARN messages. This interface must, at a minimum, provide functionality for;

- The user must be able to tag individual messages according to the filtering requirements in section 12.2.

- A Capability for an on demand listing of all (S-124) NAVWARN messages in the system, and sorting these according to: received date and time, issue date and time, warning type, producer and series, must be provided. Additionally, a means to list according to user classification should be provided.

- Provide an indication when a new NAVWARN message is received until it has been displayed or 24 hours have passed. This indication may be suppressed if the NAVWARN message does not meet filtering criteria set by the mariner (see 12.2).

- Means must be provided for the operator to enter criteria for filtering of indication of new NAVWARN messages based on time and maximum distance from own ship, monitored route or planned route (see 12.2). Default setting must be no filtering.

- Details of the filtering options that have been enabled by user must be readily available for inspection and modification.

- Means must be provided to view the most recent message, past messages, and to view messages associated with selection of NAVWARN symbols in the graphical display area.

- Listing of all NAVWARN must include means for viewing an abbreviated view of any NAVWARNPart, warningInformation attributes present.

NOTE: It may be possible to create some of this functionality via portrayal context parameters,

### Filtering Navigational Warning information

S-124 navigational warnings datasets are intended for use in S-100 ECDIS as elements of an “always on” layer conforming to S-98 Level 1 interleaving when interoperability is enabled.

There is a risk of clutter with this level of interoperability and it is therefore necessary to include filtering functionality for the user, to aid the removal of non-relevant information from the portrayal.

NOTE: Even though the full extent of a navigational warning may not portrayed, it must still be available and discoverable in a list of NAVWARNs that can be recalled by user action at any time.

User systems should therefore provide filtering mechanisms for the Navigational Warning information.

At a minimum, filtering functionality must be included that allows the user to classify the relevance of a NAVWARN against the intended route as:

- on chart (relevant for the route, must always be visualized), or;

- off chart (not relevant for the route, and need not be visualized), or;

- information (relevant for the route, but for information and need not be visualized).

On chart should be the default classification for all NAVWARNs.

Additional filtering functions may include options such as;

- Filtering on route with a buffer;

- Navigational warning topic;

- Date range of the hazard;

- Valid time of the navigational warning.

These filters could be used to assist the navigator in classifying a NAVWARN according to its relevance for the route.

Example 1: A self-cancelling dataset :

NAVWARNPreamble

publicationTime of 20230704T010000Z

cancellationDate of 20230711T000000Z

NAVWARNPart fixedDateRange of 20230706T010000Z to 20230710T010000Z

is visible on navigation screen during 20230706T010000Z to 20230710T010000Z, unless removed by a filter set by user, and optionally visible during 20230704T010000Z to 20230706T005959Z.

Note: It is still be possible for user to recall cancelled messages for review purposes.

Example 2: Any dangers that are in waters too shallow for the ship get classified as off chart warnings, but are discoverable in the on demand listing of active NAVWARNs.

### Cancelled datasets

When the S-124 dataset is cancelled it must not be displayed on the navigation screen, but should be available for review in the on demand listing of NAVWARNs in the navigation system and marked as cancelled.

### Portrayal of feature classes

The NAVWARNAreaAffected class does not have a portrayal defined to avoid causing significant cluttering on the navigation screen. Rather, the feature must be highlighted by the system if selected from a pick report or by other means for interrogation by user.

When a NAVWARNPart is not portrayed, such as when user selections mark it not to be visualized, any associated TextPlacement features must also not be portrayed.

…...

# Coverages and Time Series

Coverage and time series features are encoded in the HDF5 format (see S-100 Part 10c). S-100 provides for the following types of coverage and time series data:

- Gridded data with different types of spatial grid coverages;

- Triangulated irregular network (TIN) data;

- Data at a set of discrete fixed points;

- Data at a set of moving platforms;

- Time series data at a set of fixed points.

Gridded data will specify either continuous or discrete interpolation between grid points. Data for discrete fixed points, moving platform and time series at fixed points is intrinsically discrete.

## Discrete coverages

Cursor queries within the extent of a grid should produce a Pick Report displaying the attributes and values at the grid data points. The neighbourhood for cursor query should correspond to the visual effects of the portrayal, for example:

- A box of dimensions one-half the grid spacing along each axis.

- The same value and geometry (box, circle, etc.) as for point features in the ENC.

Thinning for display at a smaller scale should also be taken into account, and the result should include data from.

In the absence of a prescription in the Product Specification for how the neighbourhood is to be determined, manufacturers may use any suitable method of their own devising.

## Continuous coverages

Cursor queries within the extent of a continuous coverage feature should produce a Pick Report displaying the attributes and values at the location of the cursor query. The interpolation method specified in the Product Specification should be applied.

In the absence of a prescription in the Product Specification for the values to be returned for a cursor query, manufacturers may determine the values using any suitable method of their own devising, consistent with the requirements of this clause.

## Contours from continuous coverages

The addition of S-102 datasets enhances the Mariner’s ability to render and display, using colours, higher resolution depth zoning directly from the gridded data in S-102.

At time of ingest a display system should delineate navigational depth zones by comparing the depth layer of the S-102 dataset to the Mariner defined vessel draft or default safety contour.

## Transparency

The symbol transparency must be adjusted according to the background chart/image used.

## Thinning

When gridded data is portrayed using discrete symbols at each data point, the effects of scaling the display must be considered. A high-resolution display (that is, zooming in) of regularly gridded data display produces a lower density of data in the visual field. Recommendations in the Product Specification as to whether spatial interpolation may be used to estimate data values at locations between grid points or point coverage locations must be followed.

Displaying at a low resolution (that is, zooming out) increases the density of symbols in the visual field. However, by applying a thinning algorithm, the number of symbols may be reduced.

This Specification requires only that a thinning algorithm be applied to reduce symbol density when the display is scaled. Manufacturers may use the algorithms described below or other algorithms of their own devising.

### Thinning algorithms (informative)

#### Regularly gridded data

Let the grid cell’s diagonal for the unthinned grid at the current display scale be given by D mm. Note that D is dependent on the dataset and the characteristics of the viewing monitor. If every nth cell is displayed, the displayed spacing is nD. Next, suppose the maximum dimension of the largest scaled symbol in the displayed field is Lsmax mm. Then the ratio R of the maximum symbol dimension to the displayed grid spacing is constrained to be less than a prescribed maximum value, Rmax. A typical value for Rmax can be taken to be 0.5. (Given that on a navigation display there may be point features from other products within the extent of the grid, Rmax=0.5 may be too high for practical use; the optimal value of Rmax is left to manufacturer determination, and may be different for different products, depending on the shape of the symbol.)  Then the following inequality must be satisfied for the thinned grid:

R = LsmaxnD < Rmax

If the above inequality cannot be met with increment n equal to 1, then a new value for n is computed by the following formula:

n =1+ fixLsmaxDRmax

Where fix() is a function that returns the truncated integer value of its argument. For plotting, arrows at every nth column and every nth row are drawn, making sure that the row and column with the maximum-size symbol is drawn. The value of n must be calculated by the system. It also requires identifying a “seed point,” a grid point with the maximum-size symbol from which counting starts. Designating this seed point as (x0, y0), the grid points where symbols are drawn are given by:

xk=x0±n×kyk=y0±n×k                for k=0, 1, 2, 3 …

This algorithm assumes the grid spacing can be represented by its cell diagonal. It can be adapted to allow for symbols that are aligned parallel to grid axes or for rectangular cells.

The Figures below illustrate the use of this algorithm. Figure 5 depicts a grid coverage feature symbolised by arrows of dimensions varying according to the value of an attribute at the grid point. Figure C-6 depicts the same data thinned with Rmax = 0.5 (outline-only arrows are suppressed). Figure C-7 depicts the thinning of the same data with Rmax = 0.33. The nominal dimensions and parameters for the three cases are:

Grid spacing at the display scale (D): 36mm (grid diagonal).

Scaling of symbols by data attribute values at grid points produced symbols of four sizes (arrow length):

- L0 = 30.4mm = Lsmax

- L1 = 17.7mm

- L2 = 16.2mm

- L3 = 12.7mm

Figure 5 - Grid without thinning

For Rmax = 0.5, with n = 1, the value of R is greater than Rmax:

R= Lsmax1×D = 30.436 =0.844 >0.5

Applying the formula for computing n:

n=1+ fix30.436×0.5=2

Counting in row-major order from the grid origin (0,0), the first symbol of size Lsmax is located at (1,1). Denoting this point as the seed point (x0, y0), the only grid points where symbols are displayed are:

xk = x0 ± 2×kyk = y0 ± 2×k           for k=0, 1, 2, 3 …

The results are depicted in Figure C-6.

Figure C-6 - Grid thinned with Rmax = 0.5

For Rmax = 0.33, the formula for n yields:

n=1+ fix30.436×0.33=3

and

xk =x0 ±3×kyk=y0±3×k            for k=0, 1, 2, 3…

The results are shown in Figure C-7.

Figure C-7 - Grid thinned with Rmax = 0.33

The algorithm may hide significant characteristics of the data; for example, counting displayable rows and columns starting with the first instance of a maximised symbol may suppress significant information in nearby grid points and produce the wrong overall impression. In Figure C-6, row 2 would be suppressed even if all the data points in that row are of the same size as the symbol at (1,1) — this would suppress more data points with scaled-up symbols, which may represent data of more significance to the Mariner.

Grid diagonal as a measure of cell spacing is more suitable for grids where cell dimensions along both axes are approximately equal in display units (that is, in millimetres at the display scale); if there are significant differences, the D and Rmax parameters will need to be different for the two axes.

Execution of this algorithm would be faster if the grid coordinates of the seed point (x0, y0) are known in advance, either encoded by the producer as instance metadata, or calculated when the dataset is ingested into the system. S-100 does not yet provide a standard way of encoding this information.

Manufaturers may extend or adapt this algorithm in various ways, for example:

- Adapt Rmax to the shape and proportions of the symbol (that is, its perceived effect on the display).

- For grids with cells whose dimensions in display units are very different along different axes, use different D and Rmax parameters for the two axes, giving different values of n for different axes.

- Pre-compute and cache the scale values where n changes, so that suppression or revelation of symbols can be determined by the scale of the display.

- Adapt the determination of the seed point to show as many significant values as possible.

In order to avoid confusing the mariner, reasonable consideration should be given to generally maintaining the regular appearance of the grid coverage, though some irregularity is probably unavoidable with greater thinning.

## Temporal variation

The metadata variables related to time are dateTimeOfFirstRecord, dateTimeOfLastRecord, timeRecordInterval, numberOfTimes, timeIntervalIndex, timePoint, startDateTime, and endDateTime (see S-100 Part 10c). The time selected for display (that is, past/present/future) will typically not correspond exactly to the timestamp (metadata variable timePoint) of the input data. For a correct display, the ECDIS will have to select the correct data.

For data with only a single timed record (where the timestamp of the earliest value equals that of the latest value) such as real-time data, the data values are displayed only if the display time is later than the timestamp and the absolute time difference between the display time and the data timestamp is less than a discrimination interval (for example 5 minutes). For a single record, the variable timeRecordInterval  can be used to set the discrimination interval.

For data with multiple times, if the selected display time is later than the first timestamp and earlier than the last timestamp, or the selected display time is in the interval [startDateTime, endDateTime], then the closest but immediately preceding values in the data are displayed. However, if the selected display time is earlier than the first timestamp, or outside the interval [startDateTime, endDateTime] then the data is not displayed. If the selected time is later than the last timestamp or after endDateTime, then data values at that time are displayed only if the absolute time difference between the display time and the data timestamp is less than a discrimination interval (for example the value of the variable timeRecordInterval).

Some data change rapidly, so more-or-less continual revision or updating of such data is essential. For real-time observations, new values are periodically collected (for example, on the order of once every 5 minutes).

For a forecast, the entire field may be created one or more times per day.

New issues of real-time observations or forecasts may not be considered as new editions, but as new datasets. New datasets may be distinguished by a unique datetime in the file name.

New editions may be encountered in predicted time series data.

Other data, such as tidal atlas or harmonic constant data are updated much less often, typically on an annual basis.

The system should check for the availability of new data at a frequency that matches the update frequency.

# Colours

## General

In Portrayal Catalogues, colours are specified in one or both of CIE (Commission Internationale de l'Eclairage) or sRGB colour space coordinates. CIE colour specifications use xy chromaticity coordinates and luminance L. The sRGB colour space (IEC 61966-2-1) defines colours in terms of the chromaticities of red, green, and blue primary colours.

The ECDIS colour scheme is based on specification of colour tokens and color conversion tolerances.

Note that these colour specifications apply to both the operational chart display (for route planning and route monitoring), and also to any text on the same screen as the chart display.

## Selection of colours

Since chart and navigation lines and symbols must show clearly against the background shades, the colour palettes were constructed by first selecting the background area shades and then selecting colours for lines and symbols that contrast with their background. In selecting foreground colours for point and line features, lines and symbols, the aim has been to highlight important information by giving it greater contrast with the background.

In order to accommodate the very large change in bridge lighting between bright sun and dark night, the colours switch from a light background with dark foreground details, which has been found to give the best contrast under bright sun, to a dark background with light foreground details by night. The night display has to be dim enough that it can be viewed without impairing the Mariner's night vision. The Dusk table is also a black-background table, for optional use by day as well as at twilight.

The design of both colours and symbols has concentrated on ensuring that important chart and navigation features remain clearly visible under the extremes of bright sun and dark night viewing.

## Colour palettes implemented by the portrayal catalogue

There are three colour palettes, all of which should be made available to the Mariner. They are specified as part of the colour profile file(s) in the Portrayal Catalogues. They are as follows:

DAY	The "Day" palette uses a white background as a result of a comparative test outdoors in bright sunlight which showed that a display background of maximum luminance gives the best contrast achievable under near-washout conditions. This conclusion has been confirmed by subsequent sea experience.

DUSK 	The "Dusk" colour palette is based on using a black background; this palette may also be used by day as a Mariner's option.

NIGHT 	The “Night” colour palette is intended for nighttime use. At night the light emitted by the display must be strictly limited to avoid impairing night vision. In case the luminance needs to be further reduced, the "Night" colour palette may be augmented by a luminance-reducing neutral density filter which should have 8 times attenuation, designated (logarithmically) "0.9 ND". (This is a manufacturer's option.)

## Transparency

### Use of transparency (informative)

Transparent area colour fill is used for the following purposes:

- So that the background colours, lines and symbols show through an area shade (for example, depth shades and contours should show through a traffic separation zone);

- To reduce the prominence of a large symbol (for example, too prominent a centred anchorage area symbol would cause clutter on the display);

When interoperability processing is activated, interoperability rules should take precedence over the purposes above in case of conflict; but if transparency for the above purposes can be used without overriding an interoperability rule, it should be used.

## Colour assignment for navigation systems

### Introduction to colours (Informative)

Since chart and navigation lines and symbols must show clearly against the background shades, the colour tables were constructed by first selecting the background area shades and then selecting colours for lines and symbols that contrast with their background. In selecting foreground colours for point and line features, lines and symbols, the aim has been to highlight important information by giving it greater contrast with the background.

In order to accommodate the very large change in bridge lighting between bright sun and dark night, the colours switch from a light background with dark foreground details, which has been found to give the best contrast under bright sun, to a dark background with light foreground details by night. The night display has to be dim enough that it can be viewed without impairing the mariner's night vision. The Dusk table is also a black-background table, for optional use by day as well as at twilight.

The design of both colours and symbols has concentrated on ensuring that important chart and navigation features remain clearly visible under the extremes of bright sun and dark night viewing.

### Colour assignments

The general colour assignment for navigation systems is provided below.

| Colour | Usage |
| --- | --- |
| black/white | (black by day / white by night) is used for critical navigation features which need highlighting by contrast against their background to give them adequate prominence. Examples are the own-ship symbol, dangerous soundings equal to or less than the safety depth, buoys, conspicuous objects on land etc. It is also used for text, which is less clear in any other colour. |
| white/black | (white by day / black by night) as a background area shade is used for deep, safe, water. |
| magenta | is used to highlight critically important features such as isolated dangers, traffic routes, anchorages; and for restricted areas, submarine cables, gas pipelines etc. It is also used for aids to navigation and services such as daymarks, racons, and pilot stations. |
| grey | is used for many features which are black on the paper chart. It is used with thick lines for critical physical objects such as bridges and overhead cables, and with thin lines for important but less critical physical features such as non-dangerous soundings, sandwaves, overfalls, water pipelines and fish farms. It is similarly used for chart features such as fairways, harbour areas, tidal information and for information about the chart such as quality of chart data, overscale areas, etc. |
| grey | as a background area shade is used with a prominent pattern for no-data areas. |
| blue | as a background area shade is used to distinguish depth zones. |
| blue | as foreground colour for AIS and VTS information; also reserved for future requirements. |
| green | is used for the radar image and synthetics, and for buoy and lights colours. |
| blue-green | is used for transferred ARPA. |
| yellow-green | ('moss-green') as a background area shade is used for the intertidal area between high and low waterlines. |
| yellow | is used as the manufacturer's colour; for the Mariner's transparent colour fill; and for buoy and lights colours. |
| red | is used for the important planned route, for the Mariner's danger highlight, and for buoy and lights colours. |
| orange | is the Mariner's colour, for notes, chartwork, chart corrections. The scale bar, north arrow, and Mariner's navigation objects such as EBLs and VRMs are also orange. |
| brown | as a background area shade is used for the land, and dark brown is used for features on land and in the intertidal area that do not have any strong significance for navigation. |

Table C-26 - General colour assignments

## Colour tokens, profiles and palettes [Informative]

Colour tokens and colour profiles should be registered in the Portrayal Register in the IHO GI Registry prior to use in Portrayal Catalogues. The same colour tokens may be used by different data products. Product Specifications should use the same colour tokens as S-101 for feature types and sub-types that are conceptually similar to S-101 feature types.

The colours in this section are specifically designed for chart display.

Table C-27 lists colour tokens used in ECDIS displays and describes their roles and significance. This table is not a comprehensive list of colour tokens; see the colour profile files in the Portrayal Catalogues of individual products for the full list of colour tokens used by each product.

NOTE: This table is useful information for implementations and should be available to implementors for completeness. It should also be kept updated with information pertaining to other Product Specifications. It is provided as part of this edition because there is at present no apparent provision for publishing it as a “living document”. Future editions may replace it by a reference to a “living document” in the IHO GI Registry or on the IHO Web site.

| Token | Colour, day/night | Usage | Significance |
| --- | --- | --- | --- |
| Colours for chart features |  |  |  |
| CHBLK | black/grey | general | This selection of colours is used in general to design symbols and chart line features as well as fill styles. They are not used in cases where other colours are available for a special usage. |
| CHGRD | grey dominant | general |  |
| CHGRF | grey, faint | general |  |
| CHRED | red | general |  |
| CHGRN | green | general |  |
| CHYLW | yellow | general |  |
| CHMGD | magenta, dominant | general |  |
| CHMGF | magenta, faint | general |  |
| CHBRN | brown | general |  |
| CHWHT | white | general |  |
| OUTLW | black | symbol outline on sea area background | These colours are used to outline symbols depending on which background they are normally shown (water/land). |
| OUTLL | pale/dark brown | symbol outline on land area background |  |
| LITRD | red | red lights | Light symbols have their own colours to give the opportunity to influence their colour luminance individually. Yellow (LITYW) is used for white, yellow, orange and amber lights because it might be difficult to distinguish these colours from each other on a badly calibrated monitor. It also follows the tradition to show up white lights with a yellow flare or coloured arc. |
| LITGN | green | green lights |  |
| LITYW | yellow | white/yellow/orange/amber lights |  |
| ISDNG | magenta | isolated danger | Since the isolated danger symbol forms one of the most important items on the ECDIS screen, it is given a separate colour. |
| DNGHL | red | danger highlight | This colour is used for symbology that highlights Mariner selected dangers. The Mariner decides during route planning which features are highlighted by this colour. |
| TRFCD | magenta, dominant | traffic control features | Traffic separation schemes are complex chart features. The navigator is confronted with important elements of the schemes and with less important elements as well. TRFCD is used to distinguish important traffic routeing features. |
| TRFCF | magenta, faint | traffic control features |  |
| LANDA | brown | Land areas | This colour is used for land areas in general. |
| LANDF | brown | Landforms, land features | Landforms and land features are given a contrasting brown. |
| CSTLN | black/grey | Coastline, shoreline constructions | The coastline is a very important feature of the chart. If a radar image is combined with the chart picture it is required that coastline elements clearly show up on top of the green radar picture (see also RADHI/RADLO). To have full control over this combination under all conditions (day/night) a separate colour is reserved for coastline features. |
| SNDG1 | grey | deep soundings > safety depth | This colour is used for soundings that are deeper than the selected safety depth ("safe" soundings). |
| SNDG2 | black/white | shallow soundings <= safety depth |  |
| DEPSC | grey | safety contour | This colour is reserved for the selected safety contour. |
| DEPCN | grey | depth contours | All depth contours other than the safety contour should use this colour. |
| DEPDW | white/black | deeper than selected deep contour | These are depth shades. The depth zones are: DEPDW: areas deeper than the Mariner-selected deep contour; DEPMD: areas between deep contour and the Mariner-selected safety contour; DEPMS: areas between safety contour and the Mariner-selected shallow water contour; DEPVS: areas between shallow water contour and the low water line (zero meter contour); DEPIT: areas between zero meter contour and coastline (intertidal). For route monitoring it may be desirable to distinguish only two water shades, plus DEPIT: deeper than own ship's safety contour and shallower than safety contour. In that case DEPDW and DEPVS should be used. At night it may be difficult to distinguish between DEPMD and DEPDW. |
| DEPMD | pale/dark blue | safety contour to selected deep contour |  |
| DEPMS | light/medium blue | shallow contour to selected safety contour |  |
| DEPVS | medium/light blue | zero meter contour to shallow contour |  |
| DEPIT | yellow-green | high water line to zero meter contour |  |
| Radar image overlay colours |  |  |  |
| RADHI | green | high intensity echo or single intensity echo | The radar image overlay can be generated by using either one intensity colour or a range of intensities. The colour for high echo intensity (RADHI) should be used where only one intensity is used. If you prefer to show more than one echo intensity or fading target trails, the corresponding colour intensities should be interpolated between the colour for high echo intensity (RADHI) and the colour for low echo intensity (RADLO). Optionally, the manufacturer may vary the radar green overlay by making it transparent. |
| RADLO | green | low intensity echo & target trail |  |
| ARPAT | green, dashed | ARPA, target symbols & information | Used for ARPA targets and information tagged on them. |
| Mariners’ and navigation information colours |  |  |  |
| SCLBR | orange | scalebar | Used to generate the scalebar. |
| CHCOR | orange | chart corrections | Hand entered chart corrections are marked by the colour. |
| NINFO | orange | Navigators Notes | Mariners' notes of any form (Symbols, Text) are generated using the colour. |
| ADINF | yellow | Mariners' transparent area fill and manufacturers' points and lines |  |
| Ships symbol and planned route |  |  |  |
| SHIPS | black/white | own ship, Course & Speed Made Good vector |  |
| PSTRK | black/white | Past Track |  |
| SYTRK | grey | Secondary Track |  |
| PLRTE | red | planned route & notations |  |
| APLRT | orange | alternate planned route |  |
| Other colours |  |  |  |
| RESBL | blue | AIS features and symbols |  |
| RESGR | grey | reserved for line features & screened areas |  |
| BKAJ1 | black | black level test symbol background |  |
| BKAJ2 | grey | black level test symbol foreground |  |
| TRNSP | 100% transparent | transparent | This means a 100% "transparent" colour. This is not a "real" colour since it is invisible. Every pixel on the screen, which has the colour value 0 shows up as 100% transparent. In case the pixel was already painted with another (visible, for example black) colour this colour is not overwritten by the transparent colour. In case the pixel was cleared before or not yet painted the "background" colour shows up (contrast to NODTA). |
| NODTA | grey | Areas without chart data | This colour shows up on every pixel on the screen, which is neither covered by chart features nor covered by other elements of the ECDIS display (for example radar overlay, user interface). Thus, it can also be called the "empty background colour" (contrast to TRNSP). |
| CURSR | orange | Cursor | In most graphic systems the cursor is treated as an item that can be handled completely independent from the graphic of the chart area. Therefore the cursor is given its own colour and it is kept separately from the other sections of the colour scheme. The cursor colour is also used by variable range marker (VRM), electronic bearing line (EBL), parallel indexing lines and other tools to perform absolute and relative measurements in the chart. |
|  |  |  | [To be added - significant colour tokens for other “important” data products, taken from the Product Specifications, if they are defined there.] |

Table C-27 - Significance of colour tokens for ECDIS displays

The active colour palette detemines the colour coordinates assigned to colour tokens in any particular display. Colour palettes for day, night, and dusk viewing conditions are required for S-101 and any other S-100 product intended for viewing on the navigation display.

# Cursor Pick Reports and Displays in Interface Panels

ECDIS must be capable of performing spatial queries on ENC and other data during import and symbolisation. Spatial query is understood as the ability to inspect graphical position and numerical value of spatial coordinates associated with a charted feature. Spatial query could be available as by means of cursor pick or as an independent function.

It should be possible to step forward or backward in spatial chains.

## Cursor pick rules

The rules below must be applied to all ECDIS cursor Pick Reports.

- Full feature and attribute names should be displayed.

- Spot selected on chart pane should be marked by symbols from IEC 62288.

- Listed value names should be displayed.

- There should not be any padding of attribute values, for example, a height of 10 metres should not be padded to 10.000000 metres as this could potentially confuse or mislead the Mariner.

- Units of measure should be included after all attribute values which are weights or measures.

- Cursor enquiry should extend to include both information associations and feature associations, including aggregations and compositions, which may link to information types or features that carry additional information such as a featureName.

- Dates should be given in the form “Day Month Year” DD-MMM-YYYY.

Month abbreviations should be: JAN, FEB, MAR, APR, MAY, JUN, JUL, AUG, SEP, OCT, NOV, DEC.

- The Pick Report should only return information about the visible features on the ECDIS display. If the viewing group is turned on all features even no symbol features without visible presentation within that viewing group are available via the cursor Pick Report.

- Cursor enquiry should extend to the uncertainty of the spatial object, including attributes qualityOfHorizontalMeasurement, verticalUncertainty, and horizontalPositionUncertainty from an associated Spatial Quality information type.

- The Pick Report should indicate the data product from which the information is extracted. Descriptive data product names or abbreviations as should be preferred if possible (for example, “ENC” or “Electronic Navigational Chart”, not “S-101”). The Dataset Identification in the Product Specification and dataset discovery metadata in the Exchange Catalogue should contain this information.

Data from different products and viewing groups should be organized to facilitate navigation through complex reports in a manner logically consistent with the layering of products and groups on the screen.

## Pick Report descriptions

A plain language explanation of each symbol is included in the Portrayal Catalogue. This gives the Mariner quick and understandable information which is not always obvious from the feature class and attribute information. The manufacturer should always provide explanations to the Mariner in response to a cursor pick on the symbol.

Attribute values provided in addition to the above explanation should be connected to their meaning, and the definitions should also be available. The definitions are included in the XML Feature Catalogues for the data products.

### User defined cursor pick parameters

The Mariner should be able to configure the content displayed in the Pick Report.

## Visibility of feature attributes.

All Attributes and subattributes in feature catalogues have a visibility defined by the field S100_FC_AttributeVisiblity, with allowable values of public, private and protected visibility. Attributes with privateVisibility should not be shown to the user via the Pick Report. Attributes in the feature catalogue without a defined visibility or with other values have a public visibility and may be seen by the user in Pick Reports.

### Sorting order of results

The Pick Report must be organized to facilitate navigation through complex reports in a manner logically consistent with the layering presented on the screen.

The order of features must correspond to the drawing order. Features must be prioritized using the top-most drawing instruction associated with the feature. Only active drawing instructions which intersect the pick area should be evaluated during sorting.

Results must be sorted by display plane, then within each display plane by drawing priority. When the drawing priority is equal the geometric primitive must be used to order the information, points followed by lines and finally areas.

### Hover-over function

OEMs may wish to include hover-over functions for Mariners to access important charted feature details without having to select a Pick Report. If this function is implemented within an ECDIS, the Mariner should be able to configure the system to turn hover-over functionality on and off.

The hover-over function should only be used when indicated by a drawing instruction.

### Unknown attributes

When the ECDIS encounters an unknown attribute not present in the product’s Feature Catalogue its value must be available via the ECDIS cursor pick.

### Interoperability between datasets

The attribute interoperabilityIdentifier has been introduced to enable features within a dataset to refer to features contained within other datasets conforming to different product specifications. When a feature is being accessed via pick report, and it contains the attribute interoperabilityIdentifier, then any other features in the cursor query with the same value of interoperabilityIdentifier should also be included in the pick report details listed for the end user.

## Cursor queries on coverage data

Cursor queries on continuous gridded coverage data must report the coverage feature’s data values corresponding to the spatial location indicated by the cursor query. The data values reported should be those at the nearest data point for discrete coverages,.

For discrete coverages, the data point from which the result is reported should be highlighted. Highlighting grid data points is not required for continuous coverages.

Depictions of results may be customized by the product specification or manufacturer, provided important information in other parts of the display is not obscured. For example, time series information may be depicted using a graph.

See clauses C-13.1 and C-13.2 for more information on methods and rules for cursor queries on coverage features.

## Tidal stream data panels

Tidal stream information in the form of speed and direction at intervals relative to the time of a “reference tide” (generally high or low water) should be formatted for display in the ECDIS cursor Pick Report (or other UI panel). Table C-28 shows the template that should be used for displaying the values The table layout may be modified depending on the modeling – for example, the "Rates” column may be split if both springs and neaps rates are both provided for the same direction instance, or values for different reference tides may be provided in different tabs; however, the reference tide type must be shown. The “Hours” values should be the actual values in the timeRelativeToTide attribute.

| Tidal Station: (station name) |  |  |  |
| --- | --- | --- | --- |
| Tidal Station Identifier: (station identifier) |  | Data From: (data product, from dataset metadata) |  |
|  | Hours | Direction of stream (degrees) | Rates (knots) |
|  |  |  |  |  |  | springs |  |
| Before | -6 |  |  |
|  | -5 |  |  |
|  | -4 |  |  |
|  | -3 |  |  |
|  | -2 |  |  |
|  | -1 |  |  |
| (reference tide) | 0 |  |  |
| After | +1 |  |  |
|  | +2 |  |  |
|  | +3 |  |  |
|  | +4 |  |  |
|  | +5 |  |  |
|  | +6 |  |  |

Table C-28 - Template for tidal stream values

Table C-29 below relates an exemplary Tidal Stream Panel Data feature and its attributes to its display.. Complex attributes are in italics and encoded values are in blue text.

| Tide Stream Panel Data (feature) |  |  |  | Example of ECDIS depiction |
| --- | --- | --- | --- | --- |
| Station name |  | PLYMOUTH (DEVONPORT) |  | (If values for neaps are provided in a separate tidal stream panel complex attribute in the same feature, a similar table for neaps must be provided in the UI.) |
| Station number |  | 0014 |  |  |
| Tide stream panel values |  |  |  |  |
| Reference tide | high water |  |  |  |
| Reference tide type | springs |  |  |  |
| Tide stream value | Orientation | Orientation Value | 113 |  |
|  | Time relative to tide |  | -6 |  |
|  | speed maximum |  | 0.1 |  |
| Tide stream value | Orientation | Orientation Value | 332 |  |
|  | Time relative to tide |  | -5 |  |
|  | speed maximum |  | 0.6 |  |
| Tide stream value | Orientation | Orientation Value | 331 |  |
|  | Time relative to tide |  | -4 |  |
|  | speed maximum |  | 1.1 |  |
| Tide stream value | Orientation | Orientation Value | 342 |  |
|  | Time relative to tide |  | -3 |  |
|  | speed maximum |  | 1.0 |  |
| Tide stream value | Orientation | Orientation Value | 347 |  |
|  | Time relative to tide |  | -2 |  |
|  | speed maximum |  | 0.7 |  |
| Tide stream value | Orientation | Orientation Value | 333 |  |
|  | Time relative to tide |  | -1 |  |
|  | speed maximum |  | 0.5 |  |
| Tide stream value | Orientation | Orientation Value | 317 |  |
|  | Time relative to tide |  | 0 |  |
|  | speed maximum |  | 0.3 |  |
| Tide stream value | Orientation | Orientation Value | 178 |  |
|  | Time relative to tide |  | 1 |  |
|  | speed maximum |  | 0.3 |  |
| Tide stream value | Orientation | Orientation Value | 146 |  |
|  | Time relative to tide |  | 2 |  |
|  | speed maximum |  | 0.6 |  |
| Tide stream value | Orientation | Orientation Value | 140 |  |
|  | Time relative to tide |  | 3 |  |
|  | speed maximum |  | 1.0 |  |
| Tide stream value | Orientation | Orientation Value | 143 |  |
|  | Time relative to tide |  | 4 |  |
|  | speed maximum |  | 1.1 |  |
| Tide stream value | Orientation | Orientation Value | 143 |  |
|  | Time relative to tide |  | 5 |  |
|  | speed maximum |  | 0.8 |  |
| Tide stream value | Orientation | Orientation Value | 138 |  |
|  | Time relative to tide |  | 6 |  |
|  | speed maximum |  | 0.3 |  |

Table C-29 - Example of tidal stream panel data and its ECDIS display

## Schedules, contact Information, and vessel characteristics

[Templates to be developed.]

## Special Pick Report formats

Some Product Specifications may require or suggest specific layout and contents for Pick Reports (examples are shown in Figure C-8). Since there is at present no generic format for defining such custom Pick Reports, manufacturers will need to develop their own solutions.

Figure C-8 - Examples of special formats: Water level at a station (L) and graphic showing time series of water level data (R)

## DELETED (duplicate)

# Alerts and Indications

IMO Resolution MSC.530(106) states in 11.3 and 11.4 and their sub-paragraphs how an ECDIS should respond to risk of crossing, dangers, prohibited areas or areas with special conditions, during route planning (11.3) or route monitoring (11.4). Appendix 4 and Appendix 5 of the same resolution provide details about the areas for which an ECDIS should detect incursions and provide an alert or indication.

All Product Specifications which are intended for navigation should specify any feature combinations that match one or more of the areas for which alarm or indication should be given to ensure there is a harmonized implementation of alerts and indications in navigational products. Such specification could be done using a machine-readable Alerts and Indication Catalogue.

# Use of Context Parameters

The Portrayal Register in the IHO GI Registry contains a list of predefined context parameters used in S-100 Portrayal Catalogues. Manufacturers may add additional parameters, but should not change the names, roles, or data types or predefined parameters, or substitute other parameters of the same scope.

The Portrayal Catalogue for a product contains the set of known parameters which may be used in the portrayal processing for that product. This set may be different for different products. Context parameters are used for passing portrayal-related configuration information and user settings to portrayal processing, and the values of context parameters may therefore be changed by user functions, including those defined in the IMO Performance Standards, in this document, or manufacturers’ custom user functions.

In order to be able to use the official Portrayal Catalogue for a data product, manufacturers must implement all the context parameters listed in the Portrayal Catalogue. The context parameters used in a Portrayal Catalogue are listed in the <context>…</context> section of the Portrayal Catalogue XML file.

Context parameters may be added or removed by updates to the Portrayal Catalogue. Context parameters which cannot be automatically associated with ECDIS display functions or user interface features must be exposed to the Mariner for use.

## Context parameters for ECDIS functionality

Context parameters are used in Portrayal Catalogues for implementing mandatory and optional ECDIS functionality mandated by the IMO Performance Standards. Since they are used by rules in the IHO Portrayal Catalogue, they should be implemented without change by manufacturers in order to maintain compatibility with the IHO Portrayal Catalogue.

# Dual-fuel systems

Dual-fuel systems are systems that use both older (pre-S-100) and newer (S-100) data products that contain the same type of information (for example, S-57 and S-101 ENCs).

All stakeholders should anticipate a transition period during which new S-100 formats increasingly replace older formats.

An S-100 ECDIS must be able to handle both S-57 and S-101 ENCs during the transition period.

## Display of data available in both new and legacy formats

The question of handling simultaneous display of data in old and new formats is still to be addressed by the IHO at the time this document is being written.

-

## Display of additional information layers

The display of additional information layers is generally driven by mariner need. The Interoperability Catalogue concept for ECDIS (see S-100 Part 16 and S-98) is based on using S-101 ENCs as the base layer. Therefore, in areas without S-101 ENC data, manufacturers and data producers should expect additional information layers to be displayed as overlays over S-57 ENC data.

The converse is also allowed - data in legacy formats may be displayed as overlays over S-101 data.

In both cases, the applicable requirements in the IMO Performance Standards must still be met, especially requirements about not degrading the route monitoring display.

## Concurrent applicability of S-52 and S-57

Dual-fuel capable systems should continue to use the principles defined in S-57 and S-52 for the presentation of chart data that conforms to S-57 instead of S-101. The principles in S-101 and this Annex should be concurrently applied where S-101 data is displayed. This includes the case where one part of the chart window has S-101 data as the chart layer and another has S-57 data as the chart layer - S57/S-52 apply to the portion where S-57 data is the chart layer and S-101 and this Annex apply where S-101 data is the chart layer.

Overlays of S-100-based non-ENC data over S-57 chart data are left to manufacturer discretion, pending development of guidance by the IHO. Where S-100-based non-ENC datasets are overlaid over S-57 data, they should be treated as “additional information” in the sense of IMO MSC.530(106), including the requirement to avoid degrading the display of ENC information. Activation of the new functionalities described in this Annex for screen regions where S-100 data is overlaid over S-57 ENCs is left to manufacturer discretion, pending the development of guidance by the IHO. For example, S-98 interoperability need not be implemented in the absence of an S-101 base layer, but graphical indexes can still be displayed for non-ENC S-100 products and thinning for an S-111 (Surface Currents) grid can still be applied even if it overlays S-57 data.

MSC.530(106) and other applicable IMO standards continue to apply to the “S-57 portion” of the display, and, in the absence of updated IMO standards specifically for S-100-based data, also to the “S-101 portion” of the display. The IEC standards should continue to be used for testing, with necessary extensions to test for the “S-101 portion” of the display, pending IEC update of the IEC standards for S-100-based displays and this Annex.

This Annex is designed to allow such integrated displays. Note in particular:

- The IMO functions continue to be used. This Annex describes their applicability to S-100-based data and defines additional functions for S-100-based data. Any additional functions that apply only to S-100-data should be deactivated for S-57 data.

- The IMO display categories and viewing groups for S-57 data are carried over into this Annex and include the same feature information for each group as in S-52.

- Where S-101 changed S-57 modelling, the replacement features have been assigned to the same viewing group as before.

- Features that are wholly new in S-101 compared to S-57 are assigned to new viewing groups.

- Features from new non-S-101 data products are assigned to new viewing groups.

- The principles for chart furniture and miscellaneous display elements (scale boundaries, limits of data, safety contour, depth zones, update identification, legend, etc) are the same in S-52 and this Annex.

- Additional functionalities such as graphical indexes, data quality indication, etc, are independent of the legacy/modernised nature of the information. To minimise clutter and maximise compatibility with legacy data, the functionalities for modernised data are defined so they can act on/with only ENC data in the first place, and on/with other S-100-based data only upon additional or different operator action. For example, the graphical indexes requirement (clause C-12.2) is separated into sub-requirements, one for ENCs and the other for other S-100-based products.

- The names of colour tokens are the same. (Future versions of this Annex may add additional colour tokens, but should retain the S-52 set.)

Note that some things required for complete compatibility of S-57/S-52 and S-101 presentations cannot be controlled in this Annex, depending as they do on the harmonisation of Portrayal Catalogues with S-52 symbology, colour tables, and lookup tables. Among these are the shapes and dimensions of symbols and the colour coordinates assigned to colour tokens.

# Type approval considerations

Manufacturers, type-approval authorities, and above all Mariners, are always encouraged to contact the IHO over any improvements, criticisms, questions or comments that they may have about the ECDIS display, in order that the Specifications can be kept effective and up to date.

# Specifications for the display screen

## Physical display requirements

The requirements for the main graphic display are:

Size:	Minimum effective size of the area for chart display: 270 x 270 mm (IMO MSC 530(106) 10.2).

Resolution:	Minimum lines per mm (L) given by L=864/s, where s is the smaller dimension of the chart display area. (e.g. for the minimum chart area, s=270 mm, the resolution L = 864/270 = 3.20 lines per mm, giving a "picture unit" size of 0.312 mm)

Colours:	64

The specifications above permit a chart display whose minimum resolution (lines/mm) may vary depending on the size of the display. Maintaining a clearly readable chart display under this flexibility imposes certain requirements on the display software, which are described in clause C-10.2.

Information should be displayed in the ECDIS on one or more physical screens, which may be divided into more than one chart display. Information may be displayed automatically, on demand or as a result of Mariner's selection.

The physical size of the screen(s) should be appropriate to viewing conditions. Larger screens may be more suitable for situations where the operational viewing distance is higher, because the larger physical area of the display offsets the reduction of on-screen geographical extent that is caused by the greater zoom levels and symbol sizes necessitated by greater viewing distances.

Redraw during route monitoring to follow the ship's progress, including scale changes due to change in the scale of the chart information, should take less than 5 seconds. Demands by the Mariner that cannot be predicted by the ECDIS, such as draw at a different scale or in a different area may take more than 5 seconds. In the latter case:

- The Mariner should be informed;

- The display should continue route monitoring until the new information is ready to draw within 5 seconds.

## Colour reproduction

Absolute accuracy in colour reproduction is not required but relative colour fidelity is important. To ensure clear contrast between colour-coded features the display screen should be calibrated before use, and this calibration should as far as possible be maintained while in service. If this is not done, lines, symbols, and area shades may become indistinct, and information may then be lost or become misleading.

## Display requirements for colours

[Content is from the following sections of S-52 6.1.1:

4.1	General

4.2	Colour Assignment: Content directly under this S-52 heading is now in clause C-16.2 (Selection of colours (informative) and will not be repeated here.]

4.2.2	Bright Sunlight or Night viewing - Use of filters, etc.

4.2.3	Display calibration and verification.

4.2.4	Colour control; contrast and brightness controls.

4.2.4.1	Effect of controls.

4.2.4.2.	Use of the controls.

4.2.4.3.	Initial setting of the controls.

4.2.4.4.	Re-adjustment of the controls.

4.2.4.5.	Setting the controls for route monitoring.

Section 4.2.6 (The colour scheme - colour tokens) has been rewritten as “C-14.2” and section 4.2.7 (Transparency) is included as “C-14.4 Transparency” and those sections will not be repeated here.]

### General

The ECDIS manufacturer can use any technology to build the display as long as their display fulfils the requirements of this Specification.

### Bright sunlight or night viewing - use of filters, etc

The information-carrying capability of the ECDIS display is curtailed at the extremes of bright sunlight and night viewing. The reasons, and some remedies, are described in this section.

Bright sun. Some of the strong ambient light on the bridge is reflected off the back of the display screen face plate and arrives at the Mariner's eye mixed with the light generated by the monitor which carries the image of the ECDIS display. This display image is further diluted by light entering the Mariner's eye directly from the brightly-lit bridge. The effect is to make the display look washed out; contrast between features may be severely reduced. In the extreme case of sunlight shining directly on the screen, no information at all will be visible. In addition, reflections on the face of the monitor from objects close to the screen, particularly a Mariner's white shirt, may mask parts of the display.

The ECDIS display should be situated where direct sunlight will not shine onto it, nor into the eyes of the mariner looking at it. A visor around the face of the screen may help. A filter may also help, (because it attenuates the sunlight twice, both when incident and when reflected, whereas the monitor image is attenuated only once). Low attenuation neutral density filters may be used, such as a 2 times attenuation filter designated (logarithmically) "0.3 ND". Polarised filters should be an advantage. Ideally, the bridge windows should have glare reducing glass and the inside of the bridge should have non-reflecting paint.

Night. The need to preserve night vision requires that the illumination from the screen be reduced to a lower level than is desirable from considerations of clear viewing. The light level from night colour tables is below that which provides full contrast capability. Consequently some area-fill colours are indistinguishable on the night display, as noted in S-52 Edition 6.1.1 section 3.2.2 (2), and colours of lines may be difficult to distinguish.

An additional problem for a CRT type of monitor is that the R,G & B guns of the CRT may drop to the cut-off point at these very low DAC (digital to analog converter) voltages, particularly on an ageing CRT. Because good quality neutral filters reduce illumination without significantly affecting colour, and even tend to enhance contrast their use is recommended for the night display.

### Display calibration and verification

The ECDIS display should be calibrated initially in order to transform the CIE colour table coordinates to screen coordinates. The main components of the ECDIS display are the monitor and the image generator. Both the monitor and the image generator used to drive the ECDIS display can be calibrated together as a colour generating unit. Another alternative is to calibrate separately both the monitor and image generator.

The following international standards describe methods for calibration of a monitor’s RGB values to produce an output. Other methodologies may be followed, but the same verification test requirements apply regardless of method.

CIE 122-1996		Technical Report: The Relationship between Digital and Colorimetric Data for Computer-Controlled CRT Displays.

IEC 61966-3:2000	Multimedia systems and equipment -Colour measurement and management - Part 3: Equipment using cathode ray tubes, Edition 1.

IEC 61966-4:2000	Multimedia systems and equipment-Colour measurement and management – Part 4: Equipment using liquid crystal display panels, Edition 1.

The ECDIS display calibration should be verified for type approval of all monitors. Colour control, contrast and brightness controls

There are a variety of technologies available for monitors to be used for ECDIS display. Different technologies have different methods to control colour, contrast and brightness, but some basic rules apply for all of them.

#### Effect of controls.

The contrast control of a display generally shortens or extends the range of luminance available, making the display appear darker or brighter.

On the other hand, the brightness control shortens or extends the range of colour saturation available by adding white (which extends the range by making colours other than black less saturated), or subtracting white (colours become darker, more saturated.)

To the viewer, it has much the same apparent effect as contrast, but it achieves this by altering the colour contrast of the colour tables between foreground and background colours, and this may result in some features becoming harder to see, particularly at night. More importantly, making the night display more saturated may drop some dark colours below the cut-off point, into black, thus losing distinctions such as shallow versus deep water.

#### Use of the controls.

Colour tables are provided for day, dusk and night. The mariner should be provided with the control to make fine adjustments between these tables; the calibration position should be marked as a reference for this.

For a CRT type of monitor the brightness control should be pre-set; that is, used only when essential, with provision to return to the calibrated setting. LCD and other monitor technologies have a variety of controls. The general principle is that all available controls should have a provision to return to the calibrated settings and that only appropriate controls should be made available for the Mariner.

The ECDIS manual should carry a warning that careless use of the display controls may adversely affect the visibility of information on the display.

#### Initial setting of the controls.

The controls should be set up in preparation for initial calibration, and their positions marked at that time (for example, by a detent) so that they are recoverable.

#### Re-adjustment of the controls.

In case the controls go out of adjustment in use, they may be re-adjusted on-board ship by means of the Colour Differentiation Test Diagram described in clause C-20.5 and provided by the IHO.

#### Setting the controls for route monitoring

The ECDIS display carries far more detail than a radar display, and requires correspondingly more attention to the correct selection of colour table and the correct adjustment of the controls.

This particularly affects the black-background displays, and is critically important at night, because all colours of the night table, particularly those for large areas, have to be set very dark to avoid loss of night vision, and if the control is turned down too far these colours will disappear into the black background. As a result, chart information will be lost.

The situation will be worse if one of the day tables is improperly dimmed for use at night, instead of switching to the night table.

It is important that the display be adapted to lighting conditions on the bridge by selecting the correct colour table: "Day" for bright sunlight; "Day” or "Dusk" for general daylight viewing; "Dusk" for twilight; and "Night" for the night-darkened bridge.

The controls should only be used for fine adjustment within the appropriate colour table.

To ensure that the controls are always set to a level above that at which information will be lost, the black-adjust symbol BLKADJ should be available to the Mariner, treated as "Standard display", to be called up at any convenient point on the screen. The instructions for its use by the Mariner should be incorporated in the "Users instructions" for the ECDIS.

Below is an example of instructions for a CRT type of Monitor:

1. First, set contrast to a maximum, brightness to a minimum. Look at the black-adjust symbol. Then either:

2A. If the centre square is not visible, turn up the brightness until it just appears;

or

2B. If the centre square is clearly visible (with contrast at maximum, brightness at minimum), turn the contrast down until the inner square disappears, then turn contrast back up until the inner square is just visible again.

(If the above adjustment is not successful, select a more appropriate colour table and repeat this procedure.)

The "black level" is now correctly set. If a brighter display is required use the contrast control, but it is better not to re-adjust the controls unless lighting conditions on the bridge change.

Note that the black-adjust symbol should be displayed to check that the inner square remains visible on the following occasions:

- Every time that the brightness or contrast controls are adjusted.

- Every time that the display is switched to the night colour table.

Below is an example of instruction for LCD type of Monitor. The LCD type of monitor used in this example has only one mariner control, which is called brilliance. Internally the monitor has also other controls available for service engineers. These internal controls include also controls named as contrast and brightness.

1. First, set brilliance to calibration position. Look at the black-adjust symbol.

Then either:

2A. If the centre square is not visible, turn up the brilliance until it just appears;

or

2B. If the centre square is clearly visible, turn down the brilliance until the inner square disappears, then turn brilliance back up until the inner square is just visible again.

(If the above adjustment is not successful, select a more appropriate colour table and repeat this procedure.)

The "black level" is now correctly set. If a brighter display is required use the brilliance control, but it is better not to re-adjust the controls unless lighting conditions on the bridge change.

Note that the black-adjust symbol should be displayed to check that the inner square remains visible on the following occasions:

  - Every time that the brightness or contrast controls are adjusted.

  - Every time that the display is switched to the night colour table.

It is recommended that the BLKADJ symbol be displayed automatically all the time that the night colour table is selected, with a Mariner over-ride to remove it if necessary.

## Colour display capability

Colour displays should be capable of at least 256 luminance steps in each of red, green and blue.

For night performance it is essential that the hardware has a graphics card capable of giving "blacker than black"; that is, complete control of colour, and that the software can control that function.

For CRT type of monitor cathode (beam) current stabilisation is desirable, to prevent dark colours dropping below the cut-off point.

### Colour conversion tolerances and tests

#### Tolerances

The tolerances quoted below apply only to the process of converting CIE colour coordinates to RGB values, and so should be evaluated shortly after the ECDIS leaves the manufacturer's plant.

Considerable operational experience will be needed before it will be possible to state colour maintenance tolerances for ECDIS on-board ship.

The colour tables developed have been selected to ensure maximum colour discrimination between features. Colour discrimination depends on both the colour difference and the luminance difference between two colours. Colour science (as represented by the CIE colour convention) has defined colour difference units ΔE*. The ΔE* metric is a measure of the overall discrimination (including both colour and luminance differences). As a metric for ECDIS colour accuracy, a measure of the discrimination in colour alone, excluding luminance differences has been defined as a subset of ΔE* referred to as Δ(u*,v*).

Δ(u*,v*) = SQRT [ (u2*-u1*)2 + (v2*-v1*)2 ]

ΔE* = SQRT [ (L2*-L1*)2 + (u2*-u1*)2 + (v2*-v1*)2]

Calculations of L*, u*, and v* must be made using as reference the chromaticity and luminance specified for the brightest white colour token in the respective colour table (Y0, u0, y0); where Y0 is the luminance in units of cd/m2. Note: this is not the brightest white of the monitor.

Note: “Δ” represents the Greek letter “Delta”, although it may appear differently on some computers.

The tolerances for the conversion of the colour tables from the CIE colours defined in these specifications to the actual RGB values for the ECDIS CRT are defined in three terms:

1. Overall discrimination between actual colours within the converted table: ΔE*. This is to ensure that all the colours within the RGB table remain discriminably separate; that is, that the relative colour distinctions have been maintained.

2. Colour discrimination differences between the defined and the actual values: Δ(u*,v*). This is to ensure that the actual RGB colours resulting from the conversion remain reasonably close to the CIE colours defined in the specifications; that is, that the blues stay blue and the greens stay green.

3. Luminance differences between the defined values and the actual values: L. This is to ensure that the luminance remains the same within acceptable limits.

Note: The CIE L*u*v* human perception colour model, which is the source of the tolerances described above, has not yet been evaluated at the low luminances of the night colour table, at which the less colour-sensitive rods of the eye take over from the daytime cones. Consequently these tolerances should not be applied to the night table; and for type-approval purposes they are restricted to the bright sun table.

Colour tolerance values:

1. The discrimination difference between any two colours displayed (except those with a tabular ΔE* less than 20) should be not less than 10 ΔE* units.

| Token | Colour (x, y, L) | Token | Colour (x, y, L) | ΔE* |
| --- | --- | --- | --- | --- |
| DEPMD | (.27 .30 65) pale blue | CHWHT DEPDW UIBCK | (.28 .31 80) white | 11 |
| CHBRN | (.42 .45 30) brown | ADINF | (.41, .47 35) yellow | 14 |
| DEPMS | (.24 .26 55) medium blue | DEPVS UIAFD | (.22 .24 45) medium blue | 17 |
| DEPMD | (.27 .30 65) pale blue | CHGRF NODTA (S-52 Ed 3.3 colours have greater ΔE) | (.28 .31 45) faint grey | 18 |

Table C-30 - Tolerance exceptions

2. The difference between the colour displayed and the CIE colour defined in these specifications should be not greater than 16 Δ(u*,v*) units. If a monitor is independently tested, then the difference must not be greater than 8 Δ(u*,v*) units.

3. The luminance of the colour displayed should be within 20% of its specified value. Black is a special case and the luminance of it must not be greater than 0.52 cd/m² for bright sun colour table.

#### Instrumental calibration verification test

For CRT displays, an instrumental test to check that the results of the colour conversion calibration are within tolerance should be made by displaying the colours of the Day colour table (restricted to colour pairs of tabular ΔE* greater than 20); measuring their CIE coordinates x,y and L; and applying a tolerance test. For LCD displays the instrumental test should be applied to all three colour tables.

Note that since the tolerance test is intended solely to check successful colour calibration, and not to test colour maintenance at sea, this test should be performed on the bench in the manufacturer's or type-approval authority's plant under normal conditions of temperature, humidity and vibration.

Manufacturers of ECDIS can choose between two different methods of colour calibration.

The first method is a test of a monitor as part of an integrated system. In this method both the monitor and the image generator parts of ECDIS display are tested together.

The second method is an independent test of the monitor. In this method the monitor and the image generator of ECDIS display are separately tested against a reference (that is, the monitor is tested against a reference image generator and the image generator is tested against a reference monitor). The second method has tighter tolerance for displayed colour than the first method (see clause C-20.4.1.1).

## Colour differentiation test diagram

A multi-purpose colour differentiation test diagram is illustrated in Figure C-9. This consists of 20 squares each coloured with one of the 4 main background colour fills (such as shallow water blue); and each having a diagonal line in one of the six important foreground colours (such as mariner's orange). Each diagonal line is 0.64mm.

The diagram is in the form of a dataset and so can be displayed using any of the three colour palettes. This diagram is intended:

- For use by the Mariner to check and if necessary re-adjust the controls, particularly for use at night;

- For use by the Mariner to verify that an ageing display remains capable of providing the necessary colour differentiation; and

- For initial colour verification of the day, dusk and night colour tables.

Both the Colour Test Diagram and the instructions for its use (see clause C-20.7) should be made available to the Mariner.

In addition, a grey scale is described in clause C-20.7 for use by maintenance technicians in checking colour tracking in an ageing display.

The Colour Differentiation Test diagram is intended for use off-line. It is not needed during route monitoring.

Note that the Colour Differentiation Test Diagram will not be true to colour unless it is projected on a calibrated screen and is generated using the digital format provided by IHO, which correctly reproduces the colour tokens of the Presentation Library.

The colour differentiation diagram is required in "Day" and "Dusk" colours so that the Mariner can verify that the ECDIS display monitor has the colour differentiation capability needed to distinguish between the various colour-coded areas, lines and point symbols of the ECDIS display. Both diagrams will be provided online by the IHO and should be downloaded by manufacturers for supply with ECDIS software. The diagrams will not be true to colour unless they are projected on a calibrated monitor and are generated in a manner which correctly reproduces the colour tokens.

Figure C-9 - Colour differentiation diagram - Day

[NOTE: For illustrative purposes only. The diagrams from the IHO web site should be downloaded for testing.]

## ECDIS Chart 1

The IHO provides ECDIS Chart 1 in digital form, a graphical index of ECDIS symbols including both simplified and paper-chart point symbols; and also the symbolized lines and area boundary linestyles. This is intended to familiarize the Mariner with the colour and symbol coding used by the ECDIS. The symbols are grouped according to INT1, which is familiar to the Mariner, but are numbered with a look-up sheet, not labelled. Manufacturers should provide linking by cursor interrogation between the symbols and the explanations given in ECDIS Chart 1.

Since Product Specifications for data products used on ECDIS will be updated at different times, there may be a supplementary “Chart 1” for each data product other than ENC that is intended for use in ECDIS.

The ECDIS Chart 1 and any supplements are intended for use off-line and in route planning. They are not needed during route monitoring, when the Mariner can use cursor enquiry to find the meaning of symbols.

The ECDIS Chart 1 and its indexing list of symbol names and meanings arranged numerically, together with the colour differentiation test diagrams, are intended for the Mariner’s use. The use of the Colour Differentiation Test Diagrams is described in C-20.5. A separate plot of symbols arranged alphabetically and an indexing list of symbol names and meanings arranged alphabetically will be provided for the manufacturer’s use.

Note that because colour printing and copying is not completely true to the original, a hard-copy version of Chart 1 does not accurately represent the colour requirements of the IHO Colour and Symbol Specifications.

In addition, symbol size may change in copying.  To ensure correct size, all symbols illustrated must be scaled by the factor required to make symbol CHKSYM01 measure 5mm by 5mm.

## Use of ECDIS Chart 1 and Colour Test Diagram

### Specification for ECDIS Chart 1 and the Colour Test Diagram

#### Definition (for this specification only)

A Chart 1 dataset must  be displayed so as to fill all of the standard ECDIS display area (that is, the minimum 270 x 270 mm chart area).

#### Description and purpose

The ECDIS Chart 1 and the Colour Differentiation Test are diagrams for use by the Mariner which are provided in the form of a number of ENC datasets.

The ECDIS chart 1 is intended to familiarise the Mariner with the symbology used on ECDIS. The Mariner must  be able to display each cell, and by cursor-pick get a read-out of the meaning of any feature shown.

The Colour Differentiation Test diagram is intended for display using the day or dusk colour tables so that the Mariner can check that the ECDIS display is providing adequate colour performance.  It is also used in type-approval testing. Instructions for its use are given in the sections below.

The ECDIS Chart 1 includes the CHKSYM symbol which is intended for checking the correct size of the symbols during the type approval.  The width and height of the CHKSYM is 5.0 mm.

The line width of the diagonal lines in the Colour Differentiation Test diagram is specified as 0.6 mm wide..

#### Mode of use

These diagrams are supplementary features of the ECDIS, intended for use off-line or during route planning. Because they occupy the entire display they must not be used during route monitoring.  If the Mariner needs to find the meaning of a symbol during route monitoring, they must use cursor-picking.

The operation of these diagrams is not subject to the draw-speed requirements of route monitoring.

#### Content and Encoding

Chart 1 is released as a number of S-101 datasets within the S-164 suite of test data. Portrayal and content are defined wholly by the S-101 feature and portrayal catalogues.

#### Revisions

Revisions will be made by whole file replacement; that is, by issuing a new edition.

#### Packaging

The data files may be downloaded from the IHO web site.

#### Presentation and parameter settings

.

### Displaying the Colour Test Diagram

#### Introduction; providing the diagram

The Colour Differentiation Test diagram is provided to enable the Mariner to verify that the display screen still retains the colour differentiation capability needed to distinguish between the various colour-coded areas, lines and point symbols of the ECDIS display.

The diagram will not be true to colour unless it is displayed on a calibrated monitor and is generated using the provided ENC portrayal catalogue.

The two methods of providing the diagram are:

- Use the dataset labelled “101AA005C1WOO” containing the Colour Differentiation Test Diagram.  This file must be drawn so that the extent of the imaginary chart data covers the entire ECDIS display.  Because the file uses customised features and attributes, it must be displayed using the special Portrayal and Feature Catalogues provided for the digital S-100 ECDIS Chart 1.

- Use the graphics file illustrated in Figure C-9 as a model.  Based on this model, reproduce the same pattern of rectangles and lines on the screen, but present them in the correct colours using the colour tokens given in the S-101 colour profiles provided by the IHO.

The diagram consists of twenty numbered squares extending over the whole of a 270 x 270 mm screen.  Each square is coloured with one of the four main background area shades (such as shallow water blue, DEPVS), and each carries a two-pixel wide diagonal line in one of the important line or symbol foreground colours (such as planned route red, PLRTE). These are arranged as follows:

Four main background colours:

DEPVS		(shallow water blue)				squares 3, 5, 11, 15, 18, 20.

DEPDW	(deep water, white or black)			squares 1, 7, 8, 10, 13, 19.

LANDA 	(land colour)					squares 6, 14, 17.

NODTA 	(no data shade: radar, navigation safety		squares 2, 4, 9, 12, 16.

lines and chartwork must be visible on the

no-data part of a display)

Six important foreground colours:

DEPSC		(safety contour grey)				squares 3, 10, 17.

NINFO 		(orange, Mariner's information)			squares 5, 8, 14, 16.

ADINF 		(yellow, manufacturer's information)		squares 12, 15, 19.

TRFCD 	(magenta, traffic lanes and area 	boundaries)	squares 1, 9, 11.

RADLO 	(the lower luminance radar green)		squares 4, 6, 13, 18.

RESBL		(blue, provisionally reserved for traffic info	squares 2, 7, 20.

from transponder, VTS etc.)

Note: Remember that a TIFF, PDF or other source will not be true to colour unless it has been specifically modified to access the colour tokens and colour tables used by the ECDIS.

#### Test description

The dusk and night tables should be checked subjectively by means of the colour differentiation test diagram, which is provided as an S-101 file (.tif file diagrams must not be used for this purpose), as follows:

(1) The person carrying out the test should have passed the Isihara colour blindness test, or other test used to qualify bridge watch keepers, and should adapt to night viewing for 10 minutes before checking the night display;

(2) The controls should be set to their calibrated settings;

(3) While the display is off, adjust the ambient light reflected from white paper positioned on the display screen to the following values:

Colour profile	Light level

Day		200 cd/sq. m

Dusk		10 cd/sq. m

Night		darkness (the ECDIS display is the predominant light source)

Preferably use natural daylight for the day table;

(4) Under each of the above conditions, display the appropriate colour differentiation test diagram for the colour profiles.

Select each table in turn and ensure that:

- Each foreground diagonal line is clearly distinguished from its background; and

- The foreground lines representing yellow, orange, magenta (purple), green, blue and grey may be clearly identified.

#### Relationship to S-57 Chart 1

The Chart 1 datasets are located alongside the existing S-57 Chart 1 cells.

#### Using the diagram

The Colour Test must be applied on the day and dusk colour tables.

Before the Colour Test diagram is used, the black-adjust symbol BLKADJ01 must be brought up on the screen and the contrast and brightness controls (or equivalent controls for an LCD) must be adjusted as follows:

1.  First, set contrast to a maximum, brightness to a minimum. Look at the black-adjust symbol. Then either:

2A.  If the centre square is not visible, turn up the brightness until it just appears;

or

2B.  If the centre square is clearly visible (with contrast at maximum, brightness at minimum), turn the contrast down until the inner square disappears, then turn contrast back up until the inner square is just visible again.

(If the above adjustment is not successful, select a more appropriate colour table and repeat this procedure).

The "black level" is then correctly set. If a brighter display is required use the contrast control, but preferably do not adjust the controls unless lighting conditions on the bridge change.

The test consists of being able to distinguish the background colours and to pick out the like foreground colours, that is to say that squares 3, 5, 11, 15, 18 and 20 all have a shallow water blue background, and that squares 3, 10 and 17 have a grey line.

Note: The test above uses the black-adjust symbol for the purpose of a type approval test. The other purpose of the black-adjust symbol BLKADJ01 is to allow the Mariner to adjust the display for ambient illumination on the bridge of a ship.  An ECDIS must have the black-adjust symbol displayed whenever the mariner is adjusting the display (that is, depending on the technology of the display brilliance, brightness, contrast, etc), as required by clause C-20.3.

### Grey Scale

A grey scale may be used by service technicians to detect display ageing or other display performance issues.

Eight grey strips are recommended, spaced between the minimum and the maximum luminance for each of the five mandatory colour tables.

The bit levels or signal levels producing the grey levels are evenly spaced from the level producing white to the level producing black. In order to select the appropriate grey level from a large set of available grey levels, use one of the following models:

- Bit Levels in Software: Given n levels of grey that can be displayed on a screen, with 0 for black and w = n - 1 for white.  We want to select a subset of m levels that are as evenly spaced as possible.  The interval between the n levels to create m levels is ΔV = w/(m-1), which may not be an integer.  So, the levels to select are the (integer) values of Vi= int[(i-1) ΔV] for i = 1, 2, ..., m, or Vi= 0, int(ΔV), int(2ΔV), int(3ΔV), ..., int[(m-1) ΔV], with int[(m-1) ΔV] = w for white.  For example, if there are n = 256 = 28 levels from which we select m = 8 levels, white is w = 255; the interval is ΔV = 36.4286, and the chosen levels are: 0, 36, 73,109, 146, 182, 219, 255.

- Analog Signal Levels: For analog signals, if Vw is the white level and Vb is the black level, then for m levels the signal step size is ΔV = (Vw - Vb)/m and Vj= Vb + jΔV.

# Dataset Management

This clause covers dataset management on the ECDIS to the extent that it affects the user experience and portrayal processing.

## Multiple product versions and portrayal

The ECDIS must be able to carry and use multiple versions of the Feature Catalogue for a product.  Catalogue management is based on the version number of the Product Specification and Catalogues.  For example, the ECDIS will need to carry all valid Catalogues that are used for datasets that have been produced from an earlier edition of a Product Specification; but may retire a superseded version after the last such dataset has been cancelled.

See Appendix C-2 for loading and ingest processes including SSE Codes relating to Product Specification and Interoperability Catalogues.

IHO Publication S-97 recommends that the versioning of Product Specifications follow the same rules as S-100, which uses a three-part version number (Edition.Revision.Clarification). The significance of each component is summarised below.

- Edition: New Editions introduce significant changes, such as the ability to support new functions or applications; or the introduction of new constructs or data types. New Editions are indicated by incrementing the Edition component of the version number and resetting the other components to 0.

- Revision: Revisions introduce substantive semantic changes. Changes in a Revision ensure backward compatibility with previous versions within the same Edition. A Revision, for example, may introduce new features and attributes, but will not delete a feature or attribute. New Revisions are indicated by incrementing the Revision component of the version number and resetting the Clarification component to 0.

- Clarification: Clarifications are non-substantive changes. Typically, Clarifications remove ambiguity; correct grammatical and spelling errors; amend or update cross references; and/or insert improved graphics, spelling, punctuation and grammar. Clarifications must not cause any substantive semantic changes. Changes in a Clarification are minor and ensure backward compatibility with the previous versions within the same Edition. Clarifications are indicated by incrementing the Clarification component of the version number.

Each dataset has a product specification edition number contained within it.[add encoding specifics]. This requires the ECDIS to have installed a feature catalogue which has a matching edition and revision number. Multiple feature catalogues must therefore be supported to maintain different revisions of datasets within the System Database.

A feature catalogue is only compatible with portrayal catalogues having the same edition and revision number.

Further consequences for portrayal are:

- It should be possible to process datasets conforming to an earlier Revision within the same Edition, with the Feature and Portrayal Catalogues for the latest Revision in that Edition.

- It should be possible to process datasets conforming to an earlier Revision or Clarification within the same Edition with the Feature and Portrayal Catalogues for the latest.

- A Portrayal Catalogue for a new Clarification can always rely on earlier versions of the Feature Catalogue within the same Edition. (Note that an old Feature Catalogue cannot be relied on for processing the dataset for a newer Revision. That is OldDataset+OldFC+NewPC is processable, but NewDataset+OldFC+NewPC is not.)

- It should be possible to process datasets conforming to a Clarification with the Feature and Portrayal Catalogues for an earlier Clarification within the same Edition and Revision.

- The significance of changes to only a Portrayal Catalogue (without a concomitant change to the Product Specification or Feature Catalogue) can be more difficult to classify (as a New Edition, Revision, or Clarification), because in addition to data model semantics, human factors effects for user interfaces and IMO guidance about compatibility, text abbreviations, etc, must also be considered.

In general, the IHO and/or Type Approval Authority should be consulted to determine the significance of the change. This also applies to Revisions planned by Project Teams.

## Dataset overlaps and gaps

### Overlaps and gaps in ENC coverage

There may be cases where ENCs in the same scale range overlap. Such may be the case at agreed adjoining producer data limits, where, if it is difficult to achieve a perfect join, an overlapping buffer zone of up to 5 metres may be used.

Where an overlap of more than 5 metres between two or more datasets exists the ECDIS must only display one dataset for the overlap area and provide a permanent and persisting indication “overlap”.

### Overlaps and gaps between other data products

Overlaps in datasets of the same scale range in products other than the ENC should be indicated by a non-permanent indication “Overlap in ??? data” (where ??? is the abbreviation for the data product). If interoperability has been activated, the Interoperability Catalogue may have a rule to break the tie, in which case an indication is not needed. An exception to this rule is S-102 and S-104 data used for Water Level Adjustment. In these cases, as with ENC overlaps, the ECDIS must only select and use a single dataset from the overlapping data available and provide a permanent and persisting indication “overlap” as per ENCs.

Coverage gaps in products other than the ENC can be expected and should not be indicated. Similarly, overlaps between different data producers’ ENC and other data products other than ENC should also be expected. Certain features, such as radio service areas can extend far beyond national boundaries and may therefore overlap other data producers’ ENC data. An indication is not needed in these cases.

## Guidance on updating S-100 datasets on ECDIS

### Introduction

In a previous Edition (3rd Edition, December 1996), S-52 Appendix I provided guidance for the updating service and the ECDIS to support the updating of ENCs issued through a Regional ENC Coordinating Centre (RENC). Following a HSSC-requested review by the ENC Updating Working Group (EUWG), guidance related to ENC updates production by HOs and their distribution was mainly integrated into Edition 2.0.0 of IHO publication S-65 ENC Production, Maintenance and Distribution Guidance and Edition 3.0.0 of IHO Publication S-57 Appendix B.1, Annex A Use of the Object Catalogue for ENC. Beginning with Edition 4.0.0 (April 2012), S-52 Appendix I was limited to describing guidance for the processing of automatic and manual ENC updates by ECDIS.

The current Editions of S-57 and S-65 continue to apply to the production and data delivery for S-57 ENC updates. S-52 Appendix I continues to apply to the processing of S-57 updates on ECDIS.

The guidance in S-52 Appendix I has been updated for S-100 datasets on ECDIS and is provided in clauses C-21.3.2–C-21.3.5 below. Except for C-21.3.5 (support file updates), which is a new requirement for support files, and the addition of language relating to authentication, the requirements are the same as S-52 Edition 6.1.1 with the language updated to include data products other than ENCs.

Some products may require or permit the use of data protection. Whether data protection has been applied to a dataset can be determined from the dataset discovery metadata in the exchange catalogue file (see S-100 Part 17). Digital signatures and authentication are described in S-100 Part 15. The protection method is indicated in dataset discovery metadata (S-100 Part 17; it should also be documented in individual Product Specifications. At present there is only one method specified in S-100, which is described in Part 15. S-100 Part 15 should be referenced for authentication and integrity checks).

### General requirements

- Data Integrity. The ECDIS should be able to process updates to S-100 data products without degradation of the information content of the dataset or dataset update. For example, all information regarding attributes, logical relationships, geometry, and topology must be accounted for.

- Verification of Application. The ECDIS should provide a method to ensure that updates have been correctly applied to the System Database (or other appropriate database for products held outside the System Database). Those updates are either an official data product update integrated into the display; an update to non-official products installed on the system; or temporary information that was entered manually.

- Integrated/Non-integrated Updates Distinction. Updates should be clearly distinguishable on the display. Once accepted, integrated updates to ENCs should be indistinguishable from ENC data. Non-integrated updates (that is, those entered manually) must be distinguishable as described in clause C-12.11.1. Updates to non-ENC data must be treated as described in C-9.1.3 and C-12.11.4.

- Storage Separation. ECDIS should store all updates separately from the original datasets. However, such separate storage may utilize the same data storage device.

- Recall for Display. It should be possible on demand to review previously installed updates.

- Compatibility. Updates to S-100 products must comply with the applicable Product Specification. Only ISO8211 and GML encoded data can support updates, as described in Part 10a and 10b respectively. The use of GML updates as described in Part 10b is used, in particular, by S-128 to update catalogue information used by the ECDIS to produce update Status Reports, as described in Appendix C-3 - ECDIS Update Status Reports.

- Non-interference. ECDIS should be able to receive updates without interfering with its current operation.

- Log File. ECDIS should keep a record of updates, including time of application and identification parameters described in the applicable Product Specification, through a log file. The log file should contain, for each update applied to or rejected by the System Database, the following information:

.1 date and time of application/rejection;

.2 complete and unique identification of update as described in the applicable Product Specification;

.3 any anomalies encountered during application;

.4 type of application: manual/automatic.

- Update out of sequence. The ECDIS should warn the user when an update is applied out of sequence, terminate the update operation and restore the System Database as it was before the application of the update file.

NOTE (informative): Sequences are determined by the update and Edition numbers, not by issue date and time. However, sequential updates should have issue date/time combinations that are consistent with the sequence of updates as determined by the update and Edition numbers. Issue date/time are given in dataset discovery metadata in the Exchange Catalogue (see S-100 Part 17). Note that time is optional. Note also that it is possible for the date/time for two or more consecutive updates to be the same - in fact for data that are frequently updated, such as water levels, the date will be the same for many consecutive datasets.

### Automatic Update

- Interface

  - Fully Automatic Updates. The ECDIS should be capable of being interfaced to an appropriate telecommunication network.

  - Semi-automatic Updates. The ECDIS should be capable of receiving official updates in standard IHO format by CD-ROM and from any other interface or data storage media that are provided with the ECDIS for that purpose and through telecommunication.

- Reception of Updates

  - Update data must be recorded automatically in the update storage of the ECDIS.

  - The identification of the Issuing Authority of the update should be checked for conformance with the corresponding identifier of the dataset being updated.

  - If any errors are detected from the receiving device, the reception procedure must be terminated and the update flagged invalid in the record of updates. The user should be informed of the corruption.

- Sequence Check. The following sequence number checks should be performed at the time of application, for sequential and cumulative updates:

.1 File extension of the update (if applicable);

.2 Update number of the update;

.3 Update sequence number of the individual records in the update. Refer to the relevant Product Specification for details on how the sequence numbers are encoded in the update.

- Authentication and Integrity Check. If the Product Specification uses data protection and therefore requires the use of digital signatures, the ECDIS should authenticate updates using the applicable procedures. In some cases, updates that do not pass the authentication and integrity check should not be applied. The user should be informed of any authentication anomalies. Update authentication-related messages for the user may be combined or throttled to avoid flooding the user, but should all be logged in the log file (see appendix C-2).

- Consistency Check. The mariner should be warned of any previous updates which have not been successfully applied.

- Geographic Applicability. Updates not relating to a dataset within the set of datasets in the ECDIS may be discarded.

- Summary Report. A summary report for each of the Issuing Authority's official update files should be given after completion of receipt containing at least:

.1 Identification of Issuing Authority;

.2 Update numbers of the update files;

.3 Dataset Identifiers of datasets affected;

.4 Edition number and date of dataset involved;

.5 Number of updates in the affected dataset.

- Review of ENC Updates. It should be possible for the Mariner to review the updates applied through displaying the System Database contents with the updates highlighted.

- Modification of Updates. Rejection or amendment of an update by the Mariner must be achieved by the manual update method. The questionable update should be noted as an anomaly in the Log File (see clause C-21.3.2(h)).

- Formatted Non-integrated Updates, for example a temporary military exercise area, will be processed as manual updates.

- ECDIS Update Status Report. It should be possible for the user to inspect and demonstrate the up to date status of all datasets held within the ECDIS against a complete list supplied by a service provider. Details of the required update status reports for both electronic chart and nautical publications datasets are given in Appendix C-3

### Manual Update

- Keying and Symbology. The ECDIS should enable manual entry of updates for non-integrated presentation on the display. A capacity should exist to enable the mariner to:

.1 Enter the update so it can be displayed as described in this Annex.

.2 Ensure all update text information relevant to the new condition and to the source of the update, as entered by the Mariner, is recorded by the system for display on demand.

- Indications and Alerts. The ECDIS should be capable of sensing indications and alerts related to non-integrated (manual) updates, just as it does for integrated updates.

- Presentation. Manual updates must be displayed as described in this Annex, clause C-9.7.

- Text. It should be possible to enter text into the ECDIS.

- Archiving of Manual Updates. It should be possible to remove from the display any manual update. The removed update should be retained in the ECDIS for future review until commencement of the next voyage, but will not be otherwise displayed.

### Support file updates

When a feature pointing to a text, picture or application file is deleted or updated so that it no longer references the file, the ECDIS software should check to see whether any other feature references the same file, before that file is deleted.

## New editions, re-issues, and updates of datasets

The method for detecting whether a dataset is a New Edition, re-issue, cancellation, termination, or update is contained in the Exchange Set metadata field S100_Purpose.

The behaviour of each is described below; note that individual Product Specifications may not implement all types of dataset state. All datasets include an issue date, and optionally an Edition and update number.

- New dataset – install new dataset and any included updates and record for tracking purposes.

- New Edition – replace numbered old Edition and its updates with the New Edition. The information in clause C-13.6 about the use of New Editions and re-issues for certain coverage data products should be kept in mind

- Update – apply the numbered update and retain for tracking purposes. See C-21.3. Where updates cannot be applied a permanent indication “Chart information not up to date” must be available in the chart display area when such a chart is in use (either displayed on chart area or used as largest scale available for chart related alerts and indications). This is also covered by the ECDIS Update Status Report, Appendix C-3.

- Re-issue - replace the original datasets and its updates up to the date of the issue with the reissued dataset. Retention of the old issue should be as specified in the Product Specification. The information in clause C-13.6 about the use of New Editions and re-issues for certain coverage data products should be kept in mind. After a re-issue, subsequent updates may be incorporated from this reissue or from the original data kept continuously updated.

- Cancellation – delete the cancelled dataset and its updates. The system must report any dataset(s) that have been identified as cancelled at load time. A message must be displayed informing the user of the dataset name. Depending on the method adopted by the OEM for managing cancelled datasets one of the following conditions must be observed: (1). The cancelled dataset cannot be viewed in the ECDIS; (2). The cancelled dataset can be viewed in the ECDIS with the warning message specified “Dataset <name> has been cancelled and may not be up to date. Under no circumstances should it be used for primary navigation”.

Keep in mind the guidance about updating support files (C-21.3.5) when replacing or deleting a dataset.

# Support File Formats

ECDIS support files must be limited to the following formats out of the support file formats listed in S-100 Part 17 S100_SupportFileFormat enumeration:

- Plain text

- HTML

- JPEG 2000

- TIFF 6.0

In addition, CSS (Cascading Style Sheets) files may be used for styling HTML and XML suport files. If used only in a single support file, the styles should be embedded rather than encoded in a separate CSS file. CSS files are encoded in discovery metadata as “other” support files.

All support files except the graphic formats must use UTF-8 encoding.

Specific restrictions on support files containing textual information are described in clause C-11.5. Specific restrictions on graphics files are described in clause C-11.6.

Page intentionally left blank

# Appendix C-1 –

# Manual Editing and Update

# Introduction

- Manual updates are official data entered manually into the ENDS/ENC. Manually entered updates must be marked by specific symbols.

- Indications and alerts should be triggered as for all official ENC data.

In order to provide a harmonised and consistent approach to manual editing and update on the ECDIS a set of portrayal symbols is supplied alongside this publication. These symbols, contained within an S-100 portrayal catalogue and feature catalogue allow the OEM to implement a manual update and editing feature on the ECDIS.

A description of the supplied symbols is provided in this Appendix.

Need to include:

- Description of feature model.

- Brief Chart 1 style image of the symbols.

- Any guidance on how they should be used?

- Is there an alert/indication aspect of manual updates?

Page intentionally left blank

# Appendix C-2 - S-100 Data Import Error Codes and Explanations

Loading, Updating, Authentication and Decryption error codes and messages are defined in this section. It is expected that application developers may also support the error conditions with an appropriate error message.

When an error occurs, this can in some instances, prevent further processing of data.

| Error Code | Error Message |
| --- | --- |
| SSE 101 | Self Signed Key is invalid |
| SSE 102 | Format of Self Signed Key file is incorrect |
| SSE 103 | Signed Data Server Certificate is invalid |
| SSE 104 | Format of SA Signed DS Certificate is incorrect |
| SSE 105 | SA Digital Certificate (X509) file is not available. A valid certificate can be obtained from the IHO website or your data supplier |
| SSE 106 | The SA Signed Data Server Certificate is invalid. The SA may have issued a new public key or the dataset may originate from another service. A new SA public key can be obtained from the IHO website or from your data supplier |
| SSE 107 | SA signed DS Certificate file is missing. A valid certificate can be obtained from the IHO website or your data supplier” |
| SSE 108 | SA Digital Certificate (X509) file incorrect format. A valid certificate can be obtained from the IHO website or your data supplier |
| SSE 109 | Dataset Resource Digital Signature is invalid |
| SSE 110 | Dataset Permits not available for this Data Server. Contact your data supplier to obtain the correct permits. |
| SSE 111 | Dataset Permit not found. Load the permit file provided by the data supplier. |
| SSE 112 | Dataset Permit format is incorrect. Contact your data supplier and obtain a new permit file. |
| SSE 113 | Dataset Permit is invalid (checksum is incorrect) or the Dataset Permit is for a different system”. Contact your data supplier and obtain a new or valid permit file. |
| SSE 114 | Incorrect system date, check that the computer clock (if accessible) is set correctly or contact your system supplier. |
| SSE 115 | Subscription service has expired. Please contact your data supplier to renew the subscription licence |
| SSE 116 | ENC CRC value is incorrect. Contact your data supplier as ENC(s) may be corrupted or missing data. |
| SSE 117 | User Permit is invalid (checksum is incorrect). Check that the correct hardware device (dongle) is connected or contact your system supplier to obtain a valid User Permit. |
| SSE 118 | HW_ID has incorrect format |
| SSE 19 | Permits are not valid for this system. Contact your data supplier to obtain the correct permits |
| SSE 120 | Subscription service will expire in less than 30 days. Please contact your data supplier to renew the subscription licence |
| SSE 121 | Decryption failed no valid dataset permit found. Permits may be for another system or new permits may be required, please contact your supplier to obtain a new licence |
| SSE 122 | One or more of the SA Digital Certificate or Domain Controller certificates (X509) have expired. A new SA public key can be obtained from the IHO website or from your data supplier |
| SSE 123 | Non sequential update, previous update(s) missing try reloading from the base media. If the problem persists contact your data supplier |
| SSE 124 | Dataset Signature format incorrect, contact your data supplier |
| SSE 125 | The permit for dataset < name> has expired. This dataset may be out of date and MUST NOT be used for Primary NAVIGATION |
| SSE 126 | This Dataset is not authenticated by the IHO acting as the Scheme Administrator |
| SSE 127 | Dataset <dataset name> is not up to date. A New Edition, Re-issue or Update for this cell is missing and therefore MUST NOT be used for Primary NAVIGATION |
|  | [Added for S-100 Support] |
| SSE128 | Error installing <file name>. The format or content could not be validated and it could not be installed |
| SSE129 | This Catalogue is not authenticated by the IHO acting as the Scheme Administrator |
| SSE130 | Warning: Dataset <dataset name> is not up to date. A New Edition or Update for this cell is missing |
| SSE131 | Error: No Feature Catlaogue or Portrayal Catalogue could be found for dataset <dataset name> |
| SSE132 | Signature Path can not be validated. One or more of the Domain Controller or SA digital certificates is missing or invalid |
| SSE133 | Version mismatch between <dataset> and <catalogue>. Only <catalogue version> is supported for data of this type (<product>) |
| SSE134 | This Exchange Set is not authenticated by the IHO acting as the Scheme Administrator |
| SSE135 | This Permit file is not authenticated by the IHO acting as the Scheme Administrator |

Table C-2-1 - Error codes and messages

SSE 101 must be returned when a self signed key (SSK) cannot be validated against the public stored as part of the SSK. The data server must check that its own SSK is valid before sending it to the SA. The SA will confirm the data server SSK before returning the SA signed data server certificate.

SSE 102 must be returned if the SSK is wrongly formatted or if mandatory elements are missing. SA and data servers must complete this check.

SSE 103 must be returned if the Signed data server certificate does not authenticate correctly against the SA public key or any Domain Controller Intermediaries. This validation process must be carried out by the SA or Domain Controller before supplying it to the data server. The data server must also validate a received certificate. The data client must validate the digital signature against any file prior to its use.

SSE 104 must be returned if the SA signed data server certificate is wrongly formatted. This must be carried out by the data server on receipt from the SA.

SSE 105 must be returned if there is no SA certificate installed on the data client.

SSE 106 must be returned if the SA digital certificate (public key) does not validate against the following:

- SA digital certificate will not validate against the SA public key.

- One or more of the Domain Controller certificates do not validate against the SA certificate.

SSE 107 must be returned if the SA signed data server certificate is not available to the data server for checking or is not present in the Exchange Set Catalogue file when the data client attempts to authenticate it.

SSE 108 must be returned if the SA public key held in the SA digital certificate is wrongly formatted or the certificate file is unreadable.

SSE 109 must be returned if any resources (dataset or supplementary file) in the Exchange Set Catalogue do not authenticate against the SA authenticated data server public key or Domain Controller authenticated public key contained in the Exchange Set Catalogue metadata.

SSE 110 must be returned if there are no dataset permits available for a particular data server corresponding to encrypted data within the Exchange Set being loaded.

SSE 111 must be returned if there are no permits installed on the system for a required dataset.

SSE 112 must be returned if the dataset permits are formatted incorrectly.

SSE 113 must be returned if the calculated CRC of an individual dataset permit does not validate against the CRC held in that cell permit. [Data Clients] This may be a HW_ID problem, corruption during transmission or the permits are for a different system.

SSE 114 must be returned if the system date does not agree with the date obtained from any alternative, reliable date source; for example, GPS. [Data Clients]

SSE 115 must be returned if the expiry date of the dataset permit has an earlier date than that obtained from the validated system date. [Data Clients]

SSE 116 must be returned if the calculated CRC value of the ENC (after decryption and uncompressing) does not validate against the corresponding CRC value in the CATALOG.031 file. This also applies to the unencrypted signature, text and picture files. [Data Clients] NO CRCs]

SSE 117 must be returned if the CRC contained in the User Permit does not validate against the calculated CRC of the extracted HW_ID. [Data Servers]

SSE 118 must be returned if the if the decrypted HW_ID extracted from the User Permit is incorrectly formatted. [Data servers]

SSE 119 must be returned if the HW_ID stored within the hardware/software security device cannot decrypt the cell permits being loaded or already installed on the system. [not used anymore]

SSE 120 must be returned if the subscription licence is due to expire within 30 days or less.

SSE 121 must be returned if a valid dataset key (decryption key) cannot be obtained from the relevant dataset permit to enable the system to decrypt the corresponding dataset.

SSE 122 must be returned if the SA Digital Certificate or any of the intermediate Domain Controller Certificates have expired.  Expired is when the X.509 “Valid to” date in the certificate is older than the validated system date.

SSE 123 must be returned if a dataset update being imported is not sequential with the latest update already contained in the ENDS/System Database for any given dataset. Under these conditions the update process (for the dataset) must be terminated and the ECDIS is to display a warning when the dataset is displayed stating that it is not up to date and should not be used for navigation.

SSE 125 must be returned if the stored permit for any given dataset has expired. It should be possible to view the dataset but a permanent warning message must be displayed informing the user; for example, “The permit for dataset <name> has expired. This dataset  may be out of date and MUST NOT be used for Primary NAVIGATION”.

SSE 126 must be returned if a signed resource (dataset, catalogue or permit) in an Exchange Set is authenticated against a certificate or public key file stored on the Data Client other than the one provided by the SA. In a chain of digital signatures the root certificate must be the SA certificate.

SSE 127 must be returned if the status of the dataset being viewed is not up-to-date in respect of the latest installed S-128 Catalogue dataset loaded or maintained on the system. A permanent warning message must be displayed on screen informing the user; for example, “dataset <cell name> is not up to date. A New Edition, Re-issue or Update for this dataset is missing and therefore MUST NOT be used for Primary NAVIGATION”.

SSE 128 must be returned if one or more dataset resources (datasets, catalogues or permits) could not be loaded because the contents could not be validated according to the versions of the (XML) Schemas installed on the system. Only conformant content may be loaded onto any S-100 system.

SSE 129 must be returned if the Catalogue to be installed (Feature, Portrayal or Interoperability) has not been directly authenticated by the Scheme Adminstrator.

SSE 130 must be returned if the status of one or more non-ENC datasets or supplementary files being viewed is not up-to-date in respect of the latest installed S-128 catalogue dataset loaded or maintained on the system. A warning message must be displayed on screen informing the user, e.g. “Warning: dataset <dataset name> is not up to date. A New Edition or Update for this dataset is missing”.

SSE 131 must be returned if a dataset has no Feature Catalogue or Portrayal Catalogue installed.

SSE 132 must be returned if an intermediate digitalSignatureValue for a resource (whether dataset, supplementary file or permit) fails validation.

SSE 133 must be returned if the dataset being installed is of a version which is not identical to a Feature Catalogue or Portrayal Catalogue installed on the system.

SSE 134 must be returned if the CATALOG.XML digital signature (CATALOG.SIG) cannot be authenticated against the Scheme Adminstrator’s certificate, either through the data server’s signed public key or any specified intermediate Domain Controllers. The contents of the Catalogue must not be installed if the signature is invalid.

SSE 135 must be returned if the PERMIT.XML digital signature (PERMIT.SIG) cannot be authenticated against the Scheme Adminstrator’s certificate, either through the data server’s signed public key or any intermediate Domain Controllers.

The flow diagrams in this Appendix show the conditions under which the SSE codes are displayed. These diagrams only relate to processes carried out on the ECDIS for import of exchange set contents. Data Server and Scheme Administrator processes are not defined here.

Exchange Set Installation on ECDIS.

The following Figure shows the sequence of actions to import Exchange Set contents. These contents are all digitally signed, and may optionally be encrypted.

For encrypted data the following process is first used to establish the validity of the permit.xml and its contents in respect of the end user’s subscription details. This is only for use with datasets which are encrypted and should be carried out prior to dataset file decryption according to S-100 Part 15.

Figure C-2-10 - Check for permit validity prior to decryption

Once encryption and compression has been processed the digital signatures for all Exchange Set contents to be imported can be processed. These signatures can form a chain of authentication and the following flow Figure illustrates how such signatures should be processed.

Figure C-2-2 - Authentication of Exchange Set CATALOG.XML, permits and dataset components

Under S-100 Part 15 datasets and other Exchange Set components can have multiple digital signatures, with each signature referenced to a signed public key certificate. The Exchange Set contains all relevant certificates with the exception of the SA digital certificate which is installed separately by the end user.

Using the flow shown in Figure C-2-2 the chain of certificates for dataset components, permits and the catalog.xml itself can be authenticated.

Full documentation of the authentication processes and technical details are contained in S-100 Part 15.

Certain Exchange Set contents control the behaviour of the ECDIS and may only be digitally signed by the Scheme Administrator. These categories of content, the Feature Catalogues, Portrayal Catalogues and Interoperability Catalogues should be authenticated using the following process. IHO standards determine the content and revision of such Catalogues.

Figure C-2-3 - Authentication of Feature, Portrayal and Interoperability Catalogues

# Appendix C-3 - ECDIS Update Status Reports

Purpose

This Appendix elaborates the definition of an “up to date” dataset. This is required by the IMO Performance Standards and all ECDIS users are required to be able to demonstrate to a vetting inspector that their ECDIS and its installed datasets are “up to date”.

This Appendix specifies the format and content of an “Update Status Report” which will demonstrate the revision status of datasets within the ENDS/System Database of the ECDIS. There are two Update Status Reports required:

- An ENC Update Status Report, which reports the contents of electronic charts and directly related information.

- An ENP (Electronic Nautical Publication) Update Status Report which reports the status of ENPs

Use and Responsibility

An Update Status Report is designed as a concise and standardised format to assist end users in satisfying themselves that their data is “up to date” and help satisfy inspection requirements in that respect.

The report is designed for two individual use cases:

- To ensure that all navigational data loaded into the System Database is up to date for a section of a particular route

- To ensure that all navigational data loaded into the System Database is up to date.

All S-100 datasets use Edition and, sometimes, update numbers to denote their individual revision (see S-100 Part 17 for details). These numbers uniquely identify how up to date a dataset is. The difficulty with using Edition/update numbers in isolation is that they do not document the date of revision of all datasets relative to the installation date of a data server’s service giving the end user no indication of whether a dataset (or a collection of datasets) is up to date with respect to the last dataset installed.

S-100 supports the creation and distribution of IHO S-128 data which provides an issue date within its metadata for an entire data service as well as a snapshot of the Edition and update information for all component datasets within a data service. The issue date of the last delivered S-128 dataset is used in each update status reports to provide a reference date for every dataset either on a section of the vessel’s route or for the System Database as a whole.

The driving use case for the Update status report is that an inspector (for example flag State, port State or vetting) or end user can check that an ECDIS is up to date with respect to the last dataset installed from each data server’s data within the System Database.

Note that this report specification is not a concrete definition of what an “up to date” dataset (or ECDIS) is; it only shows a status of each dataset with respect to the last data installed on the system. If a period of time has passed since the last update then some data may well be “out of date” relative to the time the report is run – the report only records the up to date status relative to the last data installed (the date being defined by the issue date of the latest S-128 dataset). The equipment manual should explain terms “up to date” and “out of date”.

This report only reports the status of datasets delivered as part of a data service using S-100 Part 17 Exchange Set metadata and (as part of dual-fuel mode) IHO S-57 datasets (as illustrated in the hybrid Exchange Set in S-100 Part 17, clause 17-4.2). Where other S-100 datasets are installed on the system from other sources (for example, Unencrypted ENCs) then they should be displayed with a status of “unknown”. The Equipment manual should explain terms “unknown”.

This Appendix provides a specification for a report which can be run on a compliant ECDIS to determine the status of S-100 datasets held within the System Database.

Report Definitions

Report Contents

Two reports are required:

- An ENC Update Status Report, which reports the contents of electronic charts and directly related information. This report shows the status of IHO S-100 datasets of the following product types:

  - ENC, either S-101 or S-57

  - S-102

  - S-104

- An ENP (Electronic Nautical Publication) Update Status Report which reports the status of ENPs. All other S-100 products (with the exception of S-128 itself) installed in the System Database are in this category.

The report name at the top of the report header shows the type of Report being produced.

Report Filters

The Update status report defined in can take one of two forms:

- A report detailing the status of all datasets within the System Database of the ECDIS for the appropriate type (ENC or ENP). This shows the status of the datasets with respect to the date contained in the S-128 dataset of the last update for each data server’s service installed.

- A report detailing the status of each S-100 dataset along a predefined section of a route contained within the ECDIS.

Example

An example of an Update Status Report for current ECDIS with S-57 ENCs is shown in the Figure C-3-1, followed by an illustrative revised report for S-100 datasets in Figure C-3-2. The example shown is an ENC Update Status Report.

Figure C-3-1 - Update Status Report – current report example

Figure C-3-2 - Update Status Report - notional example for S-100 datasets

Report specification

The ENC Update Status Report is a formatted output from the ECDIS. It should be formatted to fit within the width of the ECDIS screen and, if printable, split into individual A4 pages.

Reports of both types (complete System Database contents and filtered by route) are divided into two sections:

- A header containing information on the vessel and report content selected (either filtered for a route (or part of a route) or for the entire System Database contents. This header contains vessel information and report reference dates followed by a summary totals for each report status.

- A table containing information for each cell within the chosen content type. This provides detailed information on each dataset within the chosen content category.

Report header

The data content of each of the header fields is defined in Table C-3-1 below:

| Field Name. | Type | Source |
| --- | --- | --- |
| - Report Name | Text | Two Report names are possible depending on which Report is being shown. Either: - Electronic Navigational Charts (ENC) Update Status Report - Electronic Nautical Publications (ENP) Update Status Report |
| - Vessel Name | Text | The name of the vessel as recorded within the ECDIS. |
| - Identifier | Text | A unique identifier, the MMSI or vessel IMO number. |
| - Update reference date | Date | The data used as the reference for the status of each of the cells. This is the issue date of the last S-128 dataset in the Exchange Set used to update the System Database. The date is taken from the latest S-128 datasets issueDate in the CATALOG.XML and is expressed in ISO8601 notation: [YYYMMDDZHHMMSS] |
| - Date of report | Date | The date the Report was run. |
| - Content | Text | Each Report can be optionally filtered for an individual route plan or report the full System Database contents for each Report type: - “Filtered for Route Plan XXX to YYY” where XXX and YYY are the textual names of the point of origin and destination on the chosen route. - Full System Database contents. |
| - Start WP | Text | This field is only present if the Report is filtered for a route. It should comprise the textual name of the starting waypoint of the route (if one exists) and the lat/long coordinates of the waypoint. There is no fixed form that the coordinates should take. |
| - End WP | Text | This field is only present if the Report is filtered for a route. It should comprise the textual name of the last waypoint of the route (if one exists) and its lat/long coordinates. There is no fixed form that the coordinates should take. |

Table C-3-1 - Data content of header fields

Filtering of Update status report for route section

Where an Update Status Report is filtered for a route plan then the datasets of the appropriate type (ENC or ENP) in the System Database whose status are checked are defined by the intersection of the route corridor with the dataset boundaries (as defined by dataset’s coverage features (or coverage defined in the S-100 CATALOG.XML within the System Database)).

The width of the filtering corridor is equal to the “user specified distance” implemented inside the ECDIS to fulfil IMO MSC.532(106) 11.3.5:

“A graphical indication should be given if the mariner plans a route closer than a user-specified distance from the boundary of a user-selectable category of prohibited area or a geographic area for which special conditions exist (see appendix 4). A graphical indication should also be given if the mariner plans a route closer than a user specified distance from a user-selectable category of point objects, such as a fixed or floating aid to navigation or isolated danger. The user-selectable categories should be the same as the user selections for the display of objects and be based on IHO standards. There should be a permanent indication when any user-selectable categories are deselected. Details of the deselection should be available on demand.“

This is not the same as the XTD distance.

Summary totals

The summary section of the Report follows directly after the header. The summary contains the following information:

- The title: “Chart Status Summary”.

- Totals of cells with the relevant status in the order defined below.

  - Total – the total number of datasets available in the System Database for the content type (ENC or ENP), selected for the Report (either full or filtered by route).

  - Up to date – the total number of datasets (for the content selected) which have status “Up to date”.

  - Not up to date – the total number of datasets (for the content selected) which have status “Not up to date”.

  - Withdrawn – the total number of datasets (for the content selected) which have status “Withdrawn”.

  - Cancelled – the total number of datasets (for the content selected) which have status “Cancelled”.

  - Unknown – the number of datasets for which a status cannot be determined for any reason.

- The possibilities for each dataset’s status are listed in Table C-3-2 below along with their definitions.

Data Server content tables

The detailed tables in each Report are arranged by data server – each separate data server or dataset data source within the System Database has its own separate table listing all datasets by content type (as reported in the “Content” field in the report header) and S-100 Product Specification order.

The detailed tables contain the following information:

- Title: Data Server Name – this is the data server identified by the S-128 Producer attribute.

- For each cell installed in the System Database from the data server:

  - Dataset Name – the name of the dataset. (DSNM).

  - Edition – the Edition of the dataset in the System Database (EDTN).

  - Update – the update number of the dataset in the System Database. If the product does not support updates, then this must be labelled “-“.

  - Issue Date – the issue date of the last applied update to the dataset in the System Database (or “-“ where no updates are supported for the S-100 product).

  - Status – the status of the cell. The status may have one of four values determined according to the criteria in the following table:

| Status | Specification |
| --- | --- |
| Up to date | This is where the System Database has all the latest Edition and (where supported by the S-100 Product Specification) update information for the dataset installed as defined by the latest S-128 dataset received from the data server. The reference date for the most up to date information is defined by the S-128 dataset issue date. The dataset update reference date must be within the last four weeks from the time of the Report execution or the cell must be displayed as “Not up to date” regardless of its status as defined by the S-128 data. |
| Not Up to date | This is where the System Database has NOT installed all the latest update and/or New Edition for the cell. Again, the reference point for what should be installed is defined by the S-128 dataset issue date. If the reference date is older than four weeks then cells must be displayed as “not up to date” by definition. |
| Withdrawn | The number of datasets which have been withdrawn by the data server or cancelled but which are still available within the System Database. |
| Unknown | Datasets for which a status cannot be determined for any reason. If the revision information in the latest S-128 dataset is incomplete for any reason then all datasets in the data server’s service not included in the partial S-128 must be deemed to be “Unknown” as no definitive information on them can be determined. S-128 content is required to specify the status of all datasets in a data server’s service. |

Table C-3-2 - Status values

Example of ENC update status report

Report Name: ENC Update Status Report

Vessel Name: HMS Goteborg

Identifier: IMO 4653321

Update Reference Date: 16 May 2013

Date of Report: 1 Jun 2013

Content: Full

Chart Status Summary:

Chart Status: Count

Total: 50

Up to Date 38/50

Not Up to Date 10/50

Withdrawn 2/50

Unknown 0/50

Table:

Data Server: XXXX

| Product | Dataset Name | Edition | Update | Issue Date | Status |
| --- | --- | --- | --- | --- | --- |
| S-101 | 101US23495820 | 10 | 4 | 2020-01-02 | Up to Date |

Example of ENP update status report

Report Name: Electronic Nautical Publications (ENP) Update Status Report

Vessel Name: HMS Goteborg

Identifier: IMO 4653321

Update Reference Date: 16 May 2013

Date of Report: 1 Jun 2013

Content: Full

Chart Status Summary:

Chart Status: Count

Total: 50

Up to Date 38/50

Not Up to Date 10/50

Withdrawn 2/50

Unknown 0/50

Table:

| Data Server: XXXX |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Product | Dataset Name | Edition | Update | Issue Date | Status |
| S-123 | 123US3245394 | 5 |  | 2020-01-02 | Up to Date |
| S-127 | 127US2345032 | 6 |  |  |  |

# Appendix C-4 - User Selected Safety Contour and Water Level Adjustment

# User Selectable Safety Contour

## Introduction

This Appendix defines how the “User Selectable Safety Contour” feature is defined and implemented.

User selectable safety contour means creation of the safety contour from the bathymetric grid data based on the value set by the user.

NOTE: Also, in the case of S-101 or S-57 the user sets the value for the safety contour, but if the exact value is not found from the available depth information in S-101 or S-57 then the safety contour defaults to the next deepest which may be over 10 metres deeper than the value set by the user.

In these processes an end user selects:

- Suppression of certain S-101 features by S-102.

- A Safety Contour value of their choice.

When a user selects a safety contour value, outside areas of S-102 coverage the source for the displayed safety contour is the existing S-101 ENC (or S-57 ENC in dual-fuel mode).

When the safety contour has been generated from S-102 data then the area over which it is generated, that is the S-102 coverage area, must be surrounded by a boundary line using colour token DEPWL  see Figure C-4-1.

Figure C-4-1 - Boundary line of S-102 coverage

## Data Constraints

- Coverage – S-102 data is not expected to overlap, but in case of overlap the ECDIS should indicate an overlap by the text “OVERLAP” and the user should have the ability to select which producer in the overlapped area has priority and will be selected for processing safety contour.

- No complex interpolation is done between points in the S-102 grid. Nearest neighbour is used to define the depth in each S-102 point's neighbourhood (that is, the same depth everywhere within the grid square), regardless of any interpolationType defined for the S-102 dataset (S-100 Part 8, clause 8-7.1.4), as illustrated by the following Figure:

Figure C-4-2 - The extents of the S-102 points overlaid on the S-102 grid (grid spacing = ‘d’)

Figure C-4-3 - Extents of each S-102 points showing nearest neighbour interpolation (S-100 Part 8-7.1.4)

- Only rectangular grids for S-102 are allowed. The method outlined may be extended to other grids; for example, triangular grids in the future.

- User Selected Contour can only be processed in areas where the Sounding Datum of suppressed S-101 features is the same as the datum defined in the S-102 Coverage.

## Implementation

The user may select S-102 to be used as the source of the depth information.

When S-102 is selected for use, in areas where S-102 coverage exists, the S-102 suppresses the following S-101 depth features,

- Depth Area

- Dredged Area

- Depth Contour

The process to construct the safety contour then consists of a selection of common edges from rectangular extents for each S-102 dataset point, as shown in Figure C-4-4.

Figure C-4-4 - Common edges from rectangular extents

Common edges are selected when the S-102 points on either side of them lie on either side of the boundary of the safety contour value. In Figure C-4-5 below 11.5 metres is the threshold value set by the user.

Edges on the boundary of the dataset, or which lie on an edge common with undefined S-102 values, are selected if the S-102 value is less than or equal to the safety contour value; for example, see Figure C-4-5:

Figure C-4-5 - Edges from rectangular extents and at boundaries

The source of the displayed safety contour are the selected edges as defined. The areas of safe and unsafe water formed by the selected edges are used for processing of the safety contour related alarms under IMO MSC 530(106) 11.4.3, which the OEM must  implement.

# Water Level Adjustment

## General

This section defines how the “ECDIS Water Level Adjustment feature” is implemented. Water Level Adjustment is referred to in this section as WLA.

This section defines how the adjustment of depth information by water level is provided for:

- Selected single date and time;

- Selected date and time period;

- Linked to an estimated route schedule with selected check distance and time resolution.

This section defines how the “ECDIS Water Level Adjustment feature” is implemented. Water Level Adjustment is referred to in this section as WLA.

## Constraints on input data

- Coverage – S-104 data is not expected to overlap, but in case of an overlap of greater than 5 metres the approach must be same as for overlapping S-102 data. The ECDIS must indicate an overlap by the text “OVERLAP” and the user must have the ability to select which producer in the overlapped area has priority and will be selected for processing WLA.

- As with S-102, each S-104 point is assigned a rectangular extent and no complex interpolation is done between the points in the S-104 grid.

- For WLA processing all S-102 and S-104 data must be on the same vertical datum.

## User inputs

The User may select WLA to be used as source of the depth information.

When WLA is selected for use, in areas where WLA coverage exists, the user selects one of three different WLA options:

- WLA Option 1: WLA at a single datetime.

- WLA Option 2: WLA for a single datetime period (from start datetime to end datetime).

- WLA Option 3: WLA linked to an estimated route schedule. In this case the user also specifies:

  - A distance parameter, the limit of the check area as specified by IMO MSC 530(106) 11.3.5.

  - A time resolution tu used to construct the individual WLA sections. This time resolution reflects the uncertainty or tolerance of the time schedule of the route, for example 10 minutes if the user assumes that they could follow the time schedule within 10 minutes.

## Implementation - general

WLA can be applied only in areas where there is data from both S-102 and S-104. for example, the intersection of the Red and Blue outline in figure C-4-6 below. WLA is applied to S-101 features after substitution of depths by S-102 defined in clauses C-4-1 and section C-4-3.1.

Figure C-4-6 - WLA can only be computed in areas where there is S-104 and S-102 data

Further WLA can only be carried out:

- When WLA option 1 or WLA option 2 is selected, where the temporal extent of the S-104 overlaps the required datetime instant or period selected by the user

- When WLA option 3 is selected, where the S-104 temporal extent of the S-104 overlaps the estimated time a part of the route; see figure C-4-16.

Where these conditions are not met the WLA processing is not carried out.

When WLA has been processed the area for which it is defined must be surrounded by a boundary line using colour token DEPWL); see Figure C-4-7.

Figure C-4-7 - Boundary line of WLA coverage

NOTE: In this example S-102 coverage is larger than WLA coverage.

In addition to the display of the boundary line of the WLA coverage there should be a permanent indication about application of WLA and the applied datetime, see details in WLA Options 1, 2 and 3.

OEMs are free to design their user interface. Usable ideas include for example mouse roller to change datetime, use of a slider to change datetime, or even to provide an animation from user selected start datetime to user selected end datetime.

## Implementation of WLA Option 1 – WLA for a single datetime instant

WLA is based on S-104 values closest to the selected datetime instant. Each S-102 point has an extent of coverage which is closest to it. Each S-104 point, similarly has an extent. The adjustment of the S-102 values is calculated by adjusting each S-102 point by the shoalest of the S-104 values, for all S-104 points whose extent intersects the extent of the S-102 point.

When an S-104 record does not exist for the precise time specified the shoalest of the two S-104 adjacent values is selected from the S-104 dataset. S-104 values can only be selected within the temporal extent of the S-104 dataset. In the example shown in Figure C-4-8 below (S-104 values, “V” defined every 15 minutes and a user selected datetime of 07:24) the two values selected are 07:00 and 07:30, WLA would select 1.3m (the 07:00 value).

Figure C-4-8 - Selection of time-varying value

In Figure C-4-9 below, the S-102 point X is adjusted by the shoalest (that is, minimum) value of the S-104 values from (a), (b), (c) and (d) at the required datetime instant because the S-102 point extent overlaps the S-104 extents of a,b,c and d.

Figure C-4-9 - Adjustment of S-102 values by S-104

Format of the permanent indication is as below:

WLA 12:34 08 Nov 2021

## Implementation of WLA Option 2 – WLA for a datetime range specified by the user as a time period

When WLA is based on a datetime range, then the process is identical to that followed for WLA Option 1 except each S-104 value selected is the shoalest of all values available in the S-104 within the selected time period.

All S-104 points contributing to the WLA must be defined across the time range required, otherwise the WLA is not computable and the user must be informed. The S-104 records selected are those which lie either within the user defined time period; or before the start point of the time period and just after the end point of the time period. In the following examples:

Figure C-4-10 - Examples of S-104 data selection

- User selected period 1, four values are from 0700, 0730, 0800 and 0830 - WLA would select 08:30 value = 1.2m (even though the time period doesn't overlap).

- User selected period 2, two values are from 0630 and 0700 - WLA would select 06:30 value V=1.2m (even though the time period doesn't overlap).

- User selected period 3, three values are from 0600, 0630 and 0700 - WLA would select 06:00 V=1.1m (even though the time period doesn't overlap).

- User selected period 4, five values are from 0630, 0700, 0730, 0800 and 0830 – WLA would select 07:30 V=1.2m.

When adjusting depth values the shoalest (smallest) value from the selected S-104 records must be used in order to produce the safest WLA values.

Format of the permanent indication is as below:

WLA from 12:34 08 Nov 2021 to 14:56 08 Nov 2021

## Implementation of WLA Option 3 – linking of WLA to a defined route with planned waypoints and times

When WLA is based on a route then the limit of check area around the route is set by the user as specified by IMO MSC 530(106) 11.3.5.

The same user-specified distance shall be used for the check of safety contour, prohibited areas, geographic areas for which special conditions exist and navigational hazards” and equivalent requirements when route monitoring

The WLA is processed within this check area. The boundary of the area of the display where WLA has been carried out must be displayed. In the process description this distance is referred to as ‘a’.

A route could be either a Planned route or a Monitored route. Both could be processed for the WLA, but not at the same time in a single display area. In case of multiple display areas, it is possible that one area is WLA processed for Planned route and another area is WLA processed for the Monitored route.

For a Planned route the datetime period applied for the WLA process is based on the schedule of the planned route itself.

For a Monitored route the datetime period applied for the WLA process is user selectable either:

- Based on the planned schedule of the monitored route itself; or

- Based on the monitored route adjusted for the current own ship position.

When WLA is based on the planned schedule and own ship is not keeping to schedule a Caution must be raised to indicate the water level being experienced may be different to that being applied by the ECDIS

The region of WLA is restricted to a distinguishable bordered polygon around the route. The process works as follows:

- For each section of the route a series of estimated time markers is defined along the route at times t0 – tn. The diagram also shows the “limit of check area” as specified by IMO MSC 530(106) 11.3.5. Each time marker is delimited before and after by the time +/- half the user selected tu interval so the rectangle in the diagram represents the extent of time t1 which ranges from time t1-tu2  to t1+tu2. This is the WLA adjustment polygon corresponding to time t1:

Figure C-4-11 - WLA adjustment polygon

- Where the route follows a curved section the polygon formed is not a rectangle but an area defined by the user selected limit as shown by areas a and b:

Figure C-4-12 - WLA adjustment polygon along curved section

- The individual S-102 points (blue dots in Figure C-4-13 below) are assigned rectangular extents. For each WLA adjustment rectangle corresponding to each ti the S-102 extents which spatially intersect are selected. A full example is shown in Figure C-4-13 for one of the WLA polygons.

Figure C-4-13 - Grid cell selection

- For each S-102 point whose extent intersects the WLA adjustment box the Water level is adjusted using a similar process to Option 1. The S-104 value used for adjustment is the shoalest value from each of the S-104 points whose extents intersect the S-102 point (as described in WLA Option 1) across the time period (ti-tu2) and (ti+tu2), In Figure C-4-14, illustrating this step, two S-102 points are adjusted by S-104 values in the time period (t1-tu2) and (t1+tu2),  for calculation of t1. S-102 point x is adjusted using values drawn from S-104 points a and b (because its extent intersects the extents of a and b) and S-102 point y is only adjusted with values from S-104 point b.

Figure C-4-14 - Selection of S-104 data

- Each of the S-102 points are assigned the adjusted water level equal to the S-102 value + the calculated (shoalest) S-104 value as defined in the previous step.

- Once the WLA polygon for time ti is processed the WLA polygon for time ti+1 is processed.

- S-102 points whose extents are intersected by the both the WLA polygon for ti and ti+1 are assigned the shoaler of the two values, that is the shoalest of the S-104 values between the two time periods  (ti-tu2) and (ti+tu2), and (ti+1-tu2) and (ti+1+tu2).

This completes the WLA process for polygon ti . In Figure C-4-15 below the orange border shows S-102 points lying in WLA rectangles t1 and t2 and which would be assigned the shoaler of the two S-104 values corresponding to the t1 and t2 WLA process.

Figure C-4-15: Time-dependent adjustment of S-102 data with S-104 data

This process is extended to all ti along the planned route.

Figure C-4-16 below shows a border line around all the WLA processed S-102 grid cells. This border marks the boundary of the Water Level Adjustment area and requires portrayal to inform the user which areas of the display are subject to WLA. This area is surrounded by a distinguishable boundary line using colour token DEPWL).

Figure C-4-16 - Boundary of adjusted area

Figure C-4-17 below shows how the rectangle is positioned in the middle of S-104 coverage area and at the edge of the S-104 coverage are.

Figure C-4-17 - Positioning in interior and at edge

Format of the permanent indication is as below:

WLA from 12:34 08 Nov 2021 to 14:56 08 Nov 2021

# Treatment of depth and water level related S-101 features

## Substitution and adjustment of depth values

### General

In areas covered by only S-102, all depth values are substituted for all ENC features at all scales which have depth attribution (that is, the attribute valueOfSounding bound to them in the S-101 Feature Catalogue)

In areas where WLA is processed, all depth values are adjusted for all ENC features at all scales which have depth attribution (that is, the attribute valueOfSounding bound to them in the S-101 Feature Catalogue).

### Areas covered by S-102 only or by both S-102 and S-104

Adjustment of different geographic primitives.

- Point Features

  - For Areas covered by S-102 only, a value for the attribute valueOfSounding must be taken from the S-102 grid cell extents which intersects the point feature.

  - For Areas covered by S-102 and S-104, the value for attribute valueOfSounding is taken from the S-102 grid cell extents which intersect the point feature. WLA is then applied to this value using S-104 records selected from the S-104 grid cell intersecting the point feature. The S-104 value selected is as defined in sections C-4-2.5, C-4-2.6 and C-4-2.7.

- For Curve and Surface features

  - For Areas covered by S-102 only, the value for valueOfSounding must be the shoalest value of all S-102 grid cells whose extents intersect the feature’s geometry within the S-102 coverage available. If the curve or surface feature is not completely within the S-102 then the value defined is the shoalest resulting from the original feature value in S-101 attribute and the value selected from the intersecting S-102 grid cells.

  - For Areas covered by S-102 and S-104, the value for valueOfSounding is first selected, as the shoalest value of all S-102 grid cells whose extents intersect the feature’s geometry. WLA adjustment is then applied to this value by selecting the shoalest value from all intersecting S-104 grid cells. The value selected is as defined in clauses C-4-2.5, C-4-2.6 and C-4-2.7. If the curve or surface feature is not completely within the S-104 coverage then the adjusted value is the shoalest value resulting from the original feature value in S-101 attribute and the WLA adjusted value.

Figure C-4-18 - Substitution of sounding value from S-102

Figure C-4-19 - Substitution of valueOfSounding attribute in S-101 Obstruction from S-102 data

The S-101 features for which this depth substitution is required in S-102 coverage areas are listed, (items 1,2 and 3 are also used as sources for indications under IMO MSC 530(106) 11.4.6).

- Underwater/Awash Rock

- Wreck

- Obstruction

- Foul Ground

- Marine Farm/Culture

- Soundings (either individual soundings or those which are part of an array) substituting the S-102 grid cell whose extents intersect the sounding position for the defined ZCOO (ISO8211).

If the depth substituted S-101 feature is covered by S-104, then WLA is processed using same method as for the underlying depth area.

Depth values must also be substituted for the attribute depthRangeMinimumValue in other S-101 features. These are:

- Gate

- Berth

- Dry Dock

- Floating Dock

- Dredged Area

- Swept Area

- Pipeline Submarine/On Land

- Recommended Track

- Fairway

- Recommended Route Centreline

- Two-Way Route Part

- Deep Water Route Centreline

- Deep Water Route Part

For all features with substituted (and possibly adjusted values), the ECDIS Pick Report shall indicate the substituted/adjusted value and its source. The format and portrayal of depths and drying heights (that is, number of decimals, etc) is unchanged.

For example, for S-102 only covered area:

Value Of Sounding 12.3 m [S-102]

When an S-101 attribute has been WLA adjusted the pick report shall indicate the WLA adjusted value, source and time/date. The format of time date is: hh:mm dd mmm yyyy.

For example, for both S-102 and S-104 covered area:

Value Of Sounding 15.5m [WLA 12:34 08 Nov 2021]

Value Of Sounding 15.5m [WLA from 12:34 08 Nov 2021 to 14:56 08 Nov 2021]

### C-4.3.[Deleted]

## Adjustment of heights and vertical clearance values

Any vertical measurement which is referenced to the same vertical datum as the S-104 data, either by the dataset metadata VDAT or Vertical Datum meta feature shall be adjusted using the intersecting S-104 grid cell water level value. This shall be applied on any features within the coverage of S-104. Where a feature is not completely within the S-104 coverage the adjusted value is the minimum value resulting from the original feature value in S-101 attribute and the WLA adjusted value. Adjustment is defined for the following S-101 features:

- Building.

- Span Fixed

- Span Opening

- Conveyer

- Cable Overhead

- Pipeline, Overhead

- Tunnel

- Wind Turbine

- Gate

- Crane

- Light All Round; Light Sectored; Light Fog detector; Light Air Obstruction.

The simple attributes adjusted are height and verticalClearanceValue. When adjusting height or clearance values the largest value from the selected S-104 records shall be used in order to produce the safest values.

The user shall be given a permanent indication that the Water Level (S-104) values have adjusted attributes in the data display. Where values are not adjusted due to an incompatibility of vertical datums in the data, the user shall be given a similar permanent notification.

When either height or verticalClearanceValue in S-101 have been substituted or adjusted the ECDIS Pick Report shall indicate the substituted value, source and time/date. The format of the height or vertical clearance values (that is, number of decimals, etc) is unchanged.

For example:

Vertical Clearance Value 5.3 m Mean Sea Level [WLA 12:34 08 Nov 2021]

Value Of Vertical Clearance 15.5m Mean Sea Level [WLA from 12:34 08 Nov 2021 to 14:56 08 Nov 2021]

## Alerts and indication details

Substituted or adjusted values in ENCs at the largest scale shall be used as input to any alarm/indication processing. These may or may not be the same as the substituted values which are portrayed as shown in Figure C-4-20 below which shows an illustration of the graphical highlight (red boundary and red transparent fill) for the intersected safety contour.

Figure C-4-20 - Graphical highlight for intersected safety contour

## Legend details

The legend shall additionally indicate the vertical datum of S-102 and S-104.  If the vertical datums are the same then a single indication is enough to cover S-101, S-102 and S-104.

S-411, S-413, S-414, and perhaps parts of S-126 will be included in this layer when those specifications are more mature.

This table covers the Basic and Other products in clauses C-6.1.1 and C-6.1.2. Note that portrayal catalogues might be revised to change assignments to viewing groups.

IEC 60945 as cited in S-52 specifies that character size in mm be not less than 3.5 x the viewing distance in metres. According to this criterion "readable from 1 metre” requires that characters be not less than 3.5 mm in size. A 3.5mm symbol or character subtends an angle of approximately 12 arc minutes at a distance of 1 metre.

The rule that symbols and characters should subtend an angle of not less than 20 arc minutes at the observer’s eye (mentioned in S-52) results in a minimum character height of 5.76mm for a 1 metre viewing distance.

National text is a supplementary option for ECDIS. If used, the style for Latin characters must be similar to the English text. Other characters should conform to the requirements in C-10.2.2 and C-10.2.3 as far as possible.

TIFF is not recommended for pictures embedded in HTML files because common HTML viewers currently need a special plugin for TIFF.

Because of potential usability and cybersecurity issues.  Revised restrictions may be adopted after the IEC cybersecurity standard is complete.

Depending on implementation design, one-time validation upon installation, removal, or update of datasets on the system may suffice.

“Discrete interpolation” means no interpolation between data points.

Values are for hypothetical data and display, and the figures have been reduced for reproduction in this document.

CSS files use hex RGB to specify colours. Colour profile files in portrayal catalogues use CIE and decimal RGB.

For example, S-101 (ENC) requires the parameter “Safety Depth”, but S-123 (Marine Radio Services) does not encode any depth information and therefore cannot use “Safety Depth”, so it may not be listed in S-123 Portrayal Catalogues.

colour token DEPWL, xyL values are: DAY x= 0.15, y=0.15, L=30.0; DUSK x=0.15, y=0.15, L=7.5; NIGHT x=0.15, y=0.15, L=1.2

Thick dark blue dash line, colour token DEPWL xyL values are: DAY x= 0.15, y=¬0.15, L=30.0; DUSK x=0.15, y=0.15, L=7.5; NIGHT x=0.15, y=0.15, L=1.2.

thick dark blue dash dot line, colour token DEPWL, xyL values are: DAY x= 0.15, y=¬0.15, L=30.0; DUSK x=0.15, y=0.15, L=7.5; NIGHT x=0.15, y=0.15, L=1.2