CANDIDATE REPLACEMENT S-98 WORDS

12.12	 Display and Management of Navigational Warnings

S-124 Navigational Warnings (NAVWARN) portrayal is provided by a portrayal catalogue that includes a symbol set and symbolisation instructions for the feature and attribute combinations.

12.12.1	 Additional Portrayal requirements of the Graphical User Interface

A dedicated interface must be provided to allow users to interact with NAVWARN messages. This interface must, provide the following minimum functionality;

- The user must be able to classify individual messages in accordance with section 12.12.2

- An on-demand listing of all (S-124) NAVWARN messages in the system must be provided allowing sorting according to: installed date and time, issue date and time, warning type, producer, series and message classification (as defined in 12.12.2)

- The on-demand listing of all NAVWARN messages in the system must include means for viewing an abbreviated view of any NAVWARNPart, warningInformation attributes present. This should consist of a minimum of the issue date, warning type and the first 64 characters of the warningInformation attributes.

- An indication must be provided when a new NAVWARN message is loaded until it has been displayed or 24 hours have passed. This indication may be suppressed if the NAVWARN message does not meet filtering criteria set by the mariner or automatically applied (as defined in 12.12.2).

- Means must be provided to view the most recent message and past messages,

12.12.2	 Classifying Navigational Warning information

To ensure only relevant information is displayed, and to ensure an uncluttered screen, the mariner must be able to classify NAVWARNs which are then filtered for display by the ECDIS :

At a minimum, functionality must be included that allows the user to classify an installed NAVWARN against the intended route as:

- “On chart” Relevant for the route, Must always be displayed, or;

- “Off chart” Not relevant for the route, Need not be visualized, or;

- “Information” For information, Need not be visualized

On chart must be the default classification for all NAVWARNs.

A means must be provided for the mariner to enter criteria for automatic filtering of NAVWARN messages based on time and maximum distance from own ship, monitored route or planned route. When automatic filtering is applied by proximity or distance any messages without spatial extent must be classified as Information.

Even though the full extent of a navigational warning may not be portrayed, it must still be available and discoverable in a list of NAVWARNs that can be recalled by user action at any time

Additional filtering functions may include options such as;

- Navigational warning topic;

- Date range of the hazard;

- Valid time of the navigational warning.

These filters may then be used to assist the navigator in classifying a NAVWARN according to its relevance for the route.

12.12.3	 Cancelled datasets

When a S-124 dataset is cancelled it must not be graphically displayed on the navigation screen, but must be available for review in the on demand listing of NAVWARNs in the navigation system and marked as cancelled.

Three months after either a message is cancelled, or when the system date has passed the defined end date for the message it can be deleted from the system unless retained by the user in accordance with [insert requirement reference 61174?]

12.12.4 Portrayal of feature types

The NAVWARNAreaAffected class does not have a portrayal defined to avoid causing significant cluttering on the navigation screen. The feature must be highlighted by the system if selected from a pick report for interrogation by user.