|  |  | 3 | EBL /VRM |  | Not referenced in document |  | Agree – will take out DONE |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 7 |  |  | As written now, there are no requirements in this entire section. It is largely repetitive of S-100 Part 9 and/or some other parts. What is the point of including basically the same information? 7.2.6, below potentially can become a requirement, but needs stronger wording. |  | Agree - but would need consensus to take out. |
|  |  | 7.2.4 (Area fills) |  |  | This paragraph is just recommendation with “should”. Is that the intent? |  | One for testing? Edge of display is similar to truncated by display? |
|  |  | 7.2.6 |  |  | Does this section have a firm “must” requirement to avoid confusing complex line rendering? Noted, incorrect presentation is acceptable for shared edges. |  | No, was agreed by working group to be advisory only. |
|  |  | 7.2.9 Display Planes |  |  | Mutually exclusive lists is not how S-100 9-11.1.5 describes this. Agreed, “into lists”. |  | Needs new words. Could we just say “lists”? |
|  |  | 9.1.2 Supplemental Items |  |  | This section seems misplaced. Most of the bullets are S-100 product related! Can it live someplace outside of section 9? I don’t love 9.1 in general, but it’s not the end of the world to leave it here. |  | Needs a proposal |
|  |  | 9.2.2 |  |  | This section , 9.2, is confusing regarding optional or mandatory requirements. Section 9.2 is declared “informative”, but here in 9.2.2, there is at least this one “must” reference. 9.2.1 also has a “must” for category n+1. It depends on the intent of 9.2 as INFORMATIVE or NORMATIVE with additional requirements above what can be captured in PCs. Either remove “must” wording in subordinate sections or change “must” to “should" or “may”. The way it is, I will discard everything as an implementor when I see INFORMATIVE. |  | Agree - can take informative out. But leave a note that much of the section is informative. Does this relate to 8.2.2 (have taken out informative delaration) - DONE |
|  |  | 10.1 |  |  | I have to continue to emphasize the need for proper presentation of all features at all boundary conditions to avoid situations as outlined in S-101 portrayal clipping at boundaries · Issue #73 · iho-ohi/98-interoperability . There are many NOTES and IMPORTANT boxes in S-52 for emphasis. Have these all be adequately captured in S-98 and/or S-164? For example, from S-52 Annex A:100, Part 1, Edition 5.0(.0)> Section 10.3.4.1: Note: To ensure seamless presentation of a single intended usage (navigational purpose) all objects of same display priority from all cells in same intended usage must be drawn together. This note above from S-52, prevents symbols from being clipped at cell boundaries, it’s subtle but that Is the result of this NOTE in S-52. This is very noticeable for point features. You can clip out an entire buoy or light if you don’t do this. The MONG algorithm partially addresses this, but I hadn’t validated the S-98 words before leaving NIWC. |  | Yes, didn’t we agree this would be dealt with by test data and words of caution rather than mandatory requirements though? Needs discussion I think. To ensure seamless presentation all features of same minimum display scale from all datasets must be drawn together CHECK S-52 and insert note. |
|  |  | 12.2 (Display of non-ENC layers) |  |  | Does this section have a better relationship with 9.1 than here |  | Maybe. Will look |
|  |  | 12.3.2 (under diagram) |  |  | The term “grossly” needs to be quantified. I’m not sure this section is correct compared to S-52 10.1.10.1 and 10.1.10.2. We’ve gone from a single entity guidance of “compilation scale” to this hybrid “optimumDisplayScale” and “maximumDisplayScale” while trying to steal words from S-52 here. Does this need a fresh look by the group? |  | We already tried and couldn’t agree. It possibly is described For MTG? Grossly is past maximum – email and clarify in text. |
|  |  | 18.2 (appendix E) |  |  | I did not see any discussion of S-57 and S-101 in Appendix E. Appendix E only addresses S-101. Either more needs to be said about both on the screen in Appendix E, or drop reference to S-57 in this sentence and live with the prior paragraphs descriptions of dual fuel display in this section. Section 18.2 calls out duel fuel in Appendix E, but Appendix E only describes S-101 (implied S-101). In fact, there is no discussion of S-57 and S-101 on the screen anywhere in S-98. |  | Don’t we mention drawing index? If not, then lose the S-57 from here. (Agree) |
|  |  | App D-1 |  |  | How is this incorporated under the new S-100 ECDIS MSC(530)? You might point out MSC(530), Section 5.10. This is the only cross-reference I could find.Draft 61174 ED5, in section 4.3.6 Dynamic water level, references how S-98 defines the indication. |  | don’t know. Needs to be discussed (can ask IEC) |
|  |  | App D – Data Constraints |  |  | There should probably be some mention about resolving projection differences between S-101 and the use of S-102 for ESC which may be in a different projection than S-101. Well, S-102 started out as WGS-84, now the thinking is to ship it as UTM, I believe. You can’t just slap that on top of S-101, or even do replacement without resolving the projections, i.e. you have to Unproject the S-102 first. That was the point of my comment. |  | Surely we do this - by allowing projections. Not sure what you mean by “resolving” differences. We can reference the projection option. |
|  |  | App D – implementation. (must be enabled) |  |  | Should this be the first sentence in this section? Noted, fine leave it where it is, or move it as the second sentence at the top of appendix D. |  | Requested by AU. If there’s a better place please propose one. |
|  |  | App E |  |  | Do you need to say anything about “maximum display scale”, which is also required.im |  | No - max doesn’t apply in this section, it only applies for overcall display. The scale band is between min and opt. |
|  |  | E-1.5 |  |  | Some mention of “correctly” rendering all features at chart boundaries should be indicated in section E someplace. See S-101 portrayal clipping at boundaries · Issue #73 · iho-ohi/98-interoperability . Further review across the S-100 documentation framework does not adequately address the issues captured in the issue cited above (#73). It might be possible to further capture details in S-164, section 3.1.2, Dataset Rendering (MONG) See earlier reference of important note extracted from S-52, which MONG partially addresses if it is captured correctly here. |  | Needs more words - but not sure exactly where or wha |
|  |  | F-1.2 (gml:id with) |  |  | “and” instead of “with” |  | Agree DONE – apply to data constraints too. |
|  |  | F-1.2 |  |  | 4 means a triangle is OK as a coverage polygon. Is that the “real” minimum intent of the coverage polygon? |  | Yes. It’s a minimum. Unlikely but still a minimum |
|  |  | G |  |  | Manufaturers – spelling errro |  | Agree DONE |